3GPP TSG RAN WG1 Meeting #86                                                        R1-167473

Gothenburg, Sweden, August 22 – 26, 2016

Source: 	Ericsson

Title:	Identified errors in K-factor scaling for TDL-D/E in TR 38.900

Agenda Item:	8.2

Document for:	Discussion and Decision

# Introduction

In RAN1#85 in Nanjing, it was agreed to add scaling of the K-factors for LOS TDLs and CDLs according to R1-165440 [1]. The procedure described in has been captured in section 7.7.6 of TR 38.900 v14.0.0 [2]. However, it appears that the procedure is erroneous and does not result in the desired K-factors for the TDLs. This contribution discusses and demonstrates the errors and proposes modifications that are verified to result in the desired behaviour. A CR is jointly prepared in R1-167475 [3].

# Discussion

The K-factor scaling in TR 38.900 v14.0.0 is copied below for convenience.

Unfortunately the equations in Table 7.7.6-2 are erroneous and do not result in a TDL with the desired K-factor. This is readily apparent from visual inspection of the last column, where a high Kdesired[dB], causes the K1stTap[dB] to become very small, contrary to expectation. Less apparent is the fact that also the third column equations are imperfect, since the power scaling is not applied to the diffuse part of the first cluster as is done for the CDLs in Table 7.7.6-1. Finally, there is a copy/paste error where the value 9.3 for TDL-E has become 9 in the lower right cell. The errors when applying the K-factor scaling according to the description in TR 38.900 v14.0.0 are illustrated in Figure 1. As can be seen, the resulting K-factors are clearly in error for larger K-factors and also for smaller K-factors for TDL-D.

Figure 1 Resulting K-factor from procedure in TR 38.900 v14.0.0 Table 7.7.6-2

Observation 1: The K-factor scaling process for TDL-D/E in 38.900 v14.0.0 is erroneous and does not result in the desired K-factors

To address this issue, new equations for the K-factor scaling have been derived according to Table 1 below.

Table 1 New equations for the K-factor scaling of TDL-D/E

| Model | Table | Equation for power scaling of all delayed taps excluding tap#1. | Equation for the Tap #1 |
| --- | --- | --- | --- |
| TDL-D | 7.7.2-4 | Pn,new[dB] = Pn,old[dB] – Kdesired[dB] + 9[dB] -10·log10((1013.3/10+10(-Kdesired[dB] + 9[dB])/10)/(1+1013.3/10)) | K1stTap[dB] = 13.3[dB] + Kdesired[dB] - 9[dB] |
| TDL-E | 7.7.2-5 | Pn,new[dB] = Pn,old[dB] – Kdesired[dB] + 9.3[dB] -10·log10((1022/10+10(-Kdesired[dB] + 9.3[dB])/10)/(1+1022/10)) | K1stTap[dB] = 22[dB] + Kdesired[dB] – 9.3[dB] |

The resulting K-factors using these new equations are shown in Figure 1. As can be seen, the new equations result in K-factors which are exactly the same as the desired ones.

Observation 2: The modified equations in Table 1 are verified to produce TDLs with the desired K-factors

Figure 2 Resulting K-factor from procedure using equations in Table 1

A CR for TR 38.900 has been prepared that incorporates a change to the new equations according to Table 1.

Proposal: The CR R1-167475 that corrects the erroneous K-factor scaling of TDL-D/E is approved

# Conclusion

Observation 1: The K-factor scaling process for TDL-D/E in 38.900 v14.0.0 is erroneous and does not result in the desired K-factors

Observation 2: The modified equations in Table 1 are verified to produce TDLs with the desired K-factors

Proposal: The CR R1-167475 that corrects the erroneous K-factor scaling of TDL-D/E is approved

# References

R1-165440 “Scaling of the K-factor of the link level channel models”, Huawei, HiSilicon, Ericsson, Nokia, Samsung, Intel, Qualcomm, NTT DOCOMO, AT&T, KT Corporation, May 2016.

3GPP TR 38.900 v14.0.0 “Channel model for frequency spectrum above 6 GHz”, v14.0.0.

R1-167475 “Correction for K-factor scaling of TDL models”, Ericsson, August 2016.