3GPP TSG-RAN WG1#86	R1- 167645

Gothenburg, Sweden 22rd - 26th August 2016

Source:	Ericsson

Title:	W1 design for implicit advanced CSI reporting

Agenda Item:	7.2.4.2.3

Document for:	Discussion and Decision

# Introduction

In RAN1#85, the following was agreed regarding advanced CSI reporting:

- Specify enhancement on CSI reporting to improve eNB precoding. The specified enhancement is to be selected from the following categories:

  - Enhancements to Rel-13 feedback codebooks (FFS which numbers of antenna ports from the set {8, 12, 16, 20, 24, 28, 32}) that increase CSI resolution through improved beam selection / construction in W1 and/or improved beam/port selection / combining / weighting mechanisms in W2

  - Parameters representing channel coefficients, or some reduced space representation thereof including beam combining / weighting with coefficient quantisation or channel quantisation or channel covariance matrix quantisation

    - Uplink physical channel enhancements to carry the representation of channel coefficients can be included if selected

In our companion contribution [2], we discuss and evaluate the two categories of CSI enhancements. In this contribution, we focus on implicit feedback enhancements and discuss the necessary components of the W1 part of a rich CSI codebook. In another companion contribution[3], we conduct system simulations comparing our proposed enhanced W1 design with W1 from legacy codebooks.

# Necessary components of enhanced W1

We propose that a new advanced CSI codebook should keep the familiar factorized codebook structure, where the precoding matrices W are decomposed into a wideband W1 matrix factor and subband W2 matrix factor as

W=W1W2

but new codebooks for W1 and W2 are designed.

To effectively express the W1 codebook, we first define a dual-polarized rotated 2D-DFT beam space transformation matrix BNV,NH qV,qH as

BNV,NH qV,qH=RNHqHDNH⨂RNVqVDNV00RNHqHDNH⨂RNVqVDNV=d10d20……dNVNH00d10d2……0dNVNH=b1b2…b2N1N2   ,

where DN is a size N×N DFT matrix, i.e. the elements of DN  are defined as  DNm,n=1Nej2πmnN. The orthogonal 2D beams may thus be indexed by the orthogonal beam indices 0≤n1<N1-1, 0≤n2<N2-1. Further,  RNq=diag(ej2π⋅0⋅qNej2π⋅1⋅qN⋯ej2π⋅(N-1)⋅qN)  is a size N×N rotation matrix, defined for 0≤q<1. Multiplying DN with RNq from the left creates a rotated DFT matrix with entries RNqDNm,n=1Nej2πm(n+q)N. Rotating the beam space basis has an effect similarly to oversampling a codebook, for example, if the channel is a pure LOS channel and the angle of the LOS ray if perfectly aligned with a constituent beam in the beam space, the channel matrix can be described by only one beam coefficient. However, if the angle of the LOS ray lies in between two beams in the beam space, two beam coefficients are required to express the channel, doubling the amount of overhead needed.

We assume that the rotation factors q are uniformly quantized, i.e. q1=iQ1, i=0,1,…,Q1-1. q2=iQ2, i=0,1,…,Q2-1. Then, a rotated beam is equivalent to an oversampled DFT beam with oversampling factors O1=Q1 and O2=Q2. An example is shown in Figure 1.

Orthogonal DFT beams

Rotated DFT beams with rotation factors : q1=24, q2=14

N1O1 Beams

Oversampled DFT beams

l

k

O1=4

O2=4

N2O2 Beams

(k,l) =(0,0)

n1=0

n1=1

n1=2

n1=3

n2=0

n2=1

Figure 1: An example of rotated orthogonal beams expressed as oversampled DFT beams

The rationale for expressing the 2D beams by means of rotation indices q1,q2 and orthogonal beam indices n1,n2 instead of flat beam indices k=O1⋅n1, l=O2⋅n2  is that feedback overhead can be saved since the rotation indices are the same for all selected beams.

Thus, the columns bii=12N1N2of BNV,NH qV,qH thus correspond to mutually orthogonal DFT beams transmitted on one polarization and together constitute an orthonormal basis of the vector space C2N1N2.

The matrices in the W1 codebook are then constructed by a multiplication of a selected beam matrix BIS and a beam power matrix P, so that

W1= BISP.

The selected beam matrix consists of columns from BNV,NH qV,qH, where Nbeams beams are selected, as

BIS=dIS(1)0dIS(2)0……dIS(Nbeams)00dIS(1)0dIS(2)……0dIS(Nbeams),

where Is denotes the selected beam indices

IS=i1,i2,…,iNbeams=N2n1(1)+n2(1),N2n1(2)+n2(2),…,N2n1(Nbeams)+n2(Nbeams).

The beam power matrix can be expressed as

P=diag1,p2,…,pNbeams,1p2,…,pNbeams=I2⨂100p2⋱⋱⋱00pNbeams,

where 0≤pi≤1 ,i=2,…,Nbeams denotes the relative power allocation for each selected beam.

Thus, feedback of W1 comprises signaling the following quantities:

- Beam space rotation indices q1,q2

- Selection of Nbeams beams: n1(1),n2(1), n1(2),n2(2),… n1(Nbeams),n2(Nbeams)

- Relative power allocations p2,p3,…,pNbeams

The W2 matrix should perform co-phasing between different beams on the same polarization, i.e. linearly combining columns of W1 by weighting each column with a co-phasing factor ejα). An enhanced W2 matrix may then be expressed as

W2=1ejα21⋮ejα2Nbeams1………1ejα2r⋮ejα2Nbeamsr,

where r is the transmission rank, Nbeams is the number of DFT beams in W1 per polarization and ejαkl is a co-phasing factor from a PSK alphabet intended for the k:th column of W1 and the l:th layer.

# Evaluation results

We have evaluated the performance of MU-MIMO where the proposed advanced implicit CSI feedback has been used in the 3GPP 3D UMi scenario with an 8x4 antenna array with 2x1 virtualization, so a 32 port system with N1=N2=4. The advanced CSI systems perform MU-MIMO transmission and utilizes SLNR precoding. Relative gains are calculated against an SU-MIMO baseline using the Rel. 13 codebook. Other simulation parameters may be found in the appendix.

First, we investigate the importance of beam space rotation, i.e. how many values of q1,q2 are needed. A multi-beam codebook with Nbeams=3 beams are used, the relative power levels of each beam is set on a wideband basis, but are unquantized. The co-phasing factors, i.e. W2 selection, are set per RB and are also unquantized. In Table 3, simulation results comparing multi-beam precoder feedback with  1, 2, 4, 8 rotation hypotheses per dimension. As seen, there is a significant performance gain of beam space rotation. This is due to that more of the channel energy is captured in the Nbeams=3 beams if the beam space basis is aligned to the strongest multi-path components of the channel.

Table 3: Multi-beam codebook with different number of rotation hypotheses

| Scheme | 50% RU |  | 70% RU |  |
| --- | --- | --- | --- | --- |
|  | CE UTP gain [%] | Mean UTP gain [%] | CE UTP gain [%] | Mean UTP gain [%] |
| DFT Codebook SU-MIMO | 0 | 0 | 0 | 0 |
| DFT Codebook MU-MIMO | 28 | 9 | 67 | 26 |
| Multi-beam CB 1 rotation hyp. Per dim | 57 | 21 | 123 | 42 |
| Multi-beam CB 2 rotation hyp. Per dim | 74 | 25 | 176 | 57 |
| Multi-beam CB 4 rotation hyp. Per dim | 93 | 29 | 179 | 58 |
| Multi-beam CB 8 rotation hyp. Per dim | 87 | 28 | 187 | 59 |

Observation:

- Significant performance increase with beam space rotation, 4 rotation hypotheses per dimension is sufficient. This corresponds to 2+2=4 bits additional overhead.

Second, we investigate the need of spending bits on quantizing the relative power levels between the beams, compared to linearly combing the orthogonal beams using phase only. Again, a multi-beam codebook with Nbeams=3 beams are used, with 4 rotation hypotheses per dimension. The co-phasing factors are set per RB and are unquantized. The beam powers are uniformly quantized in the dB domain, in the range of [-12.5 , 0] dB, but where the minimum value is forced to be identically zero. In Table 4, evaluation results comparing 0, 1,2,3,4,10 bits allocated for indicating the relative power level of each beam is shown. As seen, there is a significant gain of linearly combining the orthogonal beams using not only phase, but also controlling the amplitude.  However, there seems to be enough with only 2 bits per beam. In Table 5,  we present additional simulation results for the  Nbeams=10 case. Using a phase-only codebook in this case results in large losses, since the TX power will be divided equally between the 10 beams and the channel energy in the majority of the beams will be very weak in most cases, unless the angular spread of the channel is very large.  In this case, using 2-3 bits for power allocation seems enough for maximum performance.

Table 4: Multi-beam codebook with 3 beams and different beam power level quantization

| Scheme | 50% RU |  | 70% RU |  |
| --- | --- | --- | --- | --- |
|  | CE UTP gain [%] | Mean UTP gain [%] | CE UTP gain [%] | Mean UTP gain [%] |
| DFT Codebook SU-MIMO | 0 | 0 | 0 | 0 |
| DFT Codebook MU-MIMO | 28 | 9 | 67 | 26 |
| Multi-beam CB Phase only | 32 | 10 | 78 | 26 |
| Multi-beam CB 1 bit beam power quantization | 73 | 25 | 159 | 51 |
| Multi-beam CB 2 bit beam power quantization | 89 | 27 | 182 | 57 |
| Multi-beam CB 3 bit beam power quantization | 83 | 28 | 182 | 58 |
| Multi-beam CB 4 bit beam power quantization | 83 | 28 | 179 | 56 |
| Multi-beam CB 10 bit beam power quantization | 93 | 29 | 186 | 58 |

Table 5: Multi-beam codebook with 10 beams and different beam power level quantization

| Scheme | 50% RU |  | 70% RU |  |
| --- | --- | --- | --- | --- |
|  | CE UTP gain [%] | Mean UTP gain [%] | CE UTP gain [%] | Mean UTP gain [%] |
| DFT Codebook SU-MIMO | 0 | 0 | 0 | 0 |
| DFT Codebook MU-MIMO | 28 | 9 | 67 | 26 |
| Multi-beam CB Phase only | -32 | -16 | -47 | -31 |
| Multi-beam CB 1 bit beam power quantization | 79 | 29 | 158 | 58 |
| Multi-beam CB 2 bit beam power quantization | 93 | 33 | 207 | 68 |
| Multi-beam CB 3 bit beam power quantization | 108 | 36 | 200 | 68 |
| Multi-beam CB 4 bit beam power quantization | 97 | 34 | 201 | 67 |
| Multi-beam CB 6 bit beam power quantization | 97 | 35 | 206 | 71 |
| Multi-beam CB 10 bit beam power quantization | 105 | 35 | 182 | 67 |

Observation:

- Relative power level between beams must be included in the codebook; a phase-only multi-beam codebook gives no gains compared to Rel. 13 DFT codebook.

- Quantizing the relative beam power levels with 2-3 bits is enough.

Based on these observations, we make the following proposal:

Proposal:

- Specify a new codebook for advanced CSI, where

  - W1 selection consist of

    - Unconstrained beam selection of orthogonal beams

    - Beam power selection with at least 2 bits per beam

    - Beam space rotation selection with at least 4 rotation hypotheses per dimension

  - W2 selection consist of

    - Co-phasing of columns in W1, where co-phasing factors are selected from a PSK constellation

# Conclusions

In this contribution, we have proposed a W1 design for advanced implicit CSI feedback as well as presenting evaluation result motivating the proposal. We have made the following observations and proposals:

Observations:

- Significant performance increase with beam space rotation, 4 rotation hypotheses per dimension is sufficient. This corresponds to 2+2=4 bits additional overhead.

- Relative power level between beams must be included in the codebook; a phase-only multi-beam codebook gives no gains compared to Rel. 13 DFT codebook.

- Quantizing the relative beam power levels with 2-3 bits is enough.

Proposal:

- Specify a new codebook for advanced CSI, where

  - W1 selection consist of

    - Unconstrained beam selection of orthogonal beams

    - Beam power selection with at least 2 bits per beam

    - Beam space rotation selection with at least 4 rotation hypotheses per dimension

  - W2 selection consist of

    - Co-phasing of columns in W1, where co-phasing factors are selected from a PSK constellation

# References

RP-160623, “New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE”, Samsung, Nokia, CATT, 3GPP TSG RAN Meeting #71, Göteborg, Sweden, March 7 - 10, 2016

R1-167643, “Explicit versus implicit feedback for advanced CSI reporting”, Ericsson, RAN1#86

R1-167644, “On the need for W1 enhancements for advanced CSI” , Ericsson, RAN1#86

# Appendix

## Simulation parameters

| Simulation Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD |
| Antenna Configurations | 8x4 with 2x1 virt., UMi (130° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (UMi) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead not accounted for. Channel estimation error modeled. |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS |