3GPP TSG-RAN WG1 Meeting #86	                                                               R1-167689

Gothenburg, Sweden, 22nd – 26th  August 2016

Agenda item:		7.2.5.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	MUST Case 1 and 2 transmission schemes

Document for:		Discussion and Decision

# 1	Introduction

In the 3GPP TSG RAN Meeting #71 a MUST WID [2] has been approved. According to the WID, a MUST UE receiver is assumed to be capable to cancel or suppress intra-cell interference between co-scheduled MUST users for the following cases:

CASE-1: Superposed PDSCHs are transmitted using the same transmission scheme and the same spatial precoding vector

CASE-2: Superposed PDSCHs are transmitted using the same transmit diversity scheme.

CASE-3: Superposed PDSCHs are transmitted using the same transmission scheme, but their spatial precoding vectors are different.

In the previous RAN1 meeting #85, it has been agreed in [1] that for Case 1 and 2, and for each combination of modulation orders:

- The number of power ratios generating non-uniform composite constellation should be chosen from 0 (for some combinations, if any), 1, 2 or 3.

  - The details are FFS.

- Power ratios generating non-uniform composite constellation should be selected from the range [0.7, 0.95].

  - The values of power ratio is FFS.

  - 0.7 should be excluded in case of 64QAM (for near UE) + QPSK (for far UE).

In addition, it has been agreed that modulation order of MUST-far UE will be only QPSK. In this paper we study the implementation of Category 2 MUST at the transmitter and further down-selection of power ratios for CASE-1 and CASE-2 schemes.

# 2 	Category 2 MUST operation

The outcome of the study item defines Category 2 MUST as a scheme which jointly maps coded bits of two or more UEs to component constellations which are superposed with adaptive power ratio. The assignment of label bits to UEs is done on the composite constellation which also exhibits Gray mapping property. An implementation of Category 2 MUST at eNB for closed-loop transmission scheme (CASE-1) is illustrated in Figure 1, however similar figures can be constructed as well for rank-2 large delay CDD (CASE-1) and transmit diversity (CASE-2). According to Figure 1, the signals are superposed in the bit domain, by mapping MUST-far and MUST-near UEs layer-bits on the super-constellation. As a consequence, after modulation, the CASE1 MUST forms a single layer, which is further spatially precoded and transmitted to the UE.

Figure 1 Category 2 with closed-loop transmission scheme

At least contributions [4][5] from RAN1 #85  observed that for each modulation order combination (QPSK-QPSK, QPSK-16QAM and QPSK-64QAM) different set of power ratios is beneficial. In addition, different number of power ratios for each modulation order combination can be defined, because some modulation order combinations are selected more often than the other, as observed in [4].

Proposal-1: Define a set of super-constellations used by eNB to superpose Near- and Far-UE.

In order to allow more than one power-ratio per MO combination and at the same time preserve the benefits of using existing LTE uniform constellations, eNB may allocate only a subset of all available bits/constellation-points within the existing LTE constellation.  Table 1 shows all examples of allocating QPSK+QPSK, QPSK+16QAM and QPSK+64QAM superposition within 16QAM, 64QAM and 256QAM. The power ratios in Table 1 are compliant with RAN1 agreement, i.e. are between 0.7 and 0.95. Native uniform constellations are marked blue and allocate all constellation points within quadrant, while green constellations allocate a subset of points (marked with the fill in Table 1). Note that in Table 1, when subset of constellation points is used, the sum of modulation orders of near and far-UE does not sum-up to the order of LTE constellation.

Reuse of LTE uniform constellations has several advantages. The legacy bit to symbol modulator can be reused at the eNB and existing EVM requirements for LTE constellations can be reused. Furthermore, significant complexity reduction can be achieved at the UE side, as discussed in [6].

Table 1  An example of bit allocations within existing LTE 16, 64 and 256QAM constellations

| LTE constellation | Near UE MO | Allocated subset of constellation-points along one dimension of a quadrant | Far UE pow. ratio | Power ratio normalized | Expected power [dB] |
| --- | --- | --- | --- | --- | --- |
| 16QAM | 2 | ∎∎ | 4/5 | 0.8/0.2 | 0 |
| 64QAM | 2 | ∎∎ | 25/29 | 0.86207/0.13793 | 1.4018 |
| 64QAM | 2 | ∎∎ | 16/17 | 0.94118/0.058824 | -0.9177 |
| 64QAM | 2 | ∎∎ | 9/13 | 0.69231/0.30769 | -2.0828 |
| 64QAM | 4 | ∎∎∎∎ | 16/21 | 0.7619/0.2381 | 0 |
| 256 QAM | 4 | ∎∎∎∎ | 100/105 | 0.95238/0.047619 | 0.9177 |
| 256 QAM | 4 | ∎∎∎∎ | 64/69 | 0.92754/0.072464 | -0.9057 |
| 256 QAM | 4 | ∎∎∎∎ | 36/41 | 0.87805/0.12195 | -3.1664 |
| 256 QAM | 4 | ∎∎∎∎ | 49/69 | 0.71014/0.28986 | -0.9057 |
| 256 QAM | 6 | ∎∎∎∎∎∎∎∎ | 64/85 | 0.75294/0.24706 | 0 |
| 256 QAM | 2 | ∎∎ | 121/137 | 0.88321/0.11679 | 2.073 |
| 256 QAM | 2 | ∎∎ | 100/109 | 0.91743/0.082569 | 1.0801 |
| 256 QAM | 2 | ∎∎ | 81/97 | 0.83505/0.16495 | 0.57353 |
| 256 QAM | 2 | ∎∎ | 64/89 | 0.7191/0.2809 | 0.19971 |
| 256 QAM | 2 | ∎∎ | 81/85 | 0.95294/0.047059 | 0 |
| 256 QAM | 2 | ∎∎ | 64/73 | 0.87671/0.12329 | -0.66096 |
| 256 QAM | 2 | ∎∎ | 49/65 | 0.75385/0.24615 | -1.1651 |
| 256 QAM | 2 | ∎∎ | 49/53 | 0.92453/0.075472 | -2.0514 |

# 3 	Power ratios down-selection

In order to decide on the preferred set of power ratios, we have simulated full-buffer traffic according to assumption summarized in [3].  In addition, we consider CASE1-2 assumptions according to WID [1], where Category 2 MUST is assumed and RML being a choice of a receiver. Table 2 shows the close to uniformly quantized set of super-constellations cases, ranging MUST-near UE powers from 0.1-0.3. Each modulation order combination has 4 power ratios.  Table 3 shows a subset of super-constellation cases from Table 1, which stem from LTE constellations by reusing the subset of constellation points within the constellation. In Table 3, modulation order combinations QPSK+16QAM and QPSK+QPSK have 4 power ratios, while QPSK-64QAM has only one power ratio.

Table 2 Close to uniformly quantized set with 12 cases

| Case | Near UE power | Super-constellation | Far-MOD | Near-MOD | Sel. Prob. |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.10 | NonUni 16QAM | 2 | 2 | 0.0469 |
| 2 | 0.15 | NonUni 16QAM | 2 | 2 | 0.0442 |
| 3 | 0.20 | LTE 16QAM | 2 | 2 | 0.0412 |
| 4 | 0.30 | NonUni 16QAM | 2 | 2 | 0.0137 |
| 5 | 0.10 | NonUni 64QAM | 2 | 4 | 0.0683 |
| 6 | 0.15 | NonUni 64QAM | 2 | 4 | 0.1733 |
| 7 | 0.238 | LTE 64QAM | 2 | 4 | 0.2239 |
| 8 | 0.30 | NonUni 64QAM | 2 | 4 | 0.0321 |
| 9 | 0.10 | NonUni 256QAM | 2 | 6 | 0.0148 |
| 10 | 0.15 | NonUni 256QAM | 2 | 6 | 0.0706 |
| 11 | 0.20 | NonUni 256QAM | 2 | 6 | 0.1209 |
| 12 | 0.247 | LTE 256QAM | 2 | 6 | 0.1503 |

Table 3 Selected subset of cases from Table 1

| Case | Near UE power | Super-constellation |  | Far-MOD | Near-MOD | Sel. Prob. |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.2 | LTE 16QAM |  | 2 | 2 | 0.0462 |
| 2 | 0.13793 | LTE 64QAM |  | 2 | 2 | 0.0581 |
| 3 | 0.058824 | LTE 64QAM |  | 2 | 2 | 0.0262 |
| 4 | 0.30769 | LTE 64QAM |  | 2 | 2 | 0.0101 |
| 5 | 0.2381 | LTE 64QAM |  | 2 | 4 | 0.2456 |
| 6 | 0.072464 | LTE 256QAM |  | 2 | 4 | 0.0500 |
| 7 | 0.12195 | LTE 256QAM |  | 2 | 4 | 0.1958 |
| 8 | 0.28986 | LTE 256QAM |  | 2 | 4 | 0.0460 |
| 9 | 0.24706 | LTE 256QAM |  | 2 | 6 | 0.3219 |

Table 4 shows the full-buffer performance of 2Tx CASE 1 MUST with 10UEs per cell, while selection statistics for each set are part of Table 2 and 3.

One can see that the set from Table 3 performs as good as the set from Table 2. The set from Table 3 should be further carefully down selected to find the optimum trade-off between number of power scalers and performance. Note that having single power ratio for each constellation case causes only loss of 1.2% in cell-average and 6.5% in coverage.

Observation-1: Close to uniformly quantized power ratios in Table 2 perform as good as power ratios resulting from uniform LTE constellations in Table 3.

Observation-2: The power ratios in Table 3 should be further carefully down-selected to find the optimum trade-off between number of power ratios and performance.

Proposal-2:  For Case 1/2 MUST adopt subset of power ratios from Table 3.

Table 4 Full buffer results, 2x2 MIMO

| Throughput (bps) | Baseline | MUST Category 2, RML |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Single power per MO combination | Gain |  | Set from Table 3 | Gain | Set from Table 2 | Gain |
| Cell average | 1.47E+07 | 1.585E+07 | 7.6 % |  | 1.602E+07 | 8.8% | 1.599E+07 | 8.5% |
| Cell edge | 2.71E+05 | 3.14E+05 | 15.6% |  | 3.31E+05 | 22.1% | 3.27E+05 | 20.7% |
| Note: | Same-beam pairing only, legacy feedback, rank1 only |  |  |  |  |  |  |  |

# 4	Conclusions

In this contribution we have been presenting views with respect to the Category 2 superposed transmission schemes. The following observations can be summarized.

Proposal-1: Define a set of super-constellations used by eNB to superpose Near- and Far-UE.

Observation-1: Close to uniformly quantized power ratios in Table 2 perform as good as power ratios resulting from uniform LTE constellations in Table 3.

Observation-2: The power ratios in Table 3 should be further carefully down-selected to find the optimum trade-off between number of power ratios and performance.

Proposal-2:  For Case 1/2 MUST adopt subset of power ratios from Table 3.

# 5 References

- Chairman notes, RAN1 # 85, Nanjing, China, 23rd - 27th May 2016

- RP-160680, “DL Multiuser Superposition Transmission for LTE”, Mediatek, CMCC,  RAN #71, Gothenburg, Sweden, 7 - 10 March 2016

- TR36.859, “Study on Downlink Multiuser Superposition Transmission (MUST) for LTE (Release 13)”, 3GPP LTE

- R1-165136, “MUST Case 1 and 2 schemes”, Nokia, ASB, RAN1 # 85, Nanjing, China, 23rd - 27th May 2016

- R1-164997, “On transmission power ratios for MUST”, Sharp,  RAN1 # 85, Nanjing, China, 23rd - 27th May 2016

- R1-164435,” Power ratio design for MUST”, QC, Sharp,  RAN1 # 85, Nanjing, China, 23rd - 27th May 2016

# Appendix

Table 5 Simulation Assumptions

| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| --- | --- |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU Uma |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2Tx (0.5 lambda), cross-polarized UE: 2Rx (0.5 lambda), cross-polarized |
| Number of UEs per cell | 10 |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| Minimum distance from macro-cell to UEs | 35 m |
| Traffic model | Full buffer |
| UE receiver | MMSE-IRC, Wishart modelling of interference covariance + RML per layer |
| Transmission mode | 2x2 TM4 (rank1 only) |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Cell selection criteria | RSRP |
| Handover margin | 3 dB |
| Scheduling algorithm | Proportional fairness maximization |
| HARQ | Redundancy Version |
| Feedback | WB rank1 only |
| CQI quantization | Yes |
| Codebook | 2Tx LTE Rel. 8 |
| Power ratio sets | According to super-constellation |
| OLLA | Yes |
| Number of superposed signals in superposition transmission | 2 |
| Channel Estimation | Realistic |
| EVM | Tx/Rx 8/4% |