3GPP TSG RAN WG1 Meeting #86		                                                  R1-167782

Gothenburg, Sweden, 22-27 August, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0269 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on DL Gap on NPDSCH TS 36.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IoT |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - The condition of applying of DL gap in NPDSCH is not correctly captured, as dl-GapThreshold is the threshold on the maximum number of repetitions configured for NPDCCH before application of DL transmission gap configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the PRB positions that can be used for anchor carriers for NB-IoT in-band deployment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - There will be different interpretations when 36.331 is read together with 36.211. Therefore, it may cause different implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

#### 10.2.3.4	Mapping to resource elements

#### < Unchanged parts are omitted >

The NPDSCH transmission can be configured by higher layers with transmission gaps where the NPSDCH transmission is postponed.  There are no gaps in the NPDSCH transmission if where  is given by the higher layer parameter dlGap-Threshold and  is given by [4].

#### < Unchanged parts are omitted >