3GPP TSG RAN WG1 Meeting #86		                                                  R1-167989

Gothenburg, Sweden, August 22-26, 2016                                   Revision of R1-166183

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0689 | rev | 1 | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on random access procedure for NB-IoT on TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IoT |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - The DCI format of NPDCCH order should be N1. - It is stated in section 16.3.2 that for NPRACH transmission following a “PDCCH order”, “a repetition number () for NPRACH determined by the repetition number field () in the corresponding DCI”, however, it is not specified how to relate the 2-bit to the exact number of NPRACH repetitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 16.3.2, change format “N0” to format “N1”. - In section 16.3.2, add a mapping table for and . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - TS 36.213 is inconsistent with TS 36.212 and agreements on NPDCCH order. - For NPRACH transmission following a “PDCCH order”, it is not specified how to relate the 2-bit in DCI to the exact number of NPRACH repetitions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 16.3.2	Timing

#### < Unchanged parts are omitted >

In case a random access procedure is initiated by a "PDCCH order" ending in subframe n, the UE shall, if requested by higher layers, start transmission of random access preamble at the end of the first subframe , , where a NPRACH resource is available. The "PDCCH order" in DCI format N1indicates to the UE,

- allocated subcarrier for NPRACH, where  is the subcarrier indication field in the corresponding DCI, is reserved.

- a repetition number () for NPRACH determined by the repetition number field () in the corresponding DCI according to Table 16.3.2-1 where R1, R2 (if any) and R3 (if any) are given by the higher layer parameter numRepetitionsPerPreambleAttempt for each NPRACH resource, respectively. R1 < R2 <R3.

Table 16.3.2-1: Number of repetitions () for NPRACH following a “PDCCH order”

| 0 | R1 |
| --- | --- |
| 1 | R2 |
| 2 | R3 |
| 3 | Reserved |

#### < Unchanged parts are omitted >