3GPP TSG RAN WG1 Meeting #86	R1-1660XX

Gothenburg, Sweden, Aug 22 - 26, 2016

Agenda Item:	8.1.2.2

Source:	Huawei, HiSilicon, CMCC, Samsung, CATT

Title:	Way forward on physical layer abstraction for UL non-linear receiver

Document for:	Discussion and decision

# Background

In RAN1#84bis meeting [1], it was agreed that

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases;

- SLS is used for comparison of proposals, and verification with traffic/scheduling/multi-cell interference dynamics.

And in RAN1#85, it was further agreed that

- PHY abstraction (L2S mapping) of LLS is encouraged to be provided for calibration purpose

It is easy for linear receiver modeling since the post-processing SINR (pp-SINR) for each user/data layer can be directly calculated. However, when more advanced non-liner multi-user detector is considered, such as ML/R-ML/MPA, MMSE-SIC, or ESE-PIC, there is no direct way of calculating the pp-SNR for each user/data layer.

# Observations

In [2] and [3], some PHY abstraction approaches for ML/R-ML receiver have been provided for downlink (DL) case, but not for UL case. So the PHY abstraction methodology of UL LLS for non-orthogonal MA with non-linear receiver should be provided.

In [4][5][6], one PHY abstraction methodology has been proposed and verified for UL non-orthogonal MA schemes with at least ML/R-ML type receivers.

# Proposals

Proposal 1: The following methodology could be considered for PHY abstraction in SLS evaluation of NR MA schemes with at least ML type receiver.

Let Nr denote the receiver antenna number and Sn be the signature of the nth user/data layer, this method can be described in the following steps.

- Step 1: Express the UL transmission model with N non-orthogonal users/date layers of the kth RE group (each group contains NF REs, NF is the spreading factor, i.e., NF≥1) as below:

yk=n=1Nhk,nxk,n+wk=Hkxk+wk,                         (1)

where yk is a NFNr×1 received symbol vector and xk is a N×1 vector of transmitted symbols. hk,n denotes the effect channel of the nth user, taking into account both channel realizations and MA signatures such that hk,n=hk,n,1T, ⋯,hk,n,NrTT, where hk,n,r= hk,n,r⊙Sn and ⊙ denotes the component-wise multiplication, and therefore Hk=[hk,1, ⋯,hk,N]. wk represents the AWGN noise plus inter-cell interference vector with covariance matrix Rk,ww. The calculation of Rk,ww is related to the signature of specific MA scheme and its modeling method.

- Step 2: Calculate the post-processing pp-SINR of the kth RE group with perfect interference cancellation (PIC) bound for the nth user/data layer [4][5][6].

γk,nPIC=hk,nHRk,ww-1hk,n,                                 (2)

- Step 3: Do the effective SNR mapping (ESM), using the pp-SINRs and the curve fitting parameter β. With ESM functions such as

  - Shannon capacity formula [4], i.e.,

Cn(γneff)=log2(1+γneff)=1Kk=1Klog2(1+βγk,nPIC),   or,           (3)

Cn(γneff)=log2(1+γneff)=1Kk=1Kβlog2(1+γk,nPIC)                 (4)

which implies

γneff=21Kk=1Klog2(1+βγn,kPIC)-1, or,                              (5)

γneff=21Kk=1Kβlog2(1+γn,kPIC)-1                                  (6)

  - Received bit mutual information rate (RBIR) Method I [7][4][5][6], i.e.,

RBIR(γneff,Q)=1Kk=1KRBIRβγk,nPIC,Q ,or                      (7)

RBIR(γneff,Q)=1Kk=1KβRBIRγk,nPIC,Q , or                      (8)

which implies

γneff=RBIR-11Kk=1KβRBIRγk,nPIC,Q,Q, or                       (10)

γneff=RBIR-11Kk=1KRBIR(βγk,nPIC,Q),Q, or                       (11)

  - Received bit mutual information rate (RBIR) Method II [7][5], i.e.,

Ck,nγk,nMA=log21+γk,nMA=βlog21+γk,nPIC→γk,nMA=(1+γk,nPIC)β-1      (12)

RBIR(γneff,Q)=1Kk=1KRBIRγk,nMA,Q ,                        (13)

which implies

γneff=RBIR-11Kk=1KRBIRγk,nMA,Q,Q,                         (14)

where Q is the modulation order. The curve fitting parameter(s) βopt should be chosen to minimize the mean square errors (MSE) between the BLERs derived from real mutli-user UL LLS evaluation and the ones from the PHY abstraction prediction, under the given number of active users and the given MCSs.

- Step 4: Get the BLER value by looking up the SISO AWGN link performance table with the derived effective SINR value in step 3 as the input.

Proposal 2: For the purpose of accuracy, multiple β values for different SNR ranges could be considered.

Proposal 3: For cross calibration of PHY abstraction for different MA schemes, at least information in Table 1 is encouraged to be provided for the other interested companies to re-produce the results.

Table 1. Suggested parameters for cross calibration

| Items to provide | Format description |
| --- | --- |
| RBIR tables | xx.mat |
| Fitting parameter table(s), i.e, β table(s) | xx.mat |
| SISO/SIMO AWGN link performance tables | xx.mat |
| Transceiver description (including transmit MA signature set, and receiver algorithm and number of iterations) | Doc description |

# References

[1] 3GPP Chairman Notes, RAN1#84bis, Busan, Korea, April 2016.

[2] 3GPP 36.866, “Study on network-assisted interference cancellation and suppression (NAICS) for LTE (Release 12)”.

[3] 3GPP 36.859, “Study on Downlink Multiuser Superposition Transmission (MUST) for LTE (Release 13)”.

[4] R1-166096, “SLS methodology for MA evaluation in mMTC scenario”, RAN1#86, Huawei, HiSilicon, Gothenburg, Sweden, August 2016.

[5] R1-166751, “System level performance evaluation for non-orthogonal multiple access”, RAN1#86, Samsung, Gothenburg, Sweden, August 2016.

[6] R1-166470, “Initial SLS results of PDMA”, RAN1#86, CATT, Gothenburg, Sweden, August 2016.

[7] IEEE 802.16m, Evaluation Methodology Document (EMD).