3GPP TSG RAN WG1 #86bis								R1-1609131

Lisbon, Portugal, 10th – 14th October 2016

Agenda item:	8.1.7.1

Source: 	Samsung

Title:					DCI Formats and Contents for NR

Document for:	Discussion and Decision

Introduction

This contribution discusses DCI contents and relevant DCI formats for NR. In LTE, multiple DCI formats were defined and each DCI format consists of various information fields including information for DL/UL scheduling, HARQ procedure, feedback, power control, carrier aggregation, etc. NR has some different requirements for control signaling compared to LTE and RAN1 needs to discuss what contents will be included in DCI and the types of DCI formats. The following agreements and observations made in previous RAN1 meetings should also be considered in the design of DCI.

RAN1#85 Agreements:

- At least the following should be supported for NR in a frequency portion

  - A time interval X which can contain one or more of the following

    - DL transmission part

    - Guard

    - UL transmission part

  - FFS which combinations are supported and whether they are indicated dynamically and/or semi-statically

  - Furthermore, the following is supported

    - The DL transmission part of time interval X to contain downlink control information and/or downlink data transmissions and/or reference signals

    - The UL transmission part of time interval X to contain uplink control information and/or uplink data transmissions and/or reference signals

  - FFS length(s) of time interval X

  - FFS: other characteristics of time interval X

  - Note: The usage of DL and UL does not preclude other deployment scenarios e.g., sidelink, backhaul, relay

RAN1#85 Agreements:

- At least the following is supported for NR frame structure

  - Following timing relationships are indicated to a UE dynamically and/or semi-statically

    - Timing relationship between DL data reception and corresponding acknowledgement

    - Timing relationship between UL assignment and corresponding UL data transmission

  - Following timing relationship is FFS whether fixed and/or dynamically and/or semi-statically indicated

    - Timing relationship between DL assignment and corresponding DL data reception

  - For above two sub-bullets

    - Potential values for each timing relationship has to be studied further considering e.g., UE processing capability, gap overhead, UL coverage, and etc.

    - Default value, if any, for each timing relationship is FFS.

RAN1#86 Agreements:

- The following is supported for NR

  - From UE perspective, HARQ ACK/NACK feedback for multiple DL transmissions in time can be transmitted in one UL data/control region is supported

  - Some or all of the following timing relationships can be indicated to a UE dynamically by the L1 DL signaling (FFS: explicit or implicit)

    - Timing relationship between DL data reception and corresponding acknowledgement

    - Timing relationship between UL assignment and corresponding UL data transmission

    - Note: Default value, if any, for each timing relationship is FFS (agreement from RAN1 #85)

    - Note: Potential values for each timing relationship has to be studied further considering e.g., UE processing capability, gap overhead, UL coverage, and etc. (agreement from RAN1 #85)

    - Note: Other means of indicating the timing relationship are not precluded

  - Some or all of the following timing relationships can be indicated to a UE semi-statically (FFS: explicit or implicit)

    - Timing relationship between DL data reception and corresponding acknowledgement

    - Timing relationship between UL assignment and corresponding UL data transmission

    - Note: Default value, if any, for each timing relationship is FFS (agreement from RAN1 #85)

    - Note: Potential values for each timing relationship has to be studied further considering e.g., UE processing capability, gap overhead, UL coverage, and etc. (agreement from RAN1 #85)

    - Note: Other means of indicating the timing relationship are not precluded

RAN1#86 Observations:

- At least followings are potentially identified as mean to realize the short processing delay and RAN1 will continue to study it

  - Frequency-first and time-second mapping

  - Code block segmentation that facilitates symbol-by-symbol processing

  - Reduce symbol durations

  - DL assignment and UL grant before and/or at the beginning of its scheduled data duration

  - TBS restriction

  - Timing advanced restriction

  - Channel coding

  - Front loaded RS mapping for control channel

  - Grant free transmission

DCI Contents

In LTE, there are various DCI formats including DCI formats for UL or DL, for configured transmission mode, UE-common vs. UE-specific, etc. This section discusses potential information that can be included in UE-specific NR DCI for UL grant and DL assignment. Common DCI for group of UEs is discussed in our companion contribution [2].

For the discussion of NR DCI, LTE DCI could be the starting point. Table 1 summarizes the required DCI information fields for UL grant (DCI format 0) in LTE and considerations for NR DCI.

| DCI information field for PUSCH | LTE | NR considerations |
| --- | --- | --- |
| Carrier indicator | Only when CA is enabled | Needed if CA is applied |
| Flag for format0/1A | To indicate one of the two formats | Needed if DCI lengths for multiple formats are the same |
| Frequency hopping flag | To enable frequency hopping | Needed if frequency hopping is applied |
| Resource block assignment | Type 0 for a single cluster, type 1 for two clusters | May be needed but detailed RA types are FFS depending on UL waveform and support of multiple numerologies |
| Modulation and coding scheme and redundancy version | 5 bits are used for MCS and RV indication | MCS is needed, RV may be separately indicated for asynchronous HARQ |
| New data indicator | To support synchronous HARQ (No HARQ process number required) | Needed, (HARQ process number is also needed, see below) |
| TPC command | For UL power control | Needed |
| Cyclic shift for DMRS and OCC index | For orthogonal DMRS/OCC allocation between UEs | Depends on DMRS structure |
| UL index | Only for TDD UL/DL configuration 0 | Depends on TDD configuration |
| Downlink assignment index | Only for TDD | Depends on TDD configuration |
| CSI request | For aperiodic CSI measurement | Needed |
| SRS request | For SRS transmission | Needed |
| Resource allocation type | For indicating RA types | Needed if there are multiple RA types |
| HARQ process number | N.A. until Rel-13 for non-MTC UEs | Needed in order to support asynchronous HARQ |
| Redundancy Version | N.A. until Rel-13 for non-MTC UEs | Needed for asynchronous HARQ |
| UL data transmission timing | N.A. | Needed when dynamic configuration for UL scheduling timing is supported |
| Others | N.A. | Depending on detailed NR design aspects, e.g., support of multiple numerologies, multi-slot scheduling, etc. |

Table 1: DCI for UL grant

As shown in Table 1, many of the current LTE DCI information fields can be reused for NR DCI. Additional information should be included such as HARQ process number, redundancy version, etc. Furthermore, timing relationship between UL assignment and corresponding UL data transmission can be included in the DCI for flexible UL scheduling as agreed in last RAN1 meeting. In addition, other information can be included in the DCI depending on the detailed NR design.

In Table 2, required information fields for DL assignment are summarized based on LTE DCI format 2/2A/2B/2C/2D and considerations for NR DCI are also added.

| DCI information field for PDSCH | LTE | NR considerations |
| --- | --- | --- |
| Carrier indicator | Only when CA is enabled | Needed if CA is applied |
| Resource allocation header | For indicating RA types | Needed if there are multiple RA types |
| Resource block assignment | Type 0/1/2 are defined | Needed but detailed RA types are FFS considering NR aspects, e.g., RB sizes, various numerologies, etc. |
| TPC command for PUCCH | For PUCCH power control | Needed |
| Downlink Assignment Index | Only for TDD | Depends on TDD configuration and on HARQ-ACK transmission method |
| HARQ process number | 3 bits | Needed but number of bits depends on the max HARQ processes (could be smaller than 3) |
| Transport block to codeword swap flag | 1 bit | FFS |
| Modulation and coding scheme | MCS level | Needed |
| New data indicator | For HARQ | Needed |
| Redundancy version | For HARQ | Needed |
| Precoding information | For CRS based MIMO operation | Not needed, no CRS in NR |
| Antenna ports, scrambling identity and number of layers | For DMRS based MIMO operation | Needed |
| SRS request | For SRS transmission | Needed |
| PDSCH RE mapping and PQI | For QCL | Needed |
| HARQ-ACK resource offset | For EPDCCH | FFS |
| HARQ timing | N.A. | Needed if dynamic configuration for DL HARQ timing is supported |
| DL data transmission timing | N.A. | Needed if dynamic configuration for DL scheduling timing is supported |
| Others | N.A. | Depending on detailed NR design |

Table 2: DCI for DL assignment

Also for DL assignment, many of the current LTE DCI information fields can be reused for NR DCI. However by the difference between LTE and NR, some of the information in LTE DCI fields would not be required for NR DCI such as precoding information. In addition, timing relationship between DL assignment and corresponding HARQ acknowledgement can be included in the DCI for the flexible HARQ operation as agreed in last RAN1 meeting.

Observation: Many of the current LTE DCI information can be reused for NR DCI

Proposal 1: At least the following information are considered for  inclusion in NR DCI formats

    - DCI for UL grant: HARQ process number, RV, UL data transmission timing

    - DCI for DL assignment: DL HARQ timing, DL data transmission timing

Number of DCI Formats

LTE supports multiple DCI formats: DCI format 0/4 for UL grant, DCI format 1/1A/1B/1C/1D/ 2/2A/2B/2C/2D for DL assignment, DCI format 3/3A for UL power control, DCI format 5 for sidelink, and DCI format 6 families for BL/CE UEs. The number of DCI formats reflects different applications but also reflects different functionalities that were gradually introduced through different LTE releases.

Considering DCIs for DL assignments where the largest proliferation of DCI formats exists in LTE, different DCI formats are configured depending on the configured transmission mode and at a given time, a UE monitors two UE-specific DCI formats (format 1A and one other DCI format) for ‘normal’ transmission scheme and for fall back transmission scheme. DCI payloads are implicitly configured according to a configured transmission mode. However, this implicit association does not seem to be necessary. Indeed, necessary fields have been added to an existing DCI format rather than creating a new DCI format. For example, a carrier indication field (CIF) was added to an existing DCI format when the UE is configured with cross carrier scheduling for carrier aggregation in LTE. A similar approach can be applied for all other features. Hence, it does not seem to be necessary to make an implicit relation between a transmission mode with a DCI format; instead, it is possible to configure a transmission mode UE-specifically and further configure additional necessary features, e.g., number of supported layers, TxD-vs-SM, and so on. Considering fall back transmission, at least two DCI formats may be configured for a UE at a given time. By using this approach, a gNB can flexibly configure two DCI formats for dynamic switching and the specification only needs to describe a single TM with the flexibly configured features. The DCI payloads in a UE-specific search space will be UE-specifically determined according to the configured features and no padding bits are necessary in principle. This framework also naturally provides forward compatibility, in that any new features that will be introduced will be supported with just changing or inserting new fields in a DCI format.

Proposal 2: Number of NR DCI formats should be minimized as much as possible

Conclusions

This contribution discusses the design aspects for NR DCI contents and gives followings observations and proposals.

Observation: Many of the current LTE DCI information can be reused for NR DCI

Proposal 1: At least the following information fields are considered for inclusion in NR DCI formats

    - DCI for UL grant: HARQ process number, RV, UL data transmission timing

    - DCI for DL assignment: DL HARQ timing, DL data transmission timing

Proposal 2: Number of NR DCI formats should be minimized as much as possible

References:

- RAN1 #86 Chairman note,

- R1-1609130, Common Control Signaling, Samsung

- R1-1609086, Discussion on transmission schemes, transmission modes and control signaling, Samsung

- R1-1609128, DL Control Channel Design, Samsung

- R1-1609129, UL Control Channel Design, Samsung