3GPP TSG RAN WG1 Meeting #86bis                                     	    R1-1609149

Lisbon, Portugal 10th - 14th October 2016

Agenda item:	7.2.2.4

Source:	NEC

Title:               Proposals for CSI-RS with CDM4

Document for:	Discussion and Decision

- Introduction

In RAN1#86 meeting, there was one agreement on NP CSI-RS aggregation as following [1]

Agreements: - For {24, 32} ports, ∑k Mk ∈ {24, 32}, Mk = 8, where Mk is the same for all k - For {20, 28} ports, ∑k Mk ∈ {20, 28}, Mk = 4, where, at least for CDM-2, Mk is the same for all k - FFS whether Mk = 8 is also supported, and if so, whether Mk is the same or different for different k for CDM-4 - FFS port indexing - In Rel-14, CDM-2 and CDM-4 is supported. - For CDM-2, port numbering in Rel-13 is reused in order to share CSI-RS with legacy UEs. - FFS CDM-4 port numbering

And for the CSI-RS overhead reduction, the agreements were as follows:

Agreement: At least for Class A NZP CSI-RS with more than 16 CSI-RS ports: - All ports in a CSI-RS resource are transmitted within the same subframe - i.e. CSI-RS overhead reduction is done in the frequency domain - CSI-RS density d ∈ {1,1/2, and at least one other value <= 1/3} RE/RB/port - Other values of d are not precluded (e.g. 2/3, ¾) - FFS whether different ports in a CSI-RS resource may have different densities - FFS PDSCH rate matching in the REs in PRBs with no CSI-RS ports within a group - Opt-1: comb like transmission - Opt-2: frequency domain measurement restriction - FFS the detailed signalling design

Agreement: Overhead reduction for Class B – both for periodic and aperiodic CSI-RS - Support CSI-RS frequency-domain density reduction for Class B - All antenna ports of a single CSI-RS resource configuration can be transmitted every N PRBs - N = 1 (existing CSI-RS design), 2, and >2 - FFS: Exact mechanism

In this contribution, we provide our views for CSI-RS with CDM4.

- Discussion

  - Resource configuration for 20 and 28 ports with CDM4

In Rel-13, CSI-RS with CDM4 was supported for better power utilization, 4 ports are transmitted on the same 4 REs, and length-4 CDM sequences are used to distinguish the 4 ports. While the use of CDM4 is based on the assumption of the channel on the 4 REs are same, so it’s better to use the closest 4 REs for CDM4 resource configuration.

And it was agreed in Rel-13 that the CDM4 CSI-RS based on legacy 8-port will use the closest 4 REs, and CDM4 CSI-RS based on legacy 4-port will use the separate 4 REs [2], as shown in Figure 1.

Figure 1 CDM4 based on legacy 8-port and 4-port

From the figure, we can see that CDM4 based on 8-port is better as the 4 REs are closest, so for the extended >16 ports CSI-RS, CDM4 based on 8-port should be adopted to achieve better channel estimation and power balance, i.e. four legacy 8-port CSI-RS resources are used for 32 and 28 ports, and three legacy 8-port CSI-RS resources are used for 24 and 20 ports. Note that there will be 4 REs redundant for 28 and 20 ports, while considering the better channel estimation and power balance, so the waste for remained 4 REs can be ignored. In addition, the power on these REs can be boosted on the CSI-RS REs to keep the power balance between different ports, this is very useful especially for 28 ports. And considering the overhead reduction and the channel estimation accuracy for non-precoded CSI-RS, >16 ports CSI-RS can be aggregated with adjacent 2 PRB pairs. Figure 2 shows the example for 20 and 28 ports CSI-RS with CDM4.

Figure 2 Example for 20 and 28 ports CSI-RS with CDM4 based on legacy 8-port

Based on the discussion, we propose that:

Proposal 1: Mk = 8 is used for 20 and 28 CSI-RS ports with CDM4, and the 4 redundant REs can be remained.

  - Further consideration on overhead reduction with FDM scheme

As agreed in last meeting, {20, 24, 28, 32} CSI-RS ports should be aggregated with 4 or 8 ports CSI-RS within the same subframe. Based on the FDM scheme, for example the new CSI-RS pattern can be configured as, in even PRB pairs, N1 ports are configured, and in odd PRB pairs, N2 ports are configured, and N1+N2 are aggregated to {20, 24, 28, 32} ports.

For legacy UE, the new CSI-RS REs can be configured as ZP CSI-RS. While in the releases before Rel-14, ZP CSI-RS is configured with the same pattern across the whole system bandwidth. All the CSI-RS REs configured in even and odd PRB pairs across the whole system bandwidth should be rate matched, e.g. in the worst case, configuration pattern of N1 and N2 are non-overlapping, total N1+N2 ports REs should be rate matched for legacy UE, leading to more resource waste, example can be found in Figure 3.

Figure 3 Example of 24 CSI-RS ports extension with non-overlapping REs

So it’s better to configure N1 and N2 resource pattern overlapped as much as possible. With this scheme, the new introduced CSI-RS ports have less impact on legacy configurations. In addition, with the down-selected overlapped CSI-RS configurations for {20, 24, 28, 32} ports, the signaling overhead is reduced. So based on the discussion, we propose that:

Proposal 2: The resource configuration pattern for CSI-RS antenna ports configured in different PRB pairs are better to be overlapped as much as possible.

  - CSI-RS resource swapping for power balance

As discussion in contribution [3], the power imbalance issue is mainly caused by the different number of REs for CSI-RS on different symbols. So further considering the issue of power imbalance, CSI-RS resource swapping in frequency domain is a good method to solve this issue. Figure 3 is an example of CSI-RS resource swapping in frequency domain for 24 ports.

Figure 3 CSI-RS resource swapping in frequency domain for 24 ports

From Figure 3, we can see that, with resource swapping in frequency domain, the number of REs occupied by CSI-RS in different symbols is same, naturally, the power on different CSI-RS ports are balanced, so we propose that:

Proposal 3: CSI-RS resource swapping in frequency domain should be considered for imbalance issue of power boosting.

  - Lower density of ZP-CSI-RS

As FDM based scheme of lower density NZP-CSI-RS was agreed, the number of resource elements occupied by NZP-CSI-RS in different PRBs is different, especially for 20 ports, 24 ports and 28 ports. Taking 24 ports in Figure 2 as an example, some ZP CSI-RS configuration can be used only for some PRBs.

Furthermore, considering CSI-RS overhead reduction in frequency domain for class B FD-MIMO was agreed, the lower density of ZP CSI-RS should also be introduced because lower density beamformed NZP-CSI-RS is UE specific and efficient PDSCH rate matching should be done.

While in current spec, ZP CSI-RS is configured in all PRBs within the system bandwidth, so we propose that:

Proposal 4: Lower density of ZP-CSI-RS should be introduced.

- Conclusion

In this contribution, we discussed the design for CSI-RS overhead reduction, and we propose that:

Proposal 1: Mk = 8 is used for 20 and 28 CSI-RS ports with CDM4, and the 4 redundant REs can be remained.

Proposal 2: The resource configuration pattern for CSI-RS antenna ports configured in different PRB pairs are better to be overlapped as much as possible.

Proposal 3: CSI-RS resource swapping in frequency domain should be considered for imbalance issue of power boosting.

Proposal 4: Lower density of ZP-CSI-RS should be introduced.

- References

- Chairman’s notes, 3GPP TSG RAN1#86, Gothenburg, Sweden, 22nd - 26th August, 2016

- R1-157710, "WF on FD-MIMO CSI-RS", Samsung, Ericsson, etc. 3GPP TSG RAN1#83, Anaheim, USA, 16th - 20th November 2015

- R1-166633, "Proposals for CSI-RS overhead reduction ", NEC, 3GPP TSG RAN1#86, Gothenburg, Sweden, 22nd - 26th August, 2016