3GPP TSG RAN WG1 Meeting #87	R1-1611230

Reno, USA, November 14 – 18, 2016

Agenda Item:	7.1.6.1

Source:	Huawei, HiSilicon

Title:	Evaluation of flexible duplex in indoor hotspot scenario

Document for:	Discussion and decision

Introduction

At RAN1#86bis meeting, it was agreed that [1]

- Continue study considering some or all of the following aspects:

  - Deployment scenarios/bands, same-/cross-operator considerations

  - Resource assignments and rate adaptations

  - Frame structure and HARQ/scheduling timing

  - Measurements for cross-link interference management

  - Signalling (e.g., OTA, backhaul, UE capability, etc.)

  - Cross-link interference management (IC/IS, power control, etc.)

  - Centralized vs. distributed interference/resource management

  - Beamforming/MIMO

  - Duplex modes (e.g., FDD/TDD, FDM/TDM, etc.)

  - Latency reduction

  - Whether or not LTE interference/resource management can be used as a starting point (as applicable)

  - Sensing

  - RS design

  - Advanced receiver

  - Timing alignment between DL and UL

To study the feasibility as well as to identify the challenges of flexible duplex in various NR deployment scenarios including dense urban, urban macro and indoor hotspot, system-level evaluations should be conducted based on the agreed evaluation assumptions [2][3][4]. In this contribution, some preliminary simulation results in indoor hotspot scenario are given and discussed.

Discussion

In this section, some simulation scenarios and assumptions are discussed. These key assumptions are important for identifying the technical feasibility and challenges of flexible duplex.

Network Layout

In this contribution, the focus is on indoor hotspot scenario. TRPs are placed in one 120m x 50m open office room, and all UEs are uniformed in office. Unpaired spectrums with carrier frequency 4GHz and 30GHz are considered. The deployment details can be found in Appendix B which are aligned with RAN1#86 agreements.

Resource allocation and methodology

To study the potential benefit of flexible duplex for unpaired spectrum, the baseline employs a semi-static subframe allocation, and all TRPs share the same subframe ratio allocation.

In RAN1 #86 meetings [2], it was agreed that FTP traffic model 3 with packet size 0.5M bytes can be used and the ratio of DL/UL traffic load are{1:1} ,{2:1} and{4:1}.

When the traffic ratio of DL/UL is 1:1 and 2:1, the baseline downlink-uplink subframe ratio is 6:4, and when the traffic ratio of DL/UL is 4:1, the baseline downlink-uplink subframe ratio is 4:1. For flexible duplex on unpaired spectrum, dynamic DL/UL subframe ratio allocation is used to adapt to the instantaneous DL/UL traffic need.

Based on assumptions and parameters shown in appendix, several scenarios with different TRP deployment are evaluated by varying system traffic load, ratio of DL to UL, static or adaptive DL/UL subframe configurations, as shown in Table 1.

Table 1: Simulation parameters and resource allocations

|  | Baseline (Traditional TDD) | Flexible duplex |
| --- | --- | --- |
| DL/UL subframe ratio | semi-static subframe allocation - For DL:UL traffic ratio = 1:1, DL/UL subframe = 6:4 - For DL:UL traffic ratio = 2:1, DL/UL subframe = 6:4 - For DL:UL traffic ratio = 4:1, DL/UL subframe = 4:1 | Flexible UL/DL subframe ratio allocation |
| DL/UL carrier bandwidth | 4GHz: 20MHz (DL+UL); 30GHz: 80MHz (DL+UL) |  |
| TRP Number | 3 TRP/sector per site, 3 site per room |  |

To evaluate flexible duplex with flexible resource allocation, user perceived throughput (UPT) is used as evaluation metric, same as in LTE eIMTA [5].

Modeling of cross-link interference mitigation (CIM)

In flexible duplex, cross-link interference, e.g. TRP-to-TRP and UE-to-UE interference, exists in case that neighboring cells use different transmission directions on the same time-frequency resource as shown in Figure 1.

Figure 1: Example of cross-link interference

For the indoor office scenarios with multiple TPRs in one room, cross-link interference will severely degrade the potential performance gain resulted from flexible resource allocation, especially for cases with dense TPR/UE deployment. Herein, some preliminary CIM schemes, e.g. coordinated scheduling, power control and beam-forming are considered in our evaluation.

In indoor hotspot scenario, all users are uniformed in one room with less penetration loss and high LOS probability. When two adjacent UEs separately operate with transmission and reception over the overlap time/frequency resources simultaneously, the downlink reception of one UE will be impacted by the uplink transmission of another UE. The impact may be very strong for the small distance between two users. Flexible power control and coordinated scheduling can be used to mitigate the UE-UE interference based on inter-UE interference measurement and sensing mechanisms [6]. For the TRP-to-TRP interference mitigation, beam coordination and advanced IRC receiver are an efficient way for flexible duplex [7].

Simulation results

This section provides some evaluation results for some typical traffic types. Fixed ratio of UL-DL subframes is the baseline. In flexible duplex, schedulers can change its subframe direction according to the instant traffic radios.

Table 2: DL and UL Performance deployment on 4GHz carrier with CIM

| Radio of DL/UL traffic | Feature | DL/UL subframe ratio | DL RU (%) | DL Average UPT (Mbps) | DL Average UPT Gain | UL RU (%) | UL Average UPT (Mbps) | UL Average UPT Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1:1 | Baseline | 6:4 | 15 | 35.04 | - | 11 | 43.76 | - |
|  | Flexible duplex | Dynamic |  | 51.36 | 46.54% |  | 54.56 | 24.72% |
| 2:1 | Baseline | 6:4 | 21 | 33.44 | - | 8 | 44.96 | - |
|  | Flexible duplex | Dynamic |  | 50.88 | 52.17% |  | 54.56 | 21.50% |
| 4:1 | Baseline | 4:1 | 20 | 45.36 | - | 9 | 24.8 | - |
|  | Flexible duplex | Dynamic |  | 50.24 | 10.59% |  | 53.76 | 116.97% |

Table 3: DL and UL Performance deployment on 30GHz carrier with CIM

| Radio of DL/UL traffic | Feature | DL/UL subframe ratio | DL RU (%) | DL Average UPT (Mbps) | DL Average UPT Gain | UL RU (%) | UL Average UPT (Mbps) | UL Average UPT Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1:1 | Baseline | 6:4 | 13 | 130.72 | - | 14 | 125.2 | - |
|  | Flexible duplex | Dynamic |  | 167.68 | 28.27% |  | 154.64 | 23.51% |
| 2:1 | Baseline | 6:4 | 18 | 127.68 | - | 8 | 131.12 | - |
|  | Flexible duplex | Dynamic |  | 172.64 | 35.21% |  | 151.12 | 15.29% |
| 4:1 | Baseline | 4:1 | 18 | 167.68 | - | 11 | 69.44 | - |
|  | Flexible duplex | Dynamic |  | 173.44 | 3.45% |  | 144.8 | 108.55% |

In Table 2 and Table 3, we show the uplink average UPT and downlink average UPT on carrier frequency 4GHz and 30GHz with cross-link mitigation schemes. Flexible duplex can get significant gain with dynamic DL/UL subframe ratio allocation for typical traffic ratios.

Observation 1: Flexible duplex with cross-link interference mitigation can improve the DL and UL performance in indoor hotspot scenario for 4G and 30GHz unpaired spectrum. Preliminary evaluation results shows that with flexible DL and UL transmission, it achieves significant gain in downlink (e.g., up to 52%) and in uplink (e.g., up to 116%) in indoor hotspot scenario.

UE-UE interference becomes more important in indoor scenario because of the small inter-UE distance. One uplink transmission may severely degrade neighboring downlink reception especially for cell edge users. Some preliminary UE-UE interference coordination schemes are considered in our simulations.

In appendix A, we compare the downlink wideband geometry with and without UE-UE CIM. It shows that the edge users suffer strong cross-link interference @ 30GHz. In Table 4, we show the performance improvements for edge users where UE-UE interference coordination is implemented.

Table 4: DL 5% UPT Performance with/without UE-UE interference coordination

| Carrier Frequency | Radio of DL/UL traffic | Without UE-UE interference coordination (Mbps) | With UE-UE interference coordination (Mbps) | UE-UE interference coordination Gain |
| --- | --- | --- | --- | --- |
| 30GHz | 1:1 | 45.28 | 59.04 | 30.4% |
|  | 2:1 | 46.16 | 55.04 | 19.2% |
|  | 4:1 | 57.04 | 58.48 | 2.5% |

Observation 2: In flexible duplex, edge users may suffer strong cross-link interference and UE-UE interference coordination can achieve the performance improvement.

Proposal: In flexible duplex, UE-UE cross-link interference mitigation needs to be studied.

Conclusions

In this contribution, we provided some evaluation results of flexible duplex focused on indoor hotspot scenario. The simulation result show gains for typical DL/UL traffic types.

The following observations and proposal can be made,

Observation 1: Flexible duplex with cross-link interference mitigation can improve the DL and UL performance in indoor hotspot scenario for 4G and 30GHz unpaired spectrum. Preliminary evaluation results shows that with flexible DL and UL transmission, it achieves significant gain in downlink (e.g., up to 52%) and in uplink (e.g., up to 116%) in indoor hotspot scenario.

Observation 2: In flexible duplex, edge users may suffer strong cross-link interference and UE-UE interference coordination can achieve the performance improvement.

Proposal: In flexible duplex, UE-UE cross-link interference mitigation needs to be studied.

- References

- 3GPP, Chairman's Notes, RAN1 #86bis, Oct 2016.

- R1-168053, “WF on scenarios and evaluation assumptions for flexible duplex,” RAN1#86, Gothenburg, Sweden, Aug 22 - 26, 2016.

- R1-168372, “WF on channel model issues for flexible duplex evaluation,” RAN1#86, Gothenburg, Sweden, Aug 22 - 26, 2016.

- R1-168373, “ WF on penetration issues for flexible duplex evaluation,” RAN1#86, Huawei, HiSilicon, LG Electronics, Gothenburg, Sweden, Aug 22 - 26, 2016.

- 3GPP TR 36.828 V2.0.0, “Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation,” June 2012.

- R1-1611228, “Interference sensing for flexible duplex”, RAN1#87, Huawei, HiSilicon, Reno, USA, November 14-18, 2016

- R1-1611225, “Benefits of cross-link interference mitigation,” RAN1#87, Huawei, HiSilicon, Reno, USA, 14-18 November, 2016.

- Appendix A: DL Geometry for indoor hotspot scenario

In this section, we observe the impact of traffic ratio to flexible duplex. For static TDD, DL geometry is a traditional wideband and static SINR. For UL, one UE is selected randomly per cell, and the selected UE in each cell is scheduled uplink transmission over the whole bandwidth. UE transmit power control parameters P0=75dBm and =0.8 are assumed. For flexible duplex, in order to simulate cross-link interference, DL or UL is randomly determined per cell with DL and UL selection ratio DL:UL={1:1,2:1,4:1}. UL TPC parameters are identical to the case of static TDD for simplicity. All other parameters can be found in appendix B.

Figure 2: Downlink geometry for flexible duplex with different traffic ratios on 30GHz

- Appendix B: Summary of simulation parameters

| Parameters | Indoor hotspot |
| --- | --- |
| Layout | Single layer - Indoor floor: (3 site per 120m x 50m, 3 TRP per site) |
| Inter-BS distance | 40m |
| Minimum BS-UE (2D) distance | 0m |
| Carrier frequency | 4GHz/30GHz on unpaired spectrum |
| Simulation bandwidth | 20MHz per CC for 4GHz, 80MHz per CC for 30GHz |
| Channel model | Below 6GHz: TRP-to-UE: ITU InH TRP-to-TRP: ITU InH (h_UE=3m) UE-to-UE: A.2.1.2 in TR36.843 Above 6GHz: TRP-to-UE: 5GCM Indoor-office TRP-to-TRP: 5GCM Indoor-office (h_UE=3m) UE-to-UE: 5GCM Indoor-office (h_BS=1.5m) |
| Penetration loss | Follow [5] |
| BS Tx power | Below 6GHz: 24 dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 24 dBm Above 6GHz: 23 dBm [scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 23 dBm] |
| UE Tx power | Maximum 23 dBm |
| BS antenna configuration | Below 6GHz: (M,N,P,Mg,Ng)=(4,4,2,1,1) (dH,dV)=(0.5,0.5)λ For around 30GHz: (M, N, P, Mg, Ng)=(4,8,2,1,1) (dH,dV) = (0.5, 0.5)λ |
| BS antenna configuration |  |
| BS antenna height | 3m |
| BS anteena element gain + connector loss | 5dBi |
| BS antenna tilt | 110deg |
| BS receiver noise figure | Below 6GHz: 5 dB Above 6GHz: 7 dB |
| UE antenna elements | Omni TX2RX2 for start point |
| UE antenna height | 1.5m |
| UE antenna gain | For below 6GHz: Follow the modeling of TR36.873 Above 6GHz: 5dBi |
| UE receiver noise figure | Below 6GHz: 9 dB Above 6GHz: 13 dB (baseline); |
| Traffic model | FTP traffic model 3 with packet size 0.5Mbytes For bandwidth 20M Ratio of DL/UL traffic ={1:1}with {λDL, λUL} ={0.15, 0.15}, Ratio of DL/UL traffic ={2:1}with {λDL, λUL} ={0.2, 0.1}, Ratio of DL/UL traffic ={4:1}with {λDL, λUL} ={0.25,0.0625} For bandwidth 80M Ratio of DL/UL traffic ={1:1}with {λDL, λUL} ={0.45, 0.45}, Ratio of DL/UL traffic ={2:1}with {λDL, λUL} ={0.6, 0.3}, Ratio of DL/UL traffic ={4:1}with {λDL, λUL} ={0.75,0.1875} |
| UE distribution | For FTP traffic model 3: 10 users per TRP 100% indoor (3km/h) |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS receiver | MMSE-IRC as the baseline receiver |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |
| UE association | based on RSRP measurement |
| Transmission mode | SU-MIMO with rank adaptation |