3GPP TSG RAN WG1 Meeting #87	R1-1611486

Reno, USA, 14th-18th November 2016

Agenda item:		6.2.8.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	FeMTC Search Space Design for SC-MCCH and SC-MTCH

Document for:		Discussion and Decision

# Introduction

One of the objectives of the new Release 14 WI proposal on Further Enhanced MTC [1], is the extension of SC-PTM to support eMTC devices.

At RAN1#86bis meeting [3], the following FeMTC agreements were arrived at:

•	  For the search space scheduling SC-MCCH

–	UE monitoring of blind decoding candidates is based on FFS: Type1-CSS or Type2-CSS for MPDCCH

–	Rmax, and G values are configured per SC-MCCH by higher layers

•	FFS the configuration of other parameters, e.g., narrowband.

At RAN2#96bis meeting [2], the following agreements/assumptions were arrived at:

- In Rel-14 we will not have a solution with feedback.

- SC-MCCH for feMTC and NB-IoT is scheduled dynamically.

- We consider to make the following agreement (working assumption for now):

MT (paging) vs. SC-PTM: MT (Paging) has higher priority than SC-PTM

MO (except signalling) vs SC-PTM: UE implementation

MO signalling vs SC-PTM: MO signalling has priority.

- SC-PTM is supported only in IDLE also for feMTC

In this document, we discuss both the choice of CSS type for the SC-MCCH/MTCH and the recent RAN2 working assumption prioritising Paging above SC-PTM.

# Discussion

Whilst the concept of a search space based scheduling SC-MCCH was agreed at RAN1#86bis, the equivalent agreement was not made for SC-MTCH.  Given the benefit of scheduling flexibility and also that within the nB-IoT session, a similar agreement has been made for nB-IoT (see below), it is proposed that the dynamic scheduling (i.e. using a search space) is also supported for the SC-MTCH.

Also from the RAN1#86bis NB-IoT session with

- For the search space scheduling SC-MTCH (called Type2-MSS for RAN1 purposes)

  - UE monitoring of blind decoding candidates is based on FFS: Type1-CSS or Type2-CSS

  - α, Rmax, and G values for Type2-MSS are configured per SC-MTCH in SC-MCCH

- UE is not required to simultaneously monitor Type1-MSS and Type2-MSS

- UE is not required to monitor Type2-MSS while receiving NPDSCH carrying SC-MTCH

In addition to dynamic scheduling we also support SPS scheduling [4], that also requires a search space for the activation and release of sessions.

Proposal 1:			SC-MTCH for FeMTC can be scheduled either dynamically or using SPS.

From RAN1#86bis, there is still an FFS regarding the type of CSS adopted for the new search spaces for SC-MCCH and SC-MTCH (assuming proposal 1 is accepted).

For Rel-13, 4 search spaces are defined:

- Type0-MPDCCH common search space – CSS for fall back and 3/3A

  - CE mode A only

- Type1-MPDCCH common search space – Paging

  - Fixed “2+4” PRBs per narrowband, no option to use just 2 or 4 PRBs

    - Configurable number of narrowbands for paging (SIB2 paging-narrowBands-r13)

  - UE ID determined Starting subframe/radioframe & narrowband locations

    - See TS36.304, section 7.1

  - Up to 4 Blind decode candidates (different repetitions/AL) per search space

  - P-RNTI DCI format 6-2 paging DCI limited to pointing to a 6 PRB PDSCH paging resource

- Type2-MPDCCH common search space – RAR, Msg3 retransmission, Msg4

  - Fixed “2+4” PRBs per narrowband, no option to use just 2 or 4 PRBs

    - Can configure location of up to 2 narrowbands (SIB2 mpdcch-NarrowbandsToMonitor-r13)

  - Configurable MPDCCH Starting positions

    - For Type2-common search space, is given by the higher layer parameter mPDCCH-startSF-CSS-RA-r13 (SIB2)

    - Greater than 4 Blind decode candidates (different repetitions/AL) per search spaces

- UE-specific search space

  - Configurable “2, 4, 2+4” PRBs per narrowband

  - Higher layer signalling (EPDCCH-Config) provides the startingSF (mpdcch-StartSF-UESS-r13), narrowband (mpdcch-Narrowband-r13) and number of repetitions (mpdcch-NumRepetition-r13) of the MPDCCH search space.

  - Up to 16 blind decode candidates per search space

Since reception of multi-cast is to be supported by UEs in RRC_Idle mode, the 2 most obvious candidate search spaces are the type-1 and type-2 CSSs.

- Type-1 CSS

A potential advantage of using the Type-1 CSS is that UE could monitor paging and SC-MCCH/SC-MTCH simultaneously, which should reduce UE power consumption.

However, using this CSS in this manner would require the SC-MCCH/SC-MTCH MPDCCH starting subframes to be aligned to the UE ID specific paging occasions (PO).

Disadvantages associated with trying to align paging and SC-MCCH/SC-MTCH scheduling, include:

- Restricted MPDCCH scheduling configuration (now linked to the UE ID specific Paging Occasions) which cannot easily be optimised for different SC-PTM use-cases (e.g. CE Prach level 0/1 only SC-PTM service).

- Inefficient use of network resources, since the same SC-MCCH/SC-MTCH scheduling information has to be resent for each PO.

- The restrictive paging DCI (format 6-2) which currently doesn’t support PDSCH scheduling for fewer than 6 PRBs.

- Type-2 CSS

The advantages of the Type-2 CSS over Type-1 CSS include:

- Efficient use of network resources, since the network only needs to send the same SC-MCCH/MTCH information once because all UEs now monitor the same CSS.

- Scope to differentiate SC-MCCH/SC-MTCH(s) for different use-cases by defining alternative starting subframe parameter mPDCCH-startSF-CSS-RA-r13.

- Network has more scheduling opportunities per search space due to the higher number of blind decode candidates supported

The disadvantages of the Type-2 CSS, include:

- The RRC_Idle mode UE now has to monitor 2 search spaces (paging and SC-PTM) rather than 1.

- Rule needs to be devised to handle the potential overlap of paging and SC-PTM SS.

This is discussed more below given RAN2#96bis working assumption

- Currently the Type-2 CSS assumes “2+4” PRBs, which for some SC-MTCH use cases (e.g. a service targeted for only CE PRACH level 0/1 UEs) maybe unnecessary and block resources for other UEs.

Given the discussion above, it is proposed that the SC-MCCH and SC-MTCH search space is based on the Type-2 CSS design, with additional parameters (sent in SIB20 for SC-MCCH SS and SCPTMConfiguration message for SC-MTCH SS) to allow the network to optimise the SS configuration for different use cases.

Note, in some ways a Type-2 CSS with the added options listed above is very similar to a USS.

Proposal 2: 	The SC-MCCH and SC-MTCH search spaces are based on the Type-2 CSS design, with additional parameters (sent using SIB20 for SC-MCCH SS and SCPTMConfiguration for SC-MTCH SS) to allow the network to configure:

- Separate narrowbands for each SC-MCCH/SC-MTCH

- Separate values of G for each SC-MCCH/SC-MTCH

- “2” and “4” PRB options (may only be required for SC-MTCH)

Both SC-PTM and Paging use the same mechanism of having a message scheduled by the DCI. In addition, both are applicable to UE in IDLE mode. In this case, the possible collision cases can be summarized below –

- CSS for SC-PTM overlaps with CSS for Paging

- CSS for SC-PTM overlaps with Paging message (PDSCH)

- CSS for Paging overlaps with SC-MCCH/SC-MTCH (PDSCH)

From RAN2#95bis, there is the working assumption that Paging should have higher priority than SC-PTM.

One concern with this prioritization is the reliability of SC-PTM transmission. Some use cases for SC-PTM such as firmware upgrades require a high degree of reliability.  If a paging occasion occurs during the time a UDP/IP packet is being sent, the UE may switch away from SC-MTCH reception to check for Paging message. Thus, the entire UDP/IP packet will be lost to the UE.

To prevent this issue, you would need to prioritize SC-PTM over paging. However, since RAN2 has decided as a working assumption to prioritize Paging over SC-PTM, it is to better to confirm the working assumption in order to progress the work. Therefore, it is proposed that when CSS for Paging overlaps with CSS for SC-PTM (either SC-MCCH/SC-MTCH), the UE will monitor CSS for Paging. This will ensure that Paging message has priority over SC-PTM transmissions.

However, in case of on-going PDSCH transmissions for either Paging or SC-MCCH/SC-MTCH, it is proposed that the UE does not retune away to monitor CSS for Paging/SC-PTM. This is in-line with current Rel-13 behaviour and will prevent UE from abandoning ongoing transmissions to monitor for higher priority messages.

Proposal 3: 		Define the following rules in case of overlap between SC-PTM and Paging –

- In case of overlap between CSS for paging and CSS for SC-PTM, UE will monitor CSS for paging.

- When receiving PDSCH (either Paging or SC-PTM), UE will not monitor CSS for Paging/SC-PTM.

# Conclusions

In this document, we have discussed both the choice of CSS type for the SC-MCCH/MTCH and the recent RAN2 working assumption prioritising Paging above SC-PTM.  From those discussions we have the following proposals for RAN1 to consider.

Proposal 1:	SC-MTCH for FeMTC can be scheduled either dynamically or using SPS.

Proposal 2: 	The SC-MCCH and SC-MTCH search spaces are based on the Type-2 CSS design, with additional parameters (sent using SIB20 for SC-MCCH SS and SCPTMConfiguration for SC-MTCH SS) to allow the network to configure:

- Separate narrowbands for each SC-MCCH/SC-MTCH

- Separate values of G for each SC-MCCH/SC-MTCH

- “2” and “4” PRB options (may only be required for SC-MTCH)

Proposal 3: 		Define the following rules in case of overlap between SC-PTM and Paging –

- In case of overlap between CSS for paging and CSS for SC-PTM, UE will monitor CSS for paging.

- When receiving PDSCH (either Paging or SC-PTM), UE will not monitor CSS for Paging/SC-PTM.

# References

[1]		RP-161321		New WI proposal on Further Enhanced MTC			Ericsson

[2]		R2-167242		Report from the IoT breakout sessions					3GPP TSG-RAN WG2 Meeting #95bis

[3]		Report of 3GPP TSG RAN WG1 #86bis		http://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_86b/Report/

[4]		R1-1611488		SPS Design for SC-MTCH								Nokia, Alcatel-Lucent Shanghai Bell