3GPP TSG-RAN WG1 #87	R1-1612372

14th – 18th November 2016

Reno, USA

Agenda item:	7.1.5.1

Source: 	AT&T

Title: 	Link Level Evaluation of FEC for Data Channel in NR

Document for:	Discussion/Decision

# Introduction

In RAN #71, a new study item New Radio (NR) Access Technology was approved. In RAN1 #85, and 86, 86bis several contributions showed the performance of channel coding schemes for eMBB data channel and the following agreements were made.

Agreement:

- The channel coding scheme for eMBB data is LDPC, at least for information block size > X

- FFS until RAN1#87 one of Polar, LDPC, Turbo is supported for information block size of eMBB data <= X

  - The selection will focus on all categories of observation, including overall implementation complexity, regardless of the number of coding schemes in the resulting solution (except if other factors are generally roughly equal)

- The value of X is FFS until RAN1#87, 128 <= X <= 1024 bits, taking complexity into account

- The channel coding scheme(s) for URLLC, mMTC and control channels are FFS

In this contribution, we analyze the performance of polar code using list decoding and turbo code for short block lengths. Unlike the conventional method of obtaining block error rate in AWGN channels, we computed the throughput gains achieved with the FEC schemes in fading channel for better understanding of the realistic gains.

# Simulation Assumptions

We describe our evaluations with fixed modulation/code rate and also with link adaptation, where the UE recommends the channel quality indicator through feedback channel. Feedback delay of 4 msec is assumed in our evaluations. Detailed link level simulation assumptions are shown in Table 1.

Table 1 Detailed link level simulation assumptions

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Duplex | FDD |
| System Bandwidth | 10 MHz |
| TTI length | 1 ms |
| Subcarrier spacing | 15KHz, |
| Guard time interval | 4.7us (interval of LTE normal CP) as baseline |
| FFT size | 1024 |
| Data transmission bandwidth | 6RBs for QPSK, 3 RBs for 64 QAM, 2 RBs for 64QAM- Fixed modulation, for LA, 3 RBs |
| Antenna configuration | 1T1R |
| MCS | Fixed. QPSK, 16 QAM, 64QAM: 1/2 For link adaptation: QPSK, 16-QAM and 64-QAM are considered with variable code rate |
| Control Overhead | Zero |
| Channel estimation | Ideal, Practical |
| Channel Model | TDL-A with 30 ns |
| Polar encoder design | SNR independent with fixed weights |
| Polar decoder | List based decoder with list size of 32 unless specified |

# Analysis of Simulation Results

## Fixed Modulation and Code rate

Figure 1 shows the spectral efficiency comparison of turbo and polar codes with QPSK modulation with code rate of 0.5.  The number of resource blocks are allocated is 6 which corresponds to an information block length of 1008 bits (TBS).  The spectral efficiency is computed based on the following formula

Spectral efficiency = TBS*(1-BLER)/(T*BW)

Where, TBS is the transport block size in bits, BLER is the block error rate, T is the time duration of one subframe, BW is the actual bandwidth of target subband. It can be observed that the performance of turbo code and polar code performs almost similar. However, the performance of polar code is slightly better than turbo code at low to medium SNRs.

Figure 1 Spectral efficiency comparison with fixed modulation and coding with QPSK, ½ code rate

Figure 2 shows the performance with 16 QAM, ½ code rate. In this case also we observed that the performance of polar code is slightly better than the turbo code (check SNR of 10 dB). Figure 3 shows the performance with 64 QAM, ½ code rate. In this case, we can see for example an increase in throughput for polar code over turbo code.

Figure 2 Spectral efficiency comparison with fixed modulation and coding with 16 QAM, ½ code rate

Figure 3 Spectral efficiency comparison with fixed modulation and coding with 64 QAM, ½ code rate

Figure 4 shows the performance comparison with a block length of 168 bits which corresponds to 1 resource block with QPSK, ½ code rate. It can be observed that in this case also the performance of polar code is better than turbo code.

Figure 4 Spectral efficiency comparison with fixed modulation and coding with QPSK, ½ code rate, 1 RB

### Observation 1: With fixed modulation and code rate, the performance of Polar code is slightly better than turbo code.

## Impact of list size on the decoder performance

Figure 5 shows the spectral efficiency comparison of polar codes with QPSK modulation with code rate of 0.5 and various list sizes for 6 RB allocation.  From the figure, we can observe that the performance is not impacted by the list sizes of 8 and 16. This implies that the complexity of polar decoder can be significantly reduced by using list size of 8.

Figure 5 Spectral efficiency comparison with fixed modulation and coding with QPSK, ½ code rate, 1 RB, with various list sizes

### Observation 2: Polar decoder complexity can be significantly reduced without impacting the spectral efficiency

## With adaptive modulation and code rate

Figure 6 shows the spectral efficiency comparison of polar codes and turbo codes with adaptive modulation and code rate. Note that in this case, we used fixed number of resource blocks equal to 3 and adapted the modulation and code rate. In this case, we observed that the performance of polar code is slightly better at all SNRs even with list size equal to 8.

Figure 6 Spectral efficiency comparison with adaptive modulation and coding

### Observation 3: With link adaptation, spectral efficiency is improved with polar code at all SNRs withy reduced list size

## With practical channel estimation

Figure 7 shows the performance comparison with practical channel estimation and with link adaptation.   In this case too, we observed that the performance of polar code is better than turbo at all SNRs even with list size equal to 8.

Figure 2 Spectral efficiency comparison with fixed modulation and coding

### Observation 4: With practical channel estimation, the performance of polar code is slightly better than turbo code at all SNR with list size equal to 8

### From these results we conclude that the polar code is a promising FEC technique for data channels compared to turbo code.

Proposal 1:  Polar code is preferred over turbo code for short block length

# Conclusions

In this contribution we analysed the performance of polar code and turbo code for short block lengths. Our observations are as follows:

### Observation 1: With fixed modulation and code rate, the performance of Polar code is slightly better than turbo code.

### Observation 2: Polar decoder complexity can be significantly reduced without impacting the spectral efficiency

### Observation 3: With link adaptation, spectral efficiency is improved with polar code at all SNRs withy reduced list size

### Observation 4: With practical channel estimation, the performance of polar code is slightly better than turbo code at all SNR with list size equal to 8

Based on these observations, we propose that

Proposal 1:  Polar code is preferred over turbo code for short block length