TSG-RAN WG1 #87	R1-1612904

Reno, NV, USA, November 14 – 18, 2016

Source:	Ericsson

Title:	On mini-slots

Agenda Item:	7.1.1

Document for:	Discussion and Decision

# Introduction

At RAN1#86, it was agreed that

  - Mini-slot

    - Should at least support transmission shorter than y OFDM symbols in the numerology used for transmission

    - May contain ctrl at the beginning and/or ctrl at the end

    - The smallest mini-slot is the smallest possible scheduling unit (FFS: smallest number of symbols)

which was further confirmed at RAN1#86bis by

- The NR frame structure should support both slots and mini-slots

In this document we state some further views on mini-slots, or sub-slots as has been suggested as an alternative name.

# Use cases for sub-slots

The main reason for sub-slots are to support a shorter transmission duration than a regular slot. This is useful for several reasons including

- Low-latency transmissions where the regular slot duration is too long and/or a transmission need to start “immediately” without waiting for the start of a symbol boundary

- Unlicensed spectrum where it is beneficial to start transmission “immediately” after listen-before-talk

- mm-wave bands where the large amount of bandwidth available implies that the payload supported by a one or two OFDM symbols is large enough for many of the packets. Multiplexing of different users should primarily be done in the time domain to support analog/hybrid beamforming, calling for a transmission duration shorted than a slot.

Figure 1: Examples of sub-slots.

# Characteristics of a sub-slot

A sub-slot is characterized by

- May start at ‘any’ OFDM symbol

- Can have a length up to ‘slot duration – 1’ (i.e. 14-1=13 symbols or 7-1=6 symbols, depending on the configured slot duration)

- Ends at a slot boundary at the latest (at least in case of aggregation with subsequent slots)

- Can be aggregated by a subsequent slot (to allow for longer transmission durations in case of e.g. unlicensed spectrum)

- May have DL ctrl at the beginning

- May have UL ctrl at the end (pending further study)

# Conclusion

It is proposed to adopt the characteristics above for sub-slots