3GPP TSG RAN WG1 Meeting #87	 	R1-1612932

Reno, Nevada, USA, 14th - 18th November 2016

Source: 	Ericsson

Title:	Detection Sequence in LTE-ITS Sidelink for Coexistence with Other Access Technologies

Agenda Item:	6.2.1.3.1

Document for:	Discussion and Decision

# Introduction

The V2X WID [1] includes the following objective.

“4)	To specify solution(s) facilitating long-term basis co-channel coexistence between DSRC/IEEE 802.11p and LTE PC5 for V2V operating over the same frequency channels [RAN1]

a)	This objective starts from RAN#73 and target is to complete this by RAN#74. Solution(s) to be specified should avoid negative impact on the performance of LTE PC5.”

In RAN1#85, it was agreed:

“The possible solutions identified by RAN1 for high level coexistence approaches (long-term basis) between PC5 transport for V2V services and DSRC/IEEE 802.11p services in the same channel are as follows:

  - Geo-location and database.

  - Time sharing between systems based on GNSS timing; this would require some modifications to DSRC.

  - Sensing-based vacate/switching approaches with or without transmission of a predetermined signal(s) (e.g. LTE-ITS preamble, SLSS)

    - Sensing with a predetermined signal(s) would require some modifications to DSRC.

  - RAN1 has not conducted any system-level evaluations for these solutions, although some link-level results have been provided for some solutions.”

In RAN1#86bis it was also agreed that:

- The following two aspects need to be discussed with regard to the usefulness of an detection option

  - Usefulness when IEEE802.11p detects LTE

  - Usefulness when LTE detects IEEE802.11p

- Study until the next meeting on the usefulness of the following options

  - Option 1: Detecting the pattern of LTE CP

  - Option 2: Detecting a sequence of LTE transmitted within the last symbol of a subframe

  - Option 3: Detecting periodically transmitted LTE signal (e.g., SLSS)

  - Option 4: Measuring energy

In this contribution we propose a design of the detection sequence in LTE-ITS sidelink that can be efficiently detected by 3GPP and non-3GPP technologies and discuss its detection at IEEE 802.11p and 3GPP receivers.

# Detection sequence design for LTE-ITS sidelink

During RAN1#84bis it was highlighted that the LTE signal should be easily detectable by IEEE 802.11p devices with minor changes to their implementation. In the Appendix we provide a detailed review of the IEEE 802.11p preamble design and of the detection technique that commonly assumed for IEEE devices.

We suggest the following design guidelines for the LTE-ITS detection sequences:

- Signal is generated using the typical sidelink transmitter chain

- Similar used bandwidth as 802.11p signal (8.125MHz)

- Similar signal design (short repeating sequences) in order to enable reuse of typical IEEE 802.11p receivers

- Low cross correlation with IEEE 802.11p preamble

- Availability of multiple LTE-ITS detection sequences for differentiating different ITS services (e.g., safety and non-safety) and potentially different 3GPP technologies.

We observe that the above requirements are hardly fulfilled by SLSS or other SL sequences and suggest the design of a new dedicated sequence in Section 2.1.

## Detailed Design of Detection Sequence in LTE-ITS Sidelink

A 10MHz system bandwidth is assumed in the following for the reference design. The proposal can also be easily scaled for different system bandwidths if needed.

In the frequency domain, the sidelink detection sequences rSDSm are “comb”-version of the base sequences ru,vn, 0≤n≤MSCRS-1 given in clause 5.5.1.2 of [5]. The correct sequence to be transmitted by a UE (if any) is determined by higher layers and it depends on the type of service (e.g., safety, non-safety ITS) and regional regulations outside of 3GPP scope. We consider two LTE-ITS detection sequences, one for “safety” and one for “non-safety” (more sequences might be added in the future if needed):

- LTE-ITS detection sequence 1: u=0,  v=0, MSCRS=18, comb mapping to every 29th subcarrier.

- LTE-ITS detection sequence 2: u=0,  v=0, MSCRS=15, comb mapping to every 35th subcarrier.

We are not in favour of differentiating LTE-ITS detection sequences by use of different base sequences (u,v) because a Delay and Correlate detector would be unable to distinguish between such sequences.

A symmetrical mapping around the DC seems like a natural option where the sidelink detection sequence occupies the central part of the system bandwidth. Since length-{15,18} base sequences are currently undefined in LTE we truncated the current length-24 sequences for the sake of simulation and obtaining good enough results. Better sequences may be obtained by numerical search if justified.

The generated symbol is then truncated/windowed in time domain in order to span only 10 repetitions in time domain, i.e., respectively {23, 19}us. CP is included in the repetitions.

Proposals:

- For 10MHz system BW, define the following LTE-ITS detection sequences:

  - Length-18 base sequence with comb mapping every 29th subcarrier;

  - Length-15 base sequence with comb mapping every 35th subcarrier;

- The sequences are windowed in time domain such that only 10 time repetitions are used.

  - The base sequence could be obtained from a truncated length-24 LTE base sequence

- The detection sequence is selected by higher layers as a function of the ITS service (e.g., safety or non-safety)

## Resource mapping of detection sequence in LTE-ITS sidelink

One design option is to define a configurable “LTE-ITS detection sequence resource”. It also seems wasteful to use a full subframe to transmit a sequence. Mapping from LTE-ITS detection sequence ID (i.e., ITS service type) to specific symbols in the subframe may be considered for reducing inter-sequence interference.

Another preferred option is to transmit the LTE-ITS detection sequence after each transmission, within the gap. Since the sequence is only 23us it can be accommodated in the first part of the last OFDM symbol, spanning the central 7.4 MHz as discussed in Section 2.1. The second part of the last symbol includes the GP.

Figure 1: Proposed resource mapping for the LTE-ITS detection sequence. Note that the LTE-ITS detection sequence spans the central 7.4 MHz irrespective of the bandwidth spanned by the following data symbols.

Proposals:

- The LTE-ITS sidelink detection sequence is transmitted within the first part of the last OFDM symbol

  - FFS whether transmission occurs at every transmitted sidelink subframe or periodically, e.g., every 100ms

  - FFS whether an additional time gap between the data subframe and the detection sequence is useful.

- The second part of the last OFDM symbol for each sidelink subframe is used for GP.

- Transmission of the LTE-ITS sidelink detection sequence is configurable, subject to local coexistence requirements.

## Detection of LTE-ITS sidelink detection sequence at IEEE 802.11p receiver

To detect the LTE-ITS detection sequence, an IEEE 802.11p receiver may use the delay-and-correlate detector, whose structure is shown in Figure 4, where the delay time is L=23 samples for LTE-ITS detection sequence 1 and L=19 for LTE-ITS detection sequence 2 instead of L=16 as for IEEE 802.11p preamble. The detection procedure and decision condition is described in the Appendix. At the IEEE 802.11p receiver, the sampling rate for detecting LTE-ITS detection sequence is 10MHz, i.e. identical to the sampling rate for IEEE 802.11p packet detection.

Observation:

- The proposed LTE-ITS detection sequence can be efficiently detected with the “delay and correlate” method typically assumed for IEEE 802.11p with very minor software changes.

# Simulation results

It is foreseen that for coexistence between IEEE 802.11p and LTE-ITS sidelink an IEEE 802.11p receiver shall possess two delay-and-correlate detectors: one detector, named as detector A, with delay L=16 for IEEE 802.11p preamble detection, and the other, named as detector B, with L=23 for LTE-ITS safety sequence detection. Both detectors work in parallel and use identical sampling rate, i.e. 10MHz. Details of the Delay and Correlate algorithm are provided in Appendix for information.

Other access technologies (e.g., LAA or NR) may need to detect non-safety ITS in addition to safety ones, depending on local regulations. In such case a detector C with L=19 may also be needed in addition to detectors A and B.

In order to successfully detect a preamble the output of the “delay and correlate” algorithm need to exhibit a plateau for a certain number of consecutive samples (the exact details are of course up to implementation). In Figure 2 we show the output of the delay and correlate algorithm in the following cases:

- The output of detector A (802.11p) upon reception of the IEEE 802.11p preamble

- The output of detector B (LTE-ITS safety) upon reception of the LTE-ITS safety detection sequence

- The output of detector C (LTE-ITS non-safety) upon reception of the LTE-ITS non-safety detection sequence

- The output of detector B (LTE-ITS safety) upon reception of the IEEE 802.11p preamble

- The output of detector C (LTE-ITS non-safety) upon reception of the IEEE 802.11p preamble

- The output of detector A (802.11p) upon reception of the LTE-ITS safety detection sequence

- The output of detector A (802.11p) upon reception of the LTE-ITS non-safety detection sequence

- The output of detector C (LTE-ITS non-safety) upon reception of the LTE-ITS safety detection sequence

- The output of detector B (LTE-ITS safety) upon reception of the LTE-ITS non-safety detection sequence

We observe that 1), 2) and 3) provide the desired plateau output (the LTE-ITS plateau is longer because of the different signal length).

The cross-correlation outputs by 4)-9) are very low as desired, which implies low false-detection probabilities.

Figure 2: Simulation results of delay-and-correlate detectors for 802.11p and LTE-ITS with different input signals

Observations:

- The proposed LTE-ITS detection sequences have similar detection performance as the IEEE 802.11p preamble

- The false detection probability of IEEE 802.11p preamble by an LTE-ITS detector (and vice versa) is low.

# Conclusion

In this contribution we discuss the design of detection sequence in LTE-ITS for ensuring coexistence between LTE-ITS and IEEE 802.11p. We propose the following:

Proposals:

- For 10MHz system BW, define the following LTE-ITS detection sequences:

  - Length-18 base sequence with comb mapping every 29th subcarrier;

  - Length-15 base sequence with comb mapping every 35th subcarrier;

- The sequences are windowed in time domain such that only 10 time repetitions are used.

  - The base sequence could be obtained from a truncated length-24 LTE base sequence

- The detection sequence is selected by higher layers as a function of the ITS service (e.g., safety or non-safety)

- The LTE-ITS sidelink detection sequence is transmitted within the first part of the last OFDM symbol

  - FFS whether transmission occurs at every transmitted sidelink subframe or periodically, e.g., every 100ms

  - FFS whether an additional time gap between the data subframe and the detection sequence is useful.

- The second part of the last OFDM symbol for each sidelink subframe is used for GP.

- Transmission of the LTE-ITS sidelink detection sequence is configurable, subject to local coexistence requirements.

Observations:

- The proposed LTE-ITS detection sequence can be efficiently detected with the “delay and correlate” method typically assumed for IEEE 802.11p with very minor software changes.

- The proposed LTE-ITS detection sequences have similar detection performance as the IEEE 802.11p preamble

- The false detection probability of IEEE 802.11p preamble by an LTE-ITS detector (and vice versa) is low.

# Reference

- RP-161894 (revision of RP-161874), “Revised WI proposal: LTE-based V2X Services”, LG Electronics, Huawei, CATT

- IEEE 802.11™-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

- G. Maier, A. Paier, and C. F. Mecklenbräuker, “Packet detection and frequency synchronization with antenna diversity for IEEE 802.11p based on real-world measurements,” in 2011 International ITG Workshop on Smart Antennas (WSA), 2011, pp. 1–7.

- T. M. Schmidl and D. C. Cox, “Robust frequency and timing synchronization for OFDM,” IEEE Transactions on Communications, vol. 45, no. 12, pp. 1613–1621, Dec. 1997.

- 3GPP TS 36.211 V13.1.0 (2016-03), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation (Release 13)

- R1-165277,	L1 Format for V2V Transmissions using Sidelink, Ericsson

# Appendix: Preamble Design and Packet Detection in IEEE 802.11p

## Design of preamble in IEEE 802.11p

Figure 3 shows the preamble design in IEEE 802.11 OFDM PHY. [3] The Physical Layer Convergence Procedure (PLCP) preamble consists of a short and a long training sequence. The first sequence is a repetition of ten identical short training symbols each of a length of 16 samples. The long training sequence consists of two identical long training symbols, each of a length of 64 samples, preceded by a 32-sample long cyclic prefix. The short training symbols are used for signal detection, AGC, diversity selection, coarse frequency offset, and timing synchronization. The sampling rate of generating the preamble in IEEE 802.11p is 10MHz. Details about the short and long training sequences in IEEE 802.11p can be found in Clause 18.3.3 of [3].

Figure 3: Preamble structure in IEEE 802.11p OFDM PHY [3] 
(with timing information of 10 MHz channel spacing)

Observation:

- IEEE 802.11p uses repetition of ten identical short training symbols in the preamble for packet detection.

## Detection of packet in IEEE 802.11p

A common way of IEEE 802.11p packet detection is to use the delay-and-correlate algorithm, which is shown in Figure 4. [3][4]

Figure 4: Delay-and-correlate algorithm for IEEE 802.11p packet detection

The sliding window P in Figure 4 calculates a correlation between the input signal r(n) and a version of the input signal delayed in time by L=16 samples, i.e. the length of one short training symbol in IEEE 802.11p.

Pn=m=0L-1rn+m*⋅rn+m+L													(1)

The sliding window R in Figure 4 calculates the received energy in the correlation interval:

Rn=m=0L-1rn+m+L2⋅ 												(2)

M(n) is the decision statistics:

Mn=P(n)2R(n)2 																(3)

The decision metric can also be modified by the simplification [4]:

MHWn= 1,if P(n)2≥βTh⋅R(n)20,otherwise											(4)

Where, βTh is the threshold for decision that usually takes the value 0.5. To detect a valid IEEE 802.11p preamble MHWn has to be 1 for at least 16 samples and stable (consecutive 1’s) within 5 samples. [4]

The sampling rate for packet detection in IEEE 802.11p is 10MHz.

Observation:

- IEEE 802.11p receiver uses delay-and-correlate algorithm for packet detection.