3GPP TSG RAN WG1 Meeting #87	R1-1613046

Reno, USA 14th - 18th November 2016

Source: 	Intel Corporation

Title:	On multiplexing between PSS and SSS

Agenda item:	7.1.2.1

Document for:	Discussion and Decision

# 1 Introduction

In RAN1#86bis, different multiplexing options for the synchronization signals and physical channels were discussed, including TDM and FDM approaches. During the discussion, the following agreements were reached:

Agreements: - PSS, SSS and/or PBCH can be transmitted within a ‘SS block’ - FFS: details how to compose PSS, SSS and/or PBCH - Multiplexing other signals are not precluded within a ‘SS block’ - One or multiple ‘SS block(s)’ compose an ‘SS burst’ - FFS: Number of ‘SS block(s)’ (defined as duration of ‘SS burst’) - FFS: whether or not ‘SS block(s)’ are consecutive - FFS: whether or not ‘SS block(s)’ within a ‘SS burst’ are the same - One or multiple ‘SS burst(s)’ compose a ‘SS burst set’ - FFS: Periodicity and the number of ‘SS burst’ within a SS burst set - Number of SS bursts within a SS burst set is finite. - FFS: Transmission instances of ‘SS burst set’ - E.g., periodic/aperiodic transmission of SS burst sets.

In this contribution, we provide our views regarding TDM/FDM multiplexing between PSS and SSS signals within SS block for NR.

# Discussion

Multiplexing of synchronization signals

Two approaches for construction of (SS) block can be considered with FDM and TDM multiplexing of PSS and SSS. Considering the same amount of time and frequency resources the two possible options for PSS and SSS multiplexing are shown in Figure 1. For both options, we assume that SS block occupies the same transmission frequency bandwidth and time duration.

(a) Frequency domain structure of SS block	(b) Time domain structure of SS block

Figure 1. Multiplexing of PSS and SSS within SS block

For SS block structure with FDM multiplexing between PSS and SSS, the SS block consists of one OFDM symbol with PSS and SSS transmitted on non-overlapping REs. In addition, the support of guard band (GB) subcarriers to separate PSS and SSS in the frequency domain should be considered for this multiplexing option to minimize the ICI between PSS and SSS transmissions. In this case the GB can be considered as additional overhead that should reduce the SS processing gain of SS block with FDM multiplexing.

For SS block with TDM multiplexing between PSS and SSS, the SS block has two OFDM symbols. However, to accommodate the same amount of time resources as for FDM multiplexing, the duration of the OFDM symbols of PSS and SSS should be reduced by considering larger subcarrier spacing. The CP duration for TDM multiplexing should be also scaled that makes TDM based approach less robust to possible inter-symbol interference caused by channel propagation delays. In particular the effect of the propagation delays is expected to be more pronounced for PSS transmission due to the use of the limited amount of the sequences causing larger delays spread of the effective channels. Therefore, the use of the reduced CP duration for PSS transmission should be investigated for TDM option.

Dedicated OFDM symbols for PSS and SSS signal in TDM structure of SS block offers low PAPR sequence designs. The low PAPR properties of properly design PSS and SSS can be exploited for the power boosting. The power boosting, however, would require no transmission of the other signals in the corresponding OFDM symbols such as PBCH or PDSCH. When NR SS are the only signals occupying an OFDM symbol, the maximum power boost available (for either TDM or FDM structure) will be determined by the required minimum EVM needed for sufficient receiver detection performance. In such scenarios, transmission sequences with lower PAPR would potentially enable higher transmit power while obtaining the same transmit EVM. When the required EVM for PSS and SSS are different, TDM structure enables different power scaling between PSS and SSS OFDM symbols, while FDM structure would force the PSS and SSS to retain the same transmit EVM properties.

A smaller effective bandwidth of PSS transmission in SS block structure with FDM multiplexing implies processing of PSS with lower sampling rate. As the result, the amount of processed received data samples for FDM based approach can be reduced comparing to TDM based approach to cell search complexity from processing and memory requirements. On the other hand, considering the reduced transmission bandwidth of PSS and SSS, it is expected that the timing estimation accuracy from SS block with FDM multiplexing is less accurate comparing to SS block with TDM multiplexing due to finer sampling rate. In this case the reduced complexity of cell search would come at the cost of the worse performance.

A smaller effective duration of PSS transmission in SS block structure with TDM structure implies larger subcarrier spacing. As the result the impact of the carrier frequency offset (CFO) on SS performance should be smaller for TDM comparing to FDM. The frequency offset estimation accuracy of SS block with FDM structure should be more accurate due to finer subcarrier spacing of SS block with FDM structure. However, for neighbor cell search the CFO issue may not be an issue and the possible CFO benefits should be further investigated.

Summarizing discussion above it seems SS blocks with FDM and TDM multiplexing option offer similar performance in power efficiency and has opposite advantages and disadvantages w.r.t. to timing and frequency offset estimation accuracies. Considering some of the benefits above for TDM based approach and given that TDM structure is a well understood structure that is already supported in LTE, we have a preference on TDM based approach for PSS and SSS multiplexing within SS block.

Proposal:

- Consider TDM based approach for PSS and SSS multiplexing within SS block

Multiplexing of SS and PBCH

For multiplexing of SS (e.g., PSS and SSS) and PBCH, the following options, which depicted in Figure 2, may be considered:

Opt.1: FDM between every SS and PBCH;

Opt.2: TDM between every SS and PBCH;

Opt.3: PSS is transmitted on different OFDM symbol(s) comparing to SSS and PBCH. At the same time, the subcarriers allocated for PBCH are interlaced with subcarriers used for SSS transmission;

Opt.4: PSS and SSS are multiplexed in the time domain, while FDM is used for SS and PBCH multiplexing;

Opt.5: PSS and SSS are transmitted on the same OFDM symbol(s), while PBCH on other OFDM symbol(s).

(a) Opt.1	(b) Opt.2	(c) Opt.3	(d) Opt.4	(e) Opt.5

Figure 2. Multiplexing options for SS and PBCH

The following aspects can be considered for comparison of different multiplexing options:

- Impact on SS detection performance due to multiplexing with PBCH

  - FDM with PBCH would shrink transmission bandwidth of SS affecting the timing estimation accuracy

  - TDM with PBCH would increase subcarrier spacing of SS affecting the frequency offset estimation accuracy and make the CP duration shorter

- Possible use of SSS for demodulation of PBCH

- Possible use of power boosting on SS

The comparison results using the above aspects are summarized in Table 1.

Table 1. Comparison of different multiplexing options

| Aspect | Opt.1 | Opt.2 | Opt.3 | Opt.4 | Opt.5 |
| --- | --- | --- | --- | --- | --- |
| Power boosting on SS | Limited | Yes | Yes for PSS Limited for SSS | Limited | Yes |
| Timing estimation accuracy | Reduced |  |  | Reduced | Reduced |
| Frequency estimation accuracy |  | Reduced | Reduced |  |  |
| Aid channel estimation for demodulation of PBCH | No | Yes | Yes | No | No |

Taking into account the number of options and their different advantages and drawbacks, the following proposal can be make:

Proposal:

- To decide on multiplexing of SS and PBCH, further study is needed.

# Evaluation

The evaluation results of NR SS detection performance with TDM and FDM are presented in Figure 3 in terms of the timing Detection Failure Rate vs. SNR curves for NR PSS (a) and NR PSS+SSS (b). The residual timing and frequency synchronization error statistics is shown in Figures 4(a) and 4(b). To obtain the results, the non-coherent accumulation of multiple, i.e., 8, consecutive SS burst sets was used. Each burst set contains a single SS burst consisted of 16 identical SS blocks. Other evaluation assumptions are listed in the Appendix.

(a) NR PSS detection error rate	(b) NR (PSS+SSS) detection error rate

Figure 3. NR SS detection performance

(a) Timing	(b) Frequency

Figure 4. Residual timing and frequency synchronization errors for SNR = -9 dB

From Figure 3, it can be seen that both TDM and FDM options shows almost the same performance and TDM slightly outperforms FDM in terms of joint NR (PSS+SSS) detection.

# Summary

Proposal:

- Consider TDM based approach for PSS and SSS multiplexing within SS block

# References

- R1-1609111 Samsung

- R1-1610159 Qualcomm

- Chairman’s notes for RAN1#86bis, 3GPP TSG RAN WG1 Meeting #86bis, Lisbon, Portugal 10th - 14th October 2016.

# Appendix: Evaluation assumptions

| Parameter | Assumption |
| --- | --- |
| Channel model | 5GCM CDL-C, delay spread = 100ns |
| Carrier frequency | 30 GHz |
| FFT size | 1024 |
| Subcarrier spacing | 120 kHz (FDM), 240 kHz (TDM) |
| SS Bandwidth | 15.360 MHz (64x240kHz = 2x64x120kHz) |
| Cyclic prefix | 256 samples |
| PSS | Zadoff-Chu based (root = 1, length = 63) + DC removal |
| SSS | Based on two binary lengh-31 sequences from LTE SSS design |
| Frequency offset | 2, 5, 10, 20 ppm |
| Search window | 3840 OFDM symbols (one SS burst set) |
| Target false alarm probability | 0.001 |
| eNB antenna | (1,16,1), ULA (Ntx = 16), Omni-directional antenna element |
| UE antenna | Omni-directional X-Pol |