3GPP TSG-RAN WG1 #88	R1-1701869

# Athens, Greece, 13th- 17th February 2017

Agenda Item:	8.1.3.4.1

Source:	Ericsson

Title:	On eMBB/URLLC multiplexing for downlink

Document for:	Discussion, Decision

# Introduction

An email discussion on multiplexing URLLC and eMBB data transmissions in NR has been concluded [2]. According to the previous agreements, for DL, dynamic resource sharing between URLLC and eMBB is supported by transmitting URLLC scheduled traffic. Thus,

- URLLC transmission may occur in resources scheduled for ongoing eMBB traffic.

- There should be a clear indication to the eMBB receiver that its data is punctured.

# Discussion

When a gNB have eMBB and URLLC traffic, the gNB can perform either scheduling or puncturing to transmit the traffic to the URLLC receiver. As discussed in paper [1], the gNB can choose to schedule different traffic types in different parts of the spectrum if it wants to, as part of its normal set of scheduling tools. Also, not all URLLC services may need ultra-low latency, in which case normal scheduling and slot-based transmissions is sufficient and no mechanisms other than those already envisioned for NR are needed. When URLLC services need ultra-low latency and gNB already is transmitting a large volume of eMBB traffic, then puncturing eMBB traffic might be a good choice.

## Impact on eMBB BLER

Within the scope of the LTE Rel-15 WI Short TTI & Processing Time [2] a study [3] was performed to investigate the impact of a 1ms transmission being punctured by short TTI transmissions of 2 OFDM symbols. In [1], we showed how much eMBB BLER will be affected where there is no indication to eMBB receiver. In the previous meeting, it has been agreed to have an indication to the eMBB receiver where a part of its data is punctured. Here we show the impact on downlink eMBB BLER when there is an indication to the eMBB receiver and compare the results with the cases when there is no indication. We also show the impact of HARQ retransmission in presence of puncturing. The simulation details are given in Appendix 1.

Figure 1 shows the impact of puncturing of a short TTI transmission into 1ms transmission for QPSK modulation and code rate 1/2. Here 2 OFDM symbols (using the same bandwidth as the eMBB transmission) are assigned to URLLC traffic (equivalent to 16.6% of resources being punctured). The impact of HARQ retransmissions is also shown for the cases when chase combining or incremental redundancy combining is used. The figure first of all shows that the performance of the eMBB receiver is degraded by puncturing, as mentioned previously in [1].

HARQ retransmission can considerably improve the performance of the eMBB receiver. In the simulated cases, HARQ with incremental redundancy combining shows better results than chase combining HARQ. We observe good performance when puncturing information is used while combining first and second transmission. Indication with HARQ retransmissions has about 1 dB loss compared to HARQ retransmission without puncturing

Figure 1 BLER for eMBB transmission with rank 1 QPSK r1/2 when being punctured by URLLC. The punctured data is two OFDM symbols, which is equivalent to 16.6% puncturing.

Figure 2 shows the impact of puncturing of a short TTI transmission into 1ms transmission for QPSK modulation and code rate 2/3. The figure also shows that indication improves BLER of eMBB receiver in higher code rates as well. By comparing the results in Figure 1 and Figure 2, we observe that the loss is higher in higher code rates.

Figure 2  BLER for eMBB transmission for rank 1 QPSK r2/3 when being punctured by URLLC. The punctured data is two OFDM symbols, which is equivalent to 16.6% puncturing.

Table 1 summarizes the gain in SNR for the different transmission methods compared to the baseline (no puncturing, no HARQ) for BLER=0.1.

Table 1 Gain in SNR for different transmission methods compared to the baseline transmission

| Transmission method | QPSK1/2, Rank1 | QPSK2/3, Rank1 | QPSK1/2, Rank2 | QPSK2/3, Rank2 |
| --- | --- | --- | --- | --- |
| No puncturing, no HARQ (Baseline) | 0 dB | 0 dB | 0 dB | 0 dB |
| No puncturing, with HARQ, IncrRed | 3.62 dB | 4.68 dB | 4.94 dB | 6.37 dB |
| No puncturing, with HARQ, Chase | 2.97 dB | 3.25 dB | 3.06 dB | 3.18 dB |
| Puncturing, no HARQ | N/A | N/A | N/A | N/A |
| Puncturing only first transmission, with HARQ, IncrRed | 1.13 dB | -0.96 dB | 1.52 dB | -7.97 dB |
| Puncturing only first transmission, with HARQ, Chase | 0.21 dB | -6.38 dB | -1.10 dB | N/A |
| Puncturing, no HARQ, clear soft values | -1.87 dB | -3.20 dB | -2.73 dB | -5.26 dB |
| Puncturing only first transmission, with HARQ, clear soft values, IncrRed | 3.04 dB | 3.76 dB | 4.21 dB | 5.16 dB |
| Puncturing only first transmission, with HARQ, clear soft values, Chase | 2.52 dB | 2.60 dB | 2.54 dB | 2.63 dB |

An important observation is that indication of puncturing, even without retransmission, can considerably improve the performance of the eMBB receiver. As shown in the figures (purple-color curves in Figure 1 and Figure 2), even indication without retransmission is sometimes useful. The loss is about 2 dB for the r1/2 code rate in Figure 1, while higher loss for the higher code r2/3 code rate in Figure 2. Generally, it works better for low code rates. This however comes with the drawback that the UE may need to decode two PDSCH at the sametime, i.e. the one assigned in the previous slot and the one that is assigned in the current slot at the same time. Correspondingly, the simplest approach and the one that gives highest performance is if the punctured is indicated with the retransmissions of the same HARQ process.

Indication that an HARQ process on PDSCH was punctured is sent in the DL assignment for the retransmission of the same HARQ process

From the simulation results in Figure 1 and 2, we see that if there is no puncturing information (zero amount of DCI information) and there is no HARQ, the decoding breaks down (solid-red lines in Figure 1 and 2). If there is HARQ retransmission and information sent to the eMBB receiver informing whether any part of the data have been punctured or not, then the performance improves to being the same as a single, un-punctured, transmission (solid-blue) lines. The UE would then discard all information from the first transmission.

The simulations shown here use an allocation where the whole TB fits into one CB. The puncturing mini-slot transmission is here covering two full symbols, and then by using puncturing information soft values from the full two symbols are cleared. If DCI information is several bits indicating exactly the location of punctured data, then the receiver performance can be improved considerably for puncturing with HARQ. This is shown in the figures by yellow lines for puncturing with HARQ.

Single-bit DCI indication that some part of a previous transmission was punctured can slightly improve the performance of eMBB receiver.

More detailed DCI indication, giving the exact location of punctured data, can improve the performance considerably. There is a trade-off between the DCI overhead and the receiver BLER.

What to transmit as puncturing information could be further studied. One approach is to indicate which symbols that have been punctured. For larger eMBB allocations, where the TB may be split up into two or more CB, one method is to map the puncturing indication to each CB, which can be used with a multi-bit HARQ, with one bit per CB. An alternative approach to minimize the signaling in the DCI is to only indicated to the UE whether or not it was punctured. The UE tries to detect based on the retransmissions of the same TB what soft values in the previous transmission was punctured.

Study what puncturing information the UE is provided

# Conclusion

In section 2 we made the following observations:

Observation 1	Single-bit DCI indication that some part of a previous transmission was punctured can slightly improve the performance of eMBB receiver.

Observation 2	More detailed DCI indication, giving the exact location of punctured data, can improve the performance considerably. There is a trade-off between the DCI overhead and the receiver BLER.

Based on the discussion in section 2 we propose the following:

Proposal 1	Indication that an HARQ process on PDSCH was punctured is sent in the DL assignment for the retransmission of the same HARQ process

Proposal 2	Study what puncturing information the UE is provided

# References

R1-1700688, Discussion on URLLC puncturing of data, Ericsson, RAN#87, January 2017.

RP-162014, WID Short TTI & Processing Time, Ericsson, RAN#74, December 2016.

R1-1611515, Puncturing sTTI in legacy TTI, Ericsson, RAN1#87, November 2016.

Email discussion on URLLC and eMBB multiplexing.

# Appendix: Simulation assumptions

Table 2 specifies the settings used in the link level evaluations:

Table 2

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 2.0 GHz |
| Waveform | OFDM |
| Channel coding | LTE Turbo code |
| Subcarrier spacing | 15 kHz |
| Frequency allocation | 21 PRB (whole TB fits into one CB) |
| Modulation of eMBB | QPSK |
| Code rate of eMBB | 1/2, 2/3 |
| Modulation of URLLC | QPSK |
| BS antenna configuration | 2Tx |
| MIMO mode | eMBB: Spatial multiplexing, fix PMI, fixed rank 1 or 2 |
| Number of antenna ports | 2 |
| UE antenna configuration | 2 Rx |
| Channel estimation | Realistic channel estimation |
| Propagation channel & UE velocity | EVA, 3km/h |
| Number of HARQ retransmissions | 1 |
| Combining method for HARQ retransmission | Chase combining & incremental-redundancy combining |
| TTI length | eMBB : 14 OFDM symbols, URLLC : 2 OFDM symbols |