TSG-RAN WG1 Meeting #88	R1-1702111

Athens, Greece, February 13th – 17th, 2017

Source:	CATT

Title:	Comparison of LDPC and polar codes for URLLC

Agenda Item:	8.1.4.3

Document for:	Discussion/Decision

Introduction

In RAN1 #84bis meeting, the following simulation assumptions have been agreed for URLLC scenario [1].

Agreement:

- Simulation Assumptions for URLLC scenario

  - Evaluate the block error rate (BLER) performance versus SNR

| Channel* | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 16 QAM |  |  |  |
| Coding Scheme | Convolutional codes | LDPC | Polar | Turbo |
| Code rate | 1/12, 1/6, 1/3 |  |  |  |
| Decoding algorithm** | List-X Viterbi | min-sum | List-Y | Max-log-MAP |
| Info. block length*** (bits w/o CRC) | 20, 40, 200, 600, 1000 |  |  |  |

* Fading channels will be simulated in the next stage

** These algorithms are starting points for further study. Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated)

*** At least these information block length and code rate shall be evaluated. Other information block lengths and code rates are not precluded. Similar info and encoded block lengths should be used for the evaluation. Total coded bits = information Block length/code rate. Note: these information block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis.

In this contribution, the evaluation results of LDPC and polar codes for URLLC are provided.

Discussion

  - Evaluation description

- Channel coding schemes

- LDPC: LDPC parity check matrix provided in [2] with lifting, puncturing and repeating operation is used for URLLC evaluation.

- Polar codes: The PC-Polar code provided in [3] is used which contains a self-parity-check function applied on the PC (parity-check)-frozen-bit set to avoid using CRC for path selection. With this function, SCL decoding algorithm works without CRC-aided. Then, CRC attachment is used for error detection only.

  - Evaluation results

We compare the performance of LDPC and Polar codes for URLLC with various information block sizes (K=200, 600 and 1000), coding rates (R=1/12, 1/6 and 1/3) without CRC detection. Layered normalized min-sum decoding algorithm with maximum 25 iterations and scaling factor 0.8125 are adopted for LDPC evaluation. Considering BLER target of 1st Tx is about 10-1 or 10-2, maximum iterations are needed most of time for LDPC decoder. SCL decoding with list size L=8 is used for PC-polar code. The simulations are conducted over the BI-AWGN channel with QPSK modulations. The simulation results are shown in Figure 1-12

Case 1: BLER performance

Figure 1: BLER Performance of LDPC and polar codes with K=200 and QPSK modulation

Figure 2: BLER Performance of LDPC and polar codes with K=600 and QPSK modulation

Figure 3: BLER Performance of LDPC and polar codes with K=1000 and QPSK modulation

As shown in Figures 1-3, Polar codes perform slightly better than LDPC at R=1/3. The performance gain of the polar codes over the LDPC codes at R=1/3 decreases as the block size increases.  For R=1/6 and R=1/12, Polar codes outperform LDPC significantly with the coding gain of about 0.9dB at BLER=0.01.

Observation 1: Polar codes have better BLER performance than LDPC for URLLC.

Case 2: HARQ performance

Figure 4: K=200, 1st Tx code rate R=1/12                        Figure 5: K=200, 1st Tx code rate R=1/6

Figure 6: K=200, 1st Tx code rate R=1/3                     Figure 7: K=600, 1st Tx code rate R=1/12

Figure 8: K=600, 1st Tx code rate R=1/6                  Figure 9: K=600, 1st Tx code rate R=1/3

Figure 10: K=1000, 1st Tx code rate R=1/12                    Figure 11: K=1000, 1st Tx code rate R=1/6

Figure 12: K=1000, 1st Tx code rate R=1/3

We evaluate the HARQ schemes of LDPC codes and Polar codes.  The details of IR-HARQ scheme of LDPC are given in [4] and HARQ scheme of polar code is based on the proposed incremental redundancy scheme in [5].

As shown in Figures 4-12, the required Es/N0 of the Polar code is better than that of the LDPC code in all four transmissions at the BLER of 10-2.  Although LDPC code might have better HARQ performance with high complexity decoder to level up with that of the Polar code, we can see that the HARQ scheme of the Polar codes performs well with robustness to different code rate. We can also see that polar codes have slightly better performance of 1st Tx than LDPC codes at R=1/3, while the retransmission performance gain of polar codes over LDPC becomes larger with the decrease of effective coding rates. The reason is that the performance of low rate raptor-like structure LDPC codes with normalized min-sum decoding is degraded. A low complexity LDPC decoding algorithm with insensitive to fading channel is desired but challenged in meeting the performance requirements in URLLC and eMBB deployment scenario.

Observation 2: Polar codes with IR-HARQ outperform LDPC with IR-HARQ for URLLC.

Observation 3: A low complexity LDPC decoding algorithm with insensitive to fading channel is desired but challenged in meeting the performance requirements in URLLC and eMBB deployment scenario.

Conclusion

The above discussion is summarized with following observations and proposals:

Observation 1: Polar codes have better BLER performance than LDPC for URLLC.

Observation 2: Polar codes with HARQ outperform LDPC with HARQ for URLLC.

Observation 3: A low complexity LDPC decoding algorithm with insensitive to fading channel is desired but challenged in meeting the performance requirements in URLLC and eMBB deployment scenario.

References

[1].	3GPP RAN1#84bis chairman notes.

[2].	R1-1610137, “LDPC rate compatible design overview”, Qualcomm Incorporated.

[3].	R1-1608862, “Polar Code Construction for NR”, Huawei, HiSilicon.

[4].	R1-1700240, “IR-HARQ scheme for LDPC codes”, CATT

[5]          R1-1611255, “HARQ scheme for polar codes”, Huawei, HiSilicon.