3GPP TSG RAN1 WG Meeting #88			R1-1702247

Athens, Greece, Feb 13-17, 2017

Source:	Intel Corporation

Title:	Evaluation results on dynamic TDD in indoor scenario

Agenda item:	8.1.6.1

Document for:	Discussion and Decision

# Introduction

In the RAN1#87 meeting, it was agreed that [1]

- NR should support dynamically assigned DL and UL transmission directions at least for data on a per-slot basis at least in a TDM manner

- At least following schemes are identified to be further studied aiming to mitigate cross-link interference with and without the assumption on inter-cell coordination:

  - Advanced receiver for interference cancellation/suppression

  - RS design (e.g. symmetric RS) and timing alignment between DL and UL

  - Sensing/measurement scheme (e.g. LBT-like, OTA measurement if any, etc.)

  - Power control and coordinated schemes (e.g. coordinated beamforming/scheduling, OTA signalling if any, etc.)

  - Link adaptation

And in the RAN1 Jan. Ad-Hoc meeting, it was agreed that

- Companies are encouraged to

  - further update the evaluation results

  - perform evaluations under various RU percentage values

  - take into account additional overhead for the operation

When designing the interference management scheme, one of the key aspects is the timing and the resource of interference measurement in DL and UL, and its relationship with the timing and resource of resource scheduling and data transmission.

In this contribution, we evaluate the performance of dynamic TDD in indoor scenario with focus on the impact of the measurement accuracy on performance.

# Numerical evaluation

## Simulation assumptions

To understand the impact of DL and UL interference measurement timing and resource on system performance, we compare the following two schemes in the simulation. Dynamic TDD is assumed for both schemes.

Scheme 1:  Time-aligned DL and UL interference measurement in the scheduled resource

In this scheme, interference measurement and measurement reporting are in between resource scheduling and the corresponding data transmission, i.e., interference measurement is conducted in the bandwidth as assigned for the corresponding data and follows the same transmission direction (DL or UL) as the corresponding data. Figure 1 illustrates the subframe structure use in the simulation. In each slot, the DL interference measurement RS (IM-RS) and the UL IM-RS are transmitted after DL physical control channel (DLCC) which signals the resource allocation information. The measurement reporting is transmitted after DL/UL IM-RS in the DLCC/ULCC. The data transmission in the slot is based on the CQI from the measurement reporting. Detailed subframe design and its variations can be found in our previous contributions [2][3].

Figure 1 Subframe structure of interference measurement scheme 1

Scheme 1a:

Figure 2 Subframe structure of interference measurement scheme 1a

In this scheme, like in scheme 1, the DL and UL interference measurement are time aligned and the measurement RS is transmitted within the resource scheduled for the corresponding data. The difference from scheme 1 is that scheme 1a implements cross-slot scheduling, measurement, measurement reporting and data transmission is performed, which would require less number of gaps in a subframe and therefore less overhead as illustrated in Figure 2. Scheme 1a would have higher system throughput than scheme 1 due to less overhead. The constraint on scheme 1a is that the cells need to both apply cross-slot measurement, which is less flexible than scheme 1. More detailed discussion on scheme 1a can be found in our previous contribution [3].

Scheme 2:  Time-separate DL and UL interference measurement without resource scheduling information

In this scheme, each slot has one symbol on DL IM-RS and one symbol on UL SRS. The DL IM-RS is transmitted after DLCC at the front of a slot and the UL SRS is transmitted before ULCC at the end of a slot. The DL IM-RS and the UL SRS are periodically transmitted regardless of the resource scheduling of a particular data transmission. Note that the subframe structure is not sufficient for cross-link IM. In order for a gNB to measure interference from other gNBs, a gap period need to be inserted in between DL control and DL IM-RS. Similar, in order for UE to measure interference from other UEs in UL, a gap period need to be inserted between UL data and SRS.

Figure 3 Subframe structure of interference measurement scheme 2

Indoor, dense urban and urban macro scenarios as defined in [4] are considered in the simulation. The carrier frequency is assume as 4 GHz. Inter-cell coordination is not assumed throughout the simulation. Detailed simulation assumptions can be found in the Appendix.

## Simulation results and discussion

In the simulation, the three schemes are compared in indoor hotspot scenario [4] with FTP-3 traffic model. In Table 1, preliminary simulation results on per-user packet throughput (UPT) of the three interference measurement and reporting schemes are demonstrated. SU-MIMO is applied for throughout the simulation. The FTP packet size of 0.1MB with DL:UL traffic ratio of 2:1 and 1:1 are tested. For scheme 1, the number of guard symbols per slot is 4. For scheme 1a and scheme 2, the number of guard symbols per slot is 2. No inter-cell cooperation is assumed throughout the simulation. More detailed system level simulation assumptions can be found in Appendix.

Table 1 UPT comparison among three IM schemes

* UPT results in scheme 1a is a prorated version of scheme 1, assuming 2 symbols less overhead.

** Since the simulation assumes unlimited amount of ARQ attempts, unfinished packet are only due to their late arrival time. The transmission of these late-arrival packets did not finish at the end of the simulation.

For all traffic load combinations, it can be observed that scheme 1 provides notable UPT gain over scheme 2. Despite higher overhead, scheme 1 outperforms scheme 2 due to accurate SINR measurement. By comparing the cases with lambda = 0.5 and the cases with lambda = 1, it can be observed that UPT gain of scheme 1 increases as the traffic load increases. Scheme 1 shows significant gain for low-rate UEs. The reason can be can be due to the impact of SINR accuracy is essential for low-rate UEs. Scheme 2 is observed to have higher resource utilization than scheme 1 in all the simulated cases.

With 2 symbols less overhead, scheme 1a can provide higher UPT gain than scheme 1. Please note that the UPT results for scheme 1a in Table 1 is only a prorated version of scheme 1. The UPT performance of scheme 1a is expected to be even better.

In Figure 4 and Figure 5, the CDF of per-user UPT of indoor hotspot are plotted for various traffic loads. It can be observed that scheme 1 achieves higher UPT for all users in all the simulated traffic loads.

Figure 4 UPT CDF for DL:UL=2:1 with DL λ=0.5 (left) and λ=1 (right)

Figure 5 UPT CDF for DL:UL=1:1 with DL λ=0.5 (left) and λ=1 (right)

Based on the preliminary simulation results, we make the following observation on the system level performance of indoor scenario and we propose time-aligned interference measurement in NR.

Observation 1: Measurement accuracy has notable impact on user packet throughput.

Observation 2: In dynamic TDD, higher user packet throughput can be achieved by link adaptation based on time-aligned DL and UL interference measurement over the same resource as scheduled for the corresponding data.

Proposal 1: NR should support time-aligned DL and UL interference measurement over the same resource as scheduled for the corresponding data.

## Potential specification impacts

To enable scheme 1 and scheme 1a, the specification shall define

- IM-RS that can capture both BS-UE interference and BS to BS and UE to UE interference

- Slot structure that enables time-aligned DL and UL IM-RS transmission

- Physical control and scheduling mechanism that enables resource allocation and transmission rate adaption in two steps so that measurement and measurement reporting can be implemented in between

# Conclusion

In this contribution, we presented simulation results on dynamic TDD at 4GHz in indoor hotspot scenario. Based on the discussion we draw the following observations and proposals:

Observation 1: Measurement accuracy has notable impact on user packet throughput.

Observation 2: In dynamic TDD, higher user packet throughput can be achieved by link adaptation based on time-aligned DL and UL interference measurement over the same resource as scheduled for the corresponding data.

Proposal 1: NR should support time-aligned DL and UL interference measurement over the same resource as scheduled for the corresponding data.

# References

- RAN 1 #87 Chairman’s Notes

- RAN 1 Jan Ad-Hoc Chairman’s Notes

- R1-1612006, “Interference management in NR TDD”, Intel Corporation, Reno, US, Nov. 2016

- R1-1612005, “Subframe structure for NR dynamic TDD”, Intel Corporation, Reno, US, Nov. 2016

- 3GPP TR36.802, V0.3.0, “Study on New Radio (NR) Access Technology”, Oct. 2016

- 3GPP TR36.843, V12.0.1, “Study on LTE Device to Device Proximity Service”, March. 2014

- 3GPP TR36.814, V9.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA)”, March. 2013

- 3GPP TR36.873, V12.2.0, “Study on 3D channel model for LTE”, June. 2015

# Appendix

Simulation assumptions are listed in the following table, which is based on [4].

| Parameters | Indoor hotspot |
| --- | --- |
| Layout | Single layer 3 BSs in 120m x 50m |
| Inter-BS distance | 40m |
| Min BS-UE distance | 0m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 20MHz |
| Channel model [5-7] | TRP-to-UE: ITU InH [5,6] TRP-to-TRP: ITU InH (h_UE=3m), ASA statistics updated to be the same as ASD UE-to-UE: ITU InH, ASD statistics updated to be the same as ASA. |
| BS Tx power | 24dBm |
| UE Tx power | 23dBm |
| BS antenna configurations [7] | (M, N, P, Mg, Ng)=(1, 1, 2, 1, 1) Omni antenna model Boresight direction is perpendicular to ceiling. |
| BS antenna height | 3m |
| BS receiver noise figure | 5dB |
| UE antenna configuration | (M, N, P, Mg, Ng)=(1, 1, 2, 1, 1) Omni antenna model |
| UE antenna height | 1.5m |
| UE receiver noise figure | 9dB |
| UE association | Based on RSRP measurement [7] |
| Traffic model | FTP-3 model with packet size 0.1MB |
| UE distribution | 10 users per TRP - 100% indoor |