3GPP TSG RAN WG1 Meeting #88                                       R1-1702338

Athens, Greece 13th - 17th February 2017

Agenda Item:	8.1.3.4.1

Source:	InterDigital Communications

Title:	On eMBB/URLLC multiplexing for TDD

Document for:	Discussion and Decision

# Introduction

In RAN1 NR Ad-Hoc Meeting [1], the following has been agreed for eMBB/URLLC multiplexing:

- For DL, support indication of time and/or frequency region of impacted eMBB resources to respective eMBB UE(s)

  - FFS: Details of the granularity for impacted region used in the indication

    - e.g., PRB (group)/symbol (group)/mini-slot (group)/CB (group)/TB/Slot

  - The indication is transmitted at one of the following (will be down selected later)

    - during current eMBB TTI

    - after current eMBB TTI

    - during and after current eMBB TTI

  - The indication is one of the following (will be down selected later)

    - explicit

    - implicit

    - explicit and implicit

In this contribution, we provide our views on dynamic resource sharing between URLLC and eMBB data in the DL and UL when the transmissions from URLLC overlap with resources of an ongoing/scheduled longer transmission for TDD.

# Dynamic TDD eMBB/URLLC multiplexing

TDD is an important operation mode for the eMBB use case. During the transmission of eMBB traffic, URLLC traffic may arrive. If the URLLC traffic is sporadic, an efficient way to achieve high spectral efficiency while minimizing the latency is preemptive transmission of URLLC traffic over eMBB transmission. The challenge is that the TDD imposes a strong constraint on the latency that is achievable if the TDD duplexing structure is strictly followed. This is shown in Figure 1 for URLLC downlink traffic. URLLC traffic arrives at the gNB for service. However, the current time slot (Slot 2) is designated as Uplink (UL). The next Downlink (DL) time slot is Slot 4. If the gNB starts transmitting URLLC traffic in Slot 4, the delay T1 will be more than 1 time slot, which may not satisfy the latency requirement for URLLC.

To reduce the latency, the gNB may preemptively switch the duplexing direction to accommodate the URLLC traffic at the expense of disrupting the scheduled eMBB transmission in a controlled manner. Specifically, the gNB may change the duplexing direction within Slot 2 from UL to DL, transmit the URLLC traffic in the DL and change back. This is shown in the lower part of Figure 1. The latency T2 is clearly much lower than T1.

Figure 1 The latency with conventional TDD T1 and preemptive uplink switching T2.

Proposal 1: Consider dynamically changing the duplexing direction within the slot for the TDD operation when URLLC traffic is multiplexed with eMBB traffic.

To optimize the system performance, we need to consider the impact of preemptively switching the duplex direction. If the gNB operates in the half-duplex mode, the preemptive switching may cause UL transmissions to fail. The gNB may send NACKs for the failed UL eMBB transmissions, and reschedule the UL eMBB transmissions. It is also possible that there are UL URLLC transmissions. In this case the gNB shall compare the QoS classes of the DL URLLC transmissions with those of the UL URLLC transmissions, and perform the preemptive duplex direction switching (that is, from UL to DL) only if the former have higher priority than the latter.

Alternatively, the gNB may reserve some radio resource allocation regions (RRARs) in a UL time slot to be exclusively used by URLLC UEs. When the gNB preemptively switches from UL to DL, it avoids these reserved RRARs. This is shown in Figure 2. It is noted that the delay T2’ may be larger than T2 in Figure 4 where RRARs are not reserved.

Figure 2: The gNB avoids the reserved UL RRARs when pre-emptively switching from UL to DL.

The UE needs to be ready for DL URLLC transmission from the gNB when the gNB does preemptive TDD duplex direction switching without getting a control signal from the control channel that indicates the upcoming DL URLLC transmission. In the conventional TDD mode, a UE in a UL time slot does not monitor the DL control region. To support preemptive duplex direction switching, a UE that is not transmitting should be in the DL listening mode. The gNB may switch the duplexing direction mini-slot boundaries to reduce the complexity of the processing needed for the monitoring UE.

Proposal 2: NR studies mechanisms for minimizing the impact on UL transmissions resulting from dynamically changing the duplex direction in the TDD operation mode when DL URLLC traffic is multiplexed with DL eMBB traffic.

# Summary

This contribution discussed dynamic resource sharing between URLLC and eMBB data in the DL and UL when the transmissions from URLLC overlap with resources of an ongoing/scheduled longer transmission for TDD. We propose the following:

Proposal 1: NR supports dynamically changing the duplexing direction within the slot for the TDD operation when URLLC traffic is multiplexed with eMBB traffic.

Proposal 2: NR studies mechanisms for minimizing the impact on UL transmissions resulting from dynamically changing the duplex direction in the TDD operation mode when DL URLLC traffic is multiplexed with DL eMBB traffic.

# References

RAN1 NR Ad-Hoc Chairman’s Notes, Spokane, USA, 16th – 20th January 2017