3GPP TSG RAN WG1 Meeting #88				    			 	 R1-1702344

# Athens, Greece 13th - 17th February 2017

# Source:	ETRI

# Title:		Discussion on DMRS design for high speed scenario

# Agenda item:	8.1.2.4.2

# Document for:	Discussion/Decision

Introduction

Regarding the DMRS design for the 5G NR, the following agreements were made in the 3GPP RAN1 NR ad-hoc January meeting [1]:

Agreements:

- For the design of front-loaded DMRS, Alt. 1 is agreed as a working assumption.

  - Alt. 1: Front-loaded DMRS is mapped over 1 or 2 adjacent OFDM symbols.

    - FFS: Further down-selection between 1 and 2, if necessary

    - Companies are encouraged to propose further details

  - Companies are encouraged to provide comparison between Alt. 1 and Alt. 2.

    - Alt. 2: Design in R1-1700352 (Front-loaded DMRS is mapped over 3 or 4 adjacent OFDM symbols).

- Additional DMRS can be configured for the later part of the slot.

  - FFS: Density reduction compared to front-loaded DMRS

- DMRS configuration can be up to the max. number of DMRS ports.

- Note: DMRS design should take into account channel estimation accuracy for low-to-high SINR scenario, throughput performance for SU and MU-MIMO, DMRS overhead, receiver complexity, receiver implementation, support of the use cases/features, UL/DL symmetry, etc.

- FFS: Relative timing relationship between front-loaded DMRS and NR-PDSCH

  - Option 1: The first symbol of front-loaded DM-RS is fixed regardless of the first symbol of NR-PDSCH.

  - Option 2: The first symbol of front-loaded DM-RS is no later than the first symbol of NR-PDSCH.

According to the above agreements, various NR DMRS patterns are supported which can be designed in a user specific manner. This is particularly beneficial in supporting diverse use cases and deployment scenarios considered in the 5G NR. For each user having unique channel properties such as channel coherence time and channel coherence frequency, a suitable DMRS pattern is employed in order to maximize its link performance.

Among the various deployment scenarios, the high speed scenario has gained substantial interest in the 5G NR, which focuses on supporting consistent user experience with very high mobility (up to 500 km/h) along continuous railway tracks [2]. Hence, in order to achieve the above target requirements, an effort to design the DMRS pattern considering the channel characteristics of the high speed scenario while maintaining some degree of commonalities with other deployment scenarios.

In this proposal, we discuss possible DMRS patterns suitable for the NR high speed scenario Option 2 (30 GHz carrier frequency and Macro + relay nodes) and provide link-level performance comparison among them. The presented DMRS patterns and their link-level evaluation results will not be restricted only to the high speed scenario, but will also useful to other deployment scenarios with high mobility.

Possible DMRS patterns

Front-loaded DMRS patterns

In the NR, the use of baseline front-loaded RSs is supported due to its ability for achieving low latency. In the time-frequency resource grid, the front-loaded RSs can be located just after the control region, followed by data region, as seen in Figure 1a. As soon as obtaining channel estimates from the front-loaded RSs, the receiver can demodulate data in the data region. This front-loaded RS structure is particularly advantageous in decoding latency reduction for low mobility scenarios where channel coherence time is longer than the duration of two consecutive front-loaded RSs in the time domain. We assume the DMRS can be used for Ports 0 and 1.

However, allocating only the front-loaded DMRS degrades the link performance especially as the UE speed becomes higher (i.e., channel coherence time becomes shorter). Although the channel information in the data region can be obtained by interpolation, the channel information accuracy becomes worse. Hence, we also consider the front-loaded DMRS patterns with 2x- and 4x-time densities in Figure 1b and Figure 1c, respectively.

Figure 1. Front-loaded DMRS patterns with different time-densities. a) Baseline front-loaded DMRS; b) Front-loaded DMRS with 2x time-density; c) Front loaded DMRS with 4x time-density

Proposal 1: NR should study the front-loaded DMRS patterns with different time-domain densities, e.g., 2x and 4x time densities.

Reducing frequency-domain density

When the channel coherence frequency is not too short, we can consider reducing frequency-domain density of the DMRS. By doing so, the overhead due to the DMRS can be significantly reduced. Examples of frequency-domain low-density DMRS patterns are given in Figure 2a (4 REs per RB) and Figure 2b (2REs per RB), respectively. The DMRS patterns support 2 ports.

Figure 2. Introduction of time continuously allocated additional RS for phase tracking

Proposal 2: NR should study the DMRS patterns with reduced frequency-domain densities, e.g., 4 and 2 REs per RB in the frequency-domain.

Simulation results

Simulation parameters

The link-level simulation parameters are summarized in Table 2. For the phase noise model, the multi-pole/zero model for 30 GHz is used [3], which was agreed in [4]. In the simulation, the common phase error is compensated. The frequency offset due to the Doppler shift occurring from the strong LOS path is also compensated. For the multi-port DMRS allocation, FDM is used.

Table 2. Link-level simulation parameters

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30 GHz |
| System bandwidth | 80 MHz |
| Subcarrier spacing | 120 kHz |
| Channel coding | LTE Turbo |
| MCS | 16QAM 3/4 |
| Number of layers | 1, 2 |
| Channel estimation | LS estimation and linear interpolation |
| Equalizer | LMMSE |
| Channel model | - CDL-D (DS = 10ns, K-factor = 13.3 dB) - Parameter set # 1: 5(ASD), 5(ASA), 1(ZSA), 1(ZSD) - ZoD and ZoA for cluster #1 are fixed at 90 degrees |
| TRP antenna configuration | - (M,N,P,Mg,Ng) = (8,8,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=65, directivity 8dB) |
| UE antenna configuration | - (M,N,P,Mg,Ng) = (8,8,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=65, directivity 8dB) |
| Phase noise model | Multi-pole/zero model [3] |
| UE speed | 500 km/h |

BLER results

We plotted the BLER as a function of SNR for different DMRS patterns in Figure 3 (1 Layer) and in Figure 4 (2 Layers). The BLER when having the ideal channel information is also plotted for comparison. As expected, it can be seen that the BLER is decreased when denser DMRS pattern is employed both in the time and frequency domains. Due to short coherence time, we need to have more than 4 DMRS within a slot in the time domain in order to satisfactorily support the UE speed of 500 km/h. In the frequency-domain, reduced DMRS allocation does not significantly degrade BLER. This is because the employed CDL-D channel with DS 10ns has strong LOS path and weaker multi-path components, making the channel coherence bandwidth larger.

Observation 1: In the high speed scenario with UE speed of 500 km/h, dense DMRS allocation (more than 4 DMRSs within a slot) in the time domain shows lower BLER.

Observation 2: In the high speed scenario, reduced DMRS allocation does not significantly degrade BLER.

Figure 3. BLER vs. SNR for different DMRS patterns (1 Layer)

Figure 4. BLER vs. SNR for different DMRS patterns (2 Layers)

Spectral efficiency results

We plotted the spectral efficiency as a function of SNR for different DMRS patterns in Figure 5 (1 Layer) and in Figure 6(2 Layers). Overall, the performance trend is similar to that of BLER. The spectral efficiency when the ideal channel information is available is also plotted for comparison. The spectrum efficiency is degraded with less time-domain density DMRS. The effect of reducing DMRS density in the frequency domain can be advantageous in enhancing spectral efficiency. However, reducing the DMRS density in the frequency domain less than 4 REs per RB is not beneficial to improve the spectral efficiency.

Observation 3: In the high speed scenario, reducing DMRS density in the frequency domain can be advantageous in enhancing spectral efficiency, especially in the high SNR region.

Observation 4: In the high speed scenario, reducing DMRS density in the frequency domain less than 4 REs per RB is not beneficial to improve the spectral efficiency.

Figure 5. Spectral efficiency vs. SNR for different DMRS patterns (1 Layer)

Figure 6. Spectral efficiency vs. SNR for different DMRS patterns (2 Layers)

With these observation, we propose the followings.

Proposal 3: In the high speed scenario, in order to support high mobility up to 500 km/h, NR should study the DMRS patterns having more than 4 DMRSs symbols within a slot in the time-domain.

Proposal 4: In the high speed scenario, in order to enhance spectral efficiency, NR should study the DMRS patterns having 4 REs per RB in the frequency-domain.

Conclusion

In this contribution, we provided possible DMRS patterns which can be used for channel estimation for data demodulation. The presented DMRS patterns are particularly useful for high speed and high carrier frequency environments. Throughout extensive link level evaluation, we found the followings.

Proposal 1: NR should study the front-loaded DMRS patterns with different time-domain densities, e.g., 2x and 4x time densities.

Proposal 2: NR should study the DMRS patterns with reduced frequency-domain densities, e.g., 2 and 4 REs per RB in the frequency-domain.

Observation 1: In the high speed scenario with UE speed of 500 km/h, dense DMRS allocation (more than 4 DMRSs within a slot) in the time domain shows lower BLER.

Observation 2: In the high speed scenario, reduced DMRS allocation does not significantly degrade BLER.

Observation 3: In the high speed scenario, reducing DMRS density can be advantageous in enhancing spectral efficiency.

Observation 4: In the high speed scenario, reducing DMRS density in the frequency domain less than 4 REs per RB is not beneficial to improve the spectral efficiency.

Proposal 3: In the high speed scenario, in order to support high mobility up to 500 km/h, NR should study the DMRS patterns having more than 4 DMRSs symbols within a slot in the time-domain.

Proposal 4: In the high speed scenario, in order to enhance spectral efficiency, NR should study the DMRS patterns having 4 REs per RB in the frequency-domain.

References

- 3GPP Chairman’s notes RAN1# NR ad-hoc, Jan. 2017.

- 3GPP TR 38.913, “Study on scenarios and requirements for next generation access technologies,” v. 14.0.0, Oct. 2016.

- R1-163984, “Discussion on phase noise modeling,” RAN1#85, May 2016.

- R1-165685, “WF on Phase Noise Modeling,” RAN1#85, May 2016.