3GPP TSG-RAN WG1 #88	R1-1702369

Athens, Greece, 13th - 17th February 2017

Source:	Ericsson

Title:	Remaining Issues on Beamformed CSI-RS

Agenda Item:	7.2.2.2

Document for:	Discussion and Decision

# Introduction

In RAN1#86, the following conclusions were made with regards to aperiodic CSI-RS resource selection [1].

Conclusion:

- 3 alternatives are identified:

  - Alt1 Reuse A-CSI request field and add more bit(s) if necessary

  - Alt2 Reuse CS field (see R1-1610815, R1-1610816)

  - Alt3 Add ceil(log2(N))-bit or 2-bit DCI field

- Decide between the above 3 alternatives.

The three alternatives were further discussed via an email discussion where the conclusions are given in [1].  In the first part of this contribution, we present our views on the different alternatives.  Furthermore, we propose the reuse of cyclic shift field (Alt 2) for aperiodic CSI-RS resource selection.

In RAN1#86bis, the following agreements were made with regards to overhead reduction for Class B CSI-RS in eFD-MIMO:

Agreement:

- For Class B NZP CSI-RS:

  - Support CSI-RS density of d ∈ {1,1/2} RE/RB/port

  - Additional values of d=1/3 and/or 1/8 are not precluded for Class B if significant benefit can be shown at RAN1#87.

In the second part of this contribution, we present simulation results on the different CSI-RS densities (i.e., d values) that should be supported in addition CSI-RS densities of 1 and 1/2.  Based on our results, we propose that the additional density value 1/3 should be supported in Rel-14 eFD-MIMO for Class B NZP CSI-RS.

# On Aperiodic CSI-RS Resource Selection

## Drawbacks with Alt1 and Alt3

A common draw back with Alternatives 1 and 3 is that they both result in unnecessary overhead increase for PDCCH and EPDCCH messages.  As described in [2], Alt1 requires some additional B bits to be added to the A-CSI request field and Alt3 adds a new DCI field with up to two bits.  In both cases, the additional bits are added to the payload size of DCI formats 0 and 4.  In addition, the DL-related DCI format 1A needs to have the same payload size as DCI format 0.  If the payload size of DCI format 0 is increased by some additional bits, then DCI format 1A may need to be padded with additional bits to ensure the same payload size between DCI formats 0 and 1A.  Hence, using alternatives 1 and 3 to indicate one out of N aperiodic CSI-RS resources results in unnecessary overhead increase for PDCCH/EPDCCH messages carrying DCI formats 0, 4, and 1A.  Furthermore, Alternatives 1 and 3 are unsuitable for common search space for PDCCH as additional bits cannot be added DCI formal 0.

Observation 1: For aperiodic CSI-RS, introducing additional bits in DCI through Alt1 or Alt 3 to select one out of N CSI-RS resources would result in unnecessary overhead increase for PDCCH/EPDCCH messages carrying DCI formats 0, 4, and 1A.

Observation 2: Alternatives 1 and 3 are unsuitable for common search space for PDCCH as additional bits cannot be added DCI formal 0.

## Alt 2: Reusing CS Field

The Cyclic Shift field in uplink-related DCI has 3 bits and each code point in the field is used to indicate a cyclic shift parameter  and length-2 OCC code where λ=0,1,…,ν-1 and ν is the number of layers to be transmitted in the PUSCH scheduled by the uplink grant.  Since the maximum number of aperiodic CSI-RS resources that can be activated is agreed to be 4 (i.e., N ≤ 4) [1], the 3-bit length of the Cyclic Shift field provides sufficient freedom to select one out of the N aperiodic CSI-RS resources.  For instance, a 3-bit code point in the Cyclic Shift field can be mapped to one of the N aperiodic CSI-RS resources via

| , | Equation 1 |
| --- | --- |

where  is the aperiodic CSI-RS resource index and  is mapped from the Cyclic Shift field as defined in Table 9.1.2-2 of 3GPP TS 36.213.

Figure 1 below shows an example where N = 4 aperiodic CSI-RS resources can be activated and how these resources can be mapped (in a UE-specific manner) to the codepoints of the Cyclic Shift field in the case of the legacy CS/OCC table.  In the case with N = 4, a maximum of 4 UEs can be triggered for Aperiodic CSI-RS transmission in a given subframe.

- For UEs not triggered for an aperiodic report (i.e. most UEs) in a subframe, the mapping of Equation 1 does not apply and the codepoints can be chosen freely.  The mappings shown in the example of Figure 1 only applies to UEs being triggered for Aperiodic CSI-RS transmission.

- If four UEs are triggered for aperiodic CSI-RS transmission in a subframe, then four codepoints related to unique Aperiodic CSI-RS resources can be chosen.  Each Aperiodic CSI-RS resource can still be indicated by two different codepoints (i.e., Aperiodic CSI-RS resource 0 can be indicated by either 000 or 100).

Due to this flexibility associated with Alternative 2, the Cyclic Shift codepoint can be allocated first as usual.  When a UE also needs to be triggered for Aperiodic CSI-RS transmission, the Cyclic Shift codepoint for the UE can also be used to selecting a unique aperiodic CSI-RS resource.  Hence, there is almost no performance impact associated with Alternative 2 when compared to legacy operation.

Observation 3: With Alternative 2 and the agreed maximum value of N = 4, the CS field codepoints can first be allocated to UEs as usual, and when a UE also needs to be triggered for Aperiodic CSI-RS, the CS codepoint for the UE can be mapped to a unique aperiodic CSI-RS resource.

Figure 1.  Example aperiodic CSI-RS resource selection with N = 4

The only constraint with using CS codepoints to aperiodic CSI-RS resource mapping is that when UEs to be triggered for A-CSI in a subframe are allocated to code points that are mapped to the same A-CSI-RS resource.  This can be, however, resolved by eNB implementation. One way is to reallocate the codepoint(s) for some of the UEs.  This could potentially cause PHICH collision.  However, as discussed in [3], the impact of PHICH collision and scheduling restriction are quite small.  Another way is to simply trigger only the UEs whose codepoints are mapped to different aperiodic CSI-RS resources in a subframe.  Hence, the advantages of Alternative 2 clearly outweighs its disadvantages.  Furthermore, unlike Alternatives 1 and 3, Alternative 2 is suitable for common search space for PDCCH as Alternative 2 does not need any additional bits in DCI formal 0.

Given the discussion above, we propose that Alternative 2 of reusing the CS field is used to select 1 out of N activated aperiodic CSI-RS resources.  The major benefits of reusing the CS field is that it does not increase overhead for PDCCH/EPDCCH messages carrying DCI formats 0, 4, and 1A.

Observation 4: For aperiodic CSI-RS resource selection, the code points of the Cyclic Shift field can be flexibly used to select one out of N CSI-RS resources.

Proposal 1: For Aperiodic CSI-RS transmission, use code points from the existing Cyclic Shift field in the UL related DCI to indicate transmission on one out of N aperiodic CSI-RS resources.

# On Additional Density Values for Class B NZP CSI-RS

In this section, we present simulation results on the different CSI-RS densities (i.e., d values) that should be supported in addition CSI-RS densities of 1 and 1/2.  In our simulations we have considered an 4x8 antenna using 2x1 virtualization; this antenna layout will hence correspond to 32 ports. We furthermore simulate hybrid CSI-RS operation where we transmit the non-precoded CSI-RS, hence 32 ports, once every 50 subframes.  Additional, simulation assumptions are given in the Appendix.

For each UE, based on the report corresponding to the non-precoded CSI-RS, one Class B CSI-RS within a pool of eight 4-ports CSI-RSs is selected. The pool of class B CSI-RSs is transmitted once every 5 subframes except in the subframe where the non-precoded CSI-RS is transmitted. Each of the eight class B CSI-RSs are beamformed in different horizontal directions. Within a class B CSI-RS there are 4 CSI-RS ports. The hybrid CSI approach thus allows a selection in the horizontal plane every 50ms and, within the selected horizontal direction, in the vertical plane every 5ms.

The performance comparisons between different CSI-RS densities is given in Tables 1-2 for 3D-UMi and 3D-UMa where a CSI-RS reuse factor of 1 is assumed.  The performance gains shown in Tables 1-2 are relative to a 32 port hybrid CSI-RS mode scheme with 1 RE/RB/port.  From the tables, it is seen that a configuration with CSI-RS density 1/3 operates on par with a CSI-RS density 1/2 in terms of cell edge and mean user throughput for 3D-UMi.  Also a density reduction of 1/4 performs better than the baseline but reducing the density even further results in performance losses. For 3D-UMa a higher CSI-RS density appears to be required. Here the performance starts to drop already for a CSI-RS density of 1/3.

Hence, we make the following observations and proposal:

Observation 5: With CSI-RS reuse 1, Class B CSI-RS densities 1/3 and 1/4 can outperform a configuration with CSI-RS density 1 in terms of cell edge and mean user throughput for 3D-UMi.

Observation 6: With CSI-RS reuse 1, a Class B CSI-RS density of 1/8 results in performance losses compared to a configuration with CSI-RS density 1.

Proposal 2: For Class B eFD-MIMO, a CSI-RS density value of 1/3 should be supported in addition to CSI-RS density values of 1 and 1/2.

Table 1. Performance comparison between different CSI-RS densities for 3D-UMi for a non full buffer simulation at 50% resource utilization.

| CSI-RS reduced Density d | 1 | 1/2 | 1/3 | 1/4 | 1/8 |
| --- | --- | --- | --- | --- | --- |
| Cell edge gain [%] | 0 | 12 | 8 | 5 | -23 |
| Normalized user throughput gain [%] | 0 | 5 | 4 | 5 | -5 |

Table 2. Performance comparison between different CSI-RS densities for 3D-UMa for a non full buffer simulation at 50% resource utilization.

| CSI-RS reduced Density d | 1 | 1/2 | 1/3 | 1/4 | 1/8 |
| --- | --- | --- | --- | --- | --- |
| Cell edge gain [%] | 0 | 6 | -5 | -17 | -44 |
| Normalized user throughput gain [%] | 0 | 4 | -1 | -5 | -18 |

# Conclusion

In the first part of this contribution, we present our views on the different alternatives identified for aperiodic CSI-RS resource selection.  In the second part of the contribution, we present simulation results on the different CSI-RS densities (i.e., d values) that should be supported in addition CSI-RS densities of 1 and 1/2 for Class B.  Based on the discussions, we make the following observations and proposals:

Observation 1: For aperiodic CSI-RS, introducing additional bits in DCI through Alt1 or Alt 3 to select one out of N CSI-RS resources would result in unnecessary overhead increase for PDCCH/EPDCCH messages carrying DCI formats 0, 4, and 1A.

Observation 2: Alternatives 1 and 3 are unsuitable for common search space for PDCCH as additional bits cannot be added DCI formal 0.

Observation 3: With Alternative 2 and the agreed maximum value of N = 4, the CS field codepoints can first be allocated to UEs as usual, and when a UE also needs to be triggered for Aperiodic CSI-RS, the CS codepoint for the UE can be mapped to a unique aperiodic CSI-RS resource.

Observation 4: For aperiodic CSI-RS resource selection, the code points of the Cyclic Shift field can be flexibly used to select one out of N CSI-RS resources.

Observation 5: With CSI-RS reuse 1, Class B CSI-RS densities 1/3 and 1/4 can outperform a configuration with CSI-RS density 1 in terms of cell edge and mean user throughput for 3D-UMi.

Observation 6: With CSI-RS reuse 1, a Class B CSI-RS density of 1/8 results in performance losses compared to a configuration with CSI-RS density 1.

Proposal 1: For Aperiodic CSI-RS transmission, use code points from the existing Cyclic Shift field in the UL related DCI to indicate transmission on one out of N aperiodic CSI-RS resources.

Proposal 2: For Class B eFD-MIMO, a CSI-RS density value of 1/3 should be supported in addition to CSI-RS density values of 1 and 1/2.

# References

Chairman’s Notes, 3GPP TSG-RAN WG1 #86bis, Lisbon, Portugal, October 10-14, 2016.

R1-1702368, “Email Discussion [86b-12] Aperiodic CSI-RS Resource Selection”, Ericsson, 3GPP TSG-RAN WG1 #88, Athens, Greece, February 13-17, 2017.

R1-1612678, “On PHICH Collision Issue Related to Reusing CS Field for Aperiodic CSI-RS Resource Selection”, Ericsson, 3GPP TSG RAN WG1 #87, Reno, NV, USA, November 14-18, 2016.

# Appendix

| Simulation parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D-UMi 200m ISD, 3D-UMa 500m ISD |
| Antenna Configurations | 4x8 with 2x1 virtualization tilt: 130° |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (3D-UMi), 46 dBm (3D-UMa) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency |
| CRS interference | 2 CRS ports, non-shifted CRS. |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead accounted for. Channel estimation error modelled. |
| Codebook | 2D Grid of Beams based on DFT |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |