3GPP TSG RAN WG1 #88	R1-1702609

February 13th - 17th, 2017

Athens, Greece

Agenda item:	8.1.2.3.1

Source: 	Qualcomm Incorporated

Title: 	Details of CSI framework

Document for:	Discussion/Decision

# Introduction

In RAN1 #AH1_NR, several WFs on CSI acquisition [1-3] were discussed.  Some agreements are as follows [4].

CSI framework

- At least following are dynamically selected by L1 or L2 signaling, if applicable

  - One or multiple CSI reporting settings within the CSI measurement setting

  - One or multiple CSI-RS resource sets selected from at least one Resource setting

  - One or multiple CSI-RS resources selected from at least one CSI-RS resource set

- FFS until the next meeting about details of dynamic triggering

Triggering of CSI reporting

- For periodic CSI-RS,

  - Semi-persistent CSI reporting is activated/deactivated by MAC CE and/or DCI

  - Aperiodic CSI reporting is triggered by DCI

    - FFS: Necessity of additional signaling with MAC CE

- For semi-persistent CSI-RS,

  - Periodic CSI reporting is not supported.

  - Semi-persistent CSI reporting is activated/deactivated by MAC CE and/or DCI

    - Semi-persistent CSI-RS is activated/deactivated by MAC CE and/or DCI

    - FFS: Relationship of signaling between CSI reporting and CSI-RS transmission

  - Aperiodic CSI reporting is triggered by DCI

    - Semi-persistent CSI-RS is activated/deactivated by MAC CE and/or DCI

    - FFS: Necessity of additional signaling with MAC CE

- For aperiodic CSI-RS,

  - Periodic [and semi-persistent] CSI reporting is not supported

  - Aperiodic CSI reporting is triggered by DCI

    - Aperiodic CSI-RS is triggered by DCI and/or MAC CE

    - FFS: Relationship of signaling between CSI reporting and CSI-RS transmission, e.g., common DCI signaling between CSI reporting and CSI-RS transmission

    - FFS: Necessity of additional signaling with MAC CE

- Note that further down selection can be done later between MAC CE and DCI in above bullets

- Note that it is possible to dynamically trigger RS and reports through links in the measurement setting

Timing for aperiodic CSI

- NR supports mechanism(s) to trigger aperiodic CSI-RS and aperiodic CSI reporting simultaneously.

  - FFS: signaling details (e.g. single DCI or separate DCI)

  - FFS: method(s) for interference measurement

  - FFS: reference resource of aperiodic CSI reporting

- For aperiodic CSI-RS timing offset X, support X=0 at least, if aperiodic CSI-RS triggering is done by DCI

  - FFS: supporting other values of X

  - Note: ‘aperiodic CSI-RS timing offset X’ refers to the time gap between aperiodic CSI-RS triggering and aperiodic CSI-RS transmission w.r.t. number of slots.

- For CSI reporting timing offset Y that is fixed or configurable by the network but with certain restriction on lower limit of Y to provide sufficient CSI computation time.

  - Candidate values of Y are fixed or pre-determined by certain rule(s).

    - Rule is FFS (e.g. number of CSI measurement/RS/reporting settings, CSI feedback type, number of ports, nearest CSI-RS transmission timing, UE capability, etc.)

  - Note: ‘aperiodic CSI reporting timing offset Y’ refers to the time gap between aperiodic CSI reporting triggering and aperiodic CSI reporting w.r.t. number of slots.

  - FFS Y is fixed or configurable

  - FFS configuration is done by DCI and/or MAC CE and/or higher layer signaling

In this contribution, we discuss open issues in CSI-RS.

# Discussion

## Open issues in CSI framework

According to the agreed CSI framework, a UE can be configured with N CSI reporting settings, M resource settings, and a single CSI measurement setting, where N ≥ 1, M ≥ 1.  A resource setting includes S ≥ 1 CSI-RS resource sets.  In the sth CSI-RS resource set, Ks ≥ 1 CSI-RS resources can be configured.  The CSI measurement setting includes L ≥ 1 links.  Each of the L links corresponds to a CSI reporting setting and a resource setting.  Such CSI framework can support flexible configuration of CSI-RS transmission and CSI measurement/reporting.  In this section, we discuss dynamic indication of CSI-RS resources and CSI reporting settings.

### Dynamic indication of CSI-RS resources

It has been agreed that NR supports periodic, semi-persistent and aperiodic CSI-RS transmission.  For periodic CSI-RS transmission the CSI-RS resource is configured by RRC signaling.  It unnecessary to change the CSI-RS resource until the periodic CSI-RS transmission is reconfigured by RRC.  For semi-persistent and aperiodic CSI-RS transmission, the CSI-RS resource may be dynamically indicated to support CSI-RS resource sharing among a group of UEs.  The dynamic indication can be signalled by L1 and/or L2 signaling.

Observation 1: For periodic CSI-RS transmission, there’d no need to dynamically indicate the CSI-RS resource in configured resource settings.

Proposal 1: For periodic CSI-RS transmission, the CSI-RS resource is configured by RRC signaling.

According to the definition of resource setting, the dynamic triggering of a CSI-RS resource may include the signaling of s, the index of a CSI-RS resource set, if S > 1; and the signaling of the index of a CSI-RS resource within the indicated CSI-RS resource set, if Ks > 1.  There are three alternatives to signal those indices a UE:

- Alt-1: Indicated by L1 signaling (e.g., DCI)

- Alt-2: Indicated by L2 signaling (e.g., MAC CE)

- Alt-3: Indicated by a hybrid of L1 and L2 signaling

The feasibility of Alt-1 depends on the DL control channel design and the location of CSI-RS.  First, it’s related to the payload size of DCI.  Since a UE may be configured with one or multiple CSI-RS resource sets and each set may further include one or multiple CSI-RS resources.  If both indices are jointly encoded, up to log2s=1SKs bits need to be included in the DCI.  Second, it’s related to the control channel decoding latency and the location of CSI-RS.  As shown in Figure 1, in a DL-centric slot, the CSI-RS may be transmitted in the early part of a slot.  If a UE cannot decode the DCI before the symbol containing CSI-RS resources, the UE has to buffer the signals received on all CSI-RS resources in configured resource settings.  In case of UL-centric slot, the CSI-RS may be multiplexed with PDCCH and transmitted in the DL duration of a slot.  Similarly, the UE also has to buffer the DL symbols.  In both cases, there would be a significantly increase of UE complexity due to the buffering.  So, dynamic indication of CSI-RS resource solely via L1 signaling is problematic.

Observation 2: Dynamic indication of CSI-RS resource in configured resource settings solely via L1 signaling may cause an increase in DCI payload size and an increase in UE complexity.

Figure 1.  Indication of CSI-RS resource via L1 signaling.

Compared to L1 signaling, L2 signaling provides a more reliable way to indicate the CSI-RS resource for CSI-RS transmission.  As the L2 signaling is delivered via PDSCH, the gNB is aware of the reception of indication by a UE.  If the UE failed to receive the indication, the gNB can send it again.  This is an important mechanism for dynamic triggering semi-persistent CSI-RS transmission, as the UE won’t miss multiple shot of CSI-RS transmission.

Observation 3: Dynamic indication of CSI-RS resource in configured resource settings solely via L2 signaling can avoid miss of semi-persistent CSI-RS transmission.

Proposal 2: For semi-persistent CSI-RS transmission, activation/deactivation of one CSI-RS resource in the configured resource settings is indicated by L2 signaling.

But for aperiodic CSI-RS transmission, the indication is expected to be signaled in a timely fashion to facilitate fast sharing of CSI-RS resources.  Purely based on L2 signaling cannot meet such requirement.  This is because the L2 signaling is conveyed by PDSCH.  If the indication is transmitted in slot n, the CSI-RS transmission can only happens in slot n + k, where k ≥ 1.  Alt-3 provides a good tradeoff between Alt-1 and Alt-2.  The dynamic indication of CSI-RS resource can be done in a two-step manner.  First, a L2 signaling can be used to activate/deactivate a subset of the configured CSI-RS resource sets.  Then, a L1 signaling can be employed to indicate the CSI-RS transmission on one of the CSI-RS resources in the activated CSI-RS resource sets.  The activation of a smaller number of CSI-RS resource sets can reduce the DCI overhead as well as the buffering requirements.

Observation 4: Dynamic indication of CSI-RS resource in configured resource settings by a hybrid of L1 and L2 signaling provides a better tradeoff between efficiency and complexity for aperiodic CSI-RS transmission.

Proposal 3: For aperiodic CSI-RS transmission, activation/deactivation of S′ CSI-RS resource set(s) from a resource setting by L2 signaling, where S′ < S if S > 1.  The maximum number of activated CSI-RS resources is dependent on UE capability.

Proposal 4: For aperiodic CSI-RS transmission, one CSI-RS resource from the activated CSI-RS resource set(s) is indicated by L1 signaling.

### Dynamic indication of CSI reporting settings

NR supports three types of CSI reporting, i.e., aperiodic CSI reporting, semi-persistent CSI reporting, and periodic CSI reporting.  The semi-persistent CSI reporting is equivalent to periodic CSI-RS reporting with ON/OFF.  It requires multiple shots of CSI-RS.  The multi-shot CSI-RS can be transmitted as semi-persistent CSI-RS or as multiple aperiodic CSI-RS.  For semi-persistent CSI-RS transmission, one only needs an activation signaling to turn on the transmission and a deactivation signaling to turn off the transmission.  For multi-shot aperiodic CSI-RS transmission, at least one L1 signaling is needed to trigger each shot of aperiodic CSI-RS.

Observation 5: Aperiodic CSI-RS based semi-persistent CSI reporting may lead to an unnecessary increase of dynamic signaling.

Proposal 5: For aperiodic CSI-RS, semi-persistent CSI reporting is not supported.

Regarding the triggering of semi-persistent CSI reporting, one may consider L1 and/or L2 signaling.  We notice that a UE may miss the activate/deactivation indication of a semi-persistent CSI reporting if it fails to decode the corresponding L1 signaling.  If a UE misses a deactivation signaling, it will keep transmitting CSI on the UL resource which may already be allocated to some other UE.  So, it’s more preferred to use L2 signaling to activate or to deactivate the semi-persistent CSI reporting.

Proposal 6: Semi-persistent CSI reporting is activated/deactivated by L2 signaling.

To provide a timely triggering, L1 signaling is favoured for triggering aperiodic CSI reporting.  When the aperiodic CSI reporting is based on an aperiodic CSI-RS, the aperiodic CSI-RS transmission and the aperiodic CSI reporting shall be triggerd jointly.  Otherwise, a UE may receive an indication of aperiodic CSI reporting but fail to figure out which CSI-RS shall be used for the measurement, if the triggering of aperiodic CSI-RS is missed.

Proposal 7: For aperiodic CSI reporting based on aperiodic CSI-RS, aperiodic CSI reporting and aperiodic CSI-RS transmission are jointly indicated by a single L1 signaling.

## Timing for aperiodic CSI reporting

If a UE receives an indication of aperiodic CSI reporting in slot n, it shall report the measured CSI in slot n + Y.  The value of Y shall be chosen to guarantee that the UE has sufficient time for CSI computation.  The value of Y shall also be selected such that the aperiodic CSI is reported in a timely manner with an affordable UE complexity.  It is open for discussion whether the value of Y is fixed or configurable by the network.  We notice that a single fixed value of Y may be insufficient given that the NR has a flexible CSI framework which supports various CSI-RS configurations, CSI types, and different contents of CSI to report.  For example, the time for deriving a wideband CQI only reporting is much less than the time required for deriving a Type II subband PMI/CQI reporting.  For another example, the time for deriving a 4-port CSI reporting is much less than the time for deriving a 32-port CSI reporting.  So, the value of Y shall be dependent on the content of the CSI report, including number of CSI-RS ports, codebook types, and frequency granularity of CSI components to be reported, etc.

We don’t see any necessity for allowing configurable timing for aperiodic CSI reporting.  As the aperiodic CSI reporting supposes to provide the latest CSI, the value of Y should be as small as possible to make sure the gNB can acquire the latest CSI.  Not to mention that a configurable timing needs more sophisticated scheduling of UL resources and more buffering requirements in UE implementation.

Proposal 8: The value of aperiodic CSI reporting timing is fixed and is dependent on the content of CSI report, at least including the number of CSI-RS ports, the codebook type, the CSI components, and the frequency granularity of each components.

# Conclusions

To summarize, we discussed open issues in NR CSI framework.  We have following observations:

Observation 1: For periodic CSI-RS transmission, there’d no need to dynamically indicate the CSI-RS resource in configured resource settings.

Observation 2: Dynamic indication of CSI-RS resource in configured resource settings solely via L1 signaling may cause an increase in DCI payload size and an increase in UE complexity.

Observation 3: Dynamic indication of CSI-RS resource in configured resource settings solely via L2 signaling can avoid miss of semi-persistent CSI-RS transmission.

Observation 4: Dynamic indication of CSI-RS resource in configured resource settings by a hybrid of L1 and L2 signaling provides a better tradeoff between efficiency and complexity for aperiodic CSI-RS transmission.

Observation 5: Aperiodic CSI-RS based semi-persistent CSI reporting may lead to an unnecessary increase of dynamic signaling.

We propose following:

Proposal 1: For periodic CSI-RS transmission, the CSI-RS resource is configured by RRC signaling.

Proposal 2: For semi-persistent CSI-RS transmission, activation/deactivation of one CSI-RS resource in the configured resource settings is indicated by L2 signaling.

Proposal 3: For aperiodic CSI-RS transmission, activation/deactivation of S′ CSI-RS resource set(s) from a resource setting by L2 signaling, where S′ < S if S > 1.  The maximum number of activated CSI-RS resources is dependent on UE capability.

Proposal 4: For aperiodic CSI-RS transmission, one CSI-RS resource from the activated CSI-RS resource set(s) is indicated by L1 signaling.

Proposal 5: For aperiodic CSI-RS, semi-persistent CSI reporting is not supported.

Proposal 6: Semi-persistent CSI reporting is activated/deactivated by L2 signaling.

Proposal 7: For aperiodic CSI reporting based on aperiodic CSI-RS, aperiodic CSI reporting and aperiodic CSI-RS transmission are jointly indicated by a single L1 signaling.

Proposal 8: The value of aperiodic CSI reporting timing is fixed and is dependent on the content of CSI report, at least including the number of CSI-RS ports, the codebook type, the CSI components, and the frequency granularity of each components.

# References

- R1-1701292, “WF on CSI framework for NR,” Samsung, Ericsson, LGE, NTT DOCOMO, ZTE, ZTE Microelectronics, KT, Huawei, HiSilicon, Intel.

- R1-1701313, “WF on periodic/semi-persistent/aperiodic CSI reporting,” NTT DOCOMO, Intel, LGE, Mitsubishi, Sharp, Softbank.

- R1-1701356, “WF on timings for aperiodic CSI-RS and aperiodic CSI reporting,” LGE, Samsung, Ericsson, ZTE, ZTE Microelectronics.

- Draft report of 3GPP TSG RAN WG1 #AH1_NR.