3GPP TSG RAN WG1 Meeting #88                 		R1-1702766

Athens, Greece 13th – 17th February 2017

Source:	Panasonic

Title: 	Mini-slot definition and control location for eMBB/URLLC multiplexing

Agenda Item:		8.1.3.4.1

Document for:	Discussion, Decision

Discussion

The last RAN1 meeting almost agreed no need of more work on mini-slot in SI without capturing explicit agreement. The agreement in RAN1#87 is following.

Agreements: - Mini-slots have the following lengths - At least above 6 GHz, mini-slot with length 1 symbol supported - FFS below 6 GHz including unlicensed band - FFS for URLLC use case regardless frequency band - FFS whether DL control can be supported within one mini-slot of length 1 - Lengths from 2 to slot length -1 - FFS on restrictions of mini-slot length based on restrictions on starting position - For URLLC, 2 is supported, FFS other values - Note: Some UEs targeting certain use cases may not support all mini-slot lengths and all starting positions - Can start at any OFDM symbol, at least above 6 GHz - FFS below 6 GHz including unlicensed band - FFS for URLLC use case regardless frequency band - A mini-slot contains DMRS at position(s) relative to the start of the mini-slot

On the other hand, there were some concern on current situation on mini-slot like following.

- "Above or below 6GHz boundary" is not reasonable as non-analogue beam forming operation above 6GHz is also possible.

- "URLLC" would be just a type of QoS at the end. URLLC term itself would not be specification term.

It is also pointed out that it is unclear whether mini-slot definition itself includes control resource or not.

We agree these concern and propose following for mini-slot character.

Proposal: 	Mini-slot supports at least following two cases of the operations.

Case 1: When slot transmission can be punctured by mini-slot

• The supported length is only 2

• Starting position is restricted.

• Control for mini-slot is located within mini-slot.

Case 2: When slot transmission is never punctured by mini-slot

•The supported length is any OFDM symbol length from 1 to slot length -1.

• Can start any OFDM symbol i.e. starting position is not restricted.

• Control for mini-slot is located around start of slot.

Case 1 corresponds to the multiplexing between eMBB/URLLC case in the previous agreement. Case 2 corresponds to above 6GHz case in the previous agreements. But both are extended respectively by categorized by "slot transmission is punctured or not". Then above 6GHz or below categorization is not required. URLLC term is not required. In our understanding, the most concern related to full flexible mini-slot length is coming from how mini-slot is punctured. By above categorization, this concern is addressed. Another concern on full flexible mini-slot length is the location of control. In URLLC usage, control needs to be located within mini-slot in order to reduce the latency. In case of analogue beam forming case, control should be sufficient only at the around start of the slot. This means the control in slot can schedule mini-slot in case 2.

Conclusion

Following is proposed for mini-slot definition.

Proposal: 	Mini-slot supports at least following two cases of the operations.

Case 1: When slot transmission can be punctured by mini-slot

• The supported length is only 2

• Starting position is restricted.

• Control for mini-slot is located within mini-slot.

Case 2: When slot transmission is never punctured by mini-slot

•The supported length is any OFDM symbol length from 1 to slot length -1.

• Can start any OFDM symbol i.e. starting position is not restricted.

• Control for mini-slot is located around start of slot.

Reference

[1]			R1-1701343		Summary of off-line discussion Mini-slots	   ZTE, ZTE Microelectronics