3GPP TSG RAN WG1 Meeting #88	R1-1703083

Athens, Greece, February 13 – 17, 2017

Source:	Huawei, HiSilicon

Title:	Summary of email discussion on sTTI scheduling

Agenda Item:	7.2.5.2.2.1

Document for:	Discussion and Decision

Introduction

This document summarizes the email discussion [87-24] on sTTI scheduling. Summaries of the discussions are captured in section 2 while the proposed agreements are in section 3. The detailed discussion is included in the appendix.

Email discussion summaries

There are 18 companies particiapated in the email discussion.

- The contents and operations for sDCI1.

- 16 companies support that a sPDSCH/sPUSCH is scheduled by a UE-specific sDCI1, while the legacy DCI contents is the starting point. One company proposes that a sPDSCH/sPUSCH is jointly scheduled by UE-specific sDCI1 and sDCI2. One company has different proposal in different cases.

- Companies propose to add these information to sDCI1: UL/DL DMRS indication for 2-symbol sTTI , sPDCCH occupied resource.

- In order to reduce sDCI1 control overhead, 5 companies propose to increase the RA granularity for sTTI.

- Whether sDCI1 schedule a single sTTI, or multiple sTTIs?

- 5 companies think sDCI1 schedules single sTTI is sufficient. The concern comes from scheduling inflexibility and the application scenario.

- 13 companies think sDCI1 schedule single sTTI is the baseline, while multiple sTTI can also be considered, especially for UL. The main benefit comes from control overhead reduction when there is data for multiple sTTI in the buffer, lower decoding latency and fast feedback than 1ms operation.

- sPDCCH search space configuration

- 6 companies prefer sPDCCH search space to be configured by higher layer. It reduces control overhead.

- 9 companies prefer the baseline is that sPDCCH search space is configured by higher layer, while sDCI2 could indicate dynamic refinement. It reduces blind detection attempts.

- 4 companies have different proposal in different cases.

- sPDCCH frequency resource

- The summary is similar to Q3.

- Activation/deactivation information of sDCI1 monitoring

- Proponent companies believe this information can be used to reduce DL control channel blind detection attempts and therefore save UE power consumption. The main benefit may happen in TCP slow start in eMBB traffic, but not in URLLC traffic with high requirement of latency and reliability.There are questions on its relationship with sTTI-specific DRX procedure in RAN2.

- Transmission frequency of sDCI2, if supported

- 11 companies support that sDCI2 is transmitted on demand, while UE blind detecs sDCI2 in every non-DRX subframe;

- 4 companies support that UE detects sDCI2 periodically in X (X>=1) subframes.

- sDCI2 miss detection issue

- 9 companies think if UE does not detect sDCI2 in one subframe, the UE would detect sDCI1 by the default RRC configured set. In this case, the UE would detect sDCI1 successfully with increased BD attempts.

- 5 companies think if UE does not detect sDCI2 in one subframe, the UE would detect sDCI1 by the last detected sDCI2. In this case, the UE’s BD attempts is not increased, but would probably not detect sDCI1 correctly. However, eNB would know it if no ACK/NACK received in a couple of sTTIs later.

- Views from 4 companies are not clear.

- Whether sDCI2 is UE-specific, group-specific, cell-specific

- UE-specific is supported by 6 companies

- Group-specific is supported by 10 companies

- Cell-specific is supported by 9 companies

- Contents of sDCI2

- “Aggregation level and/or candidates of sDCI1” is supported by 12 companies;

- “PRB set to sDCI1 monitoring” is supported by 14 companies;

- “TPC command” is supported by 4 companies;

- “Activation/deactivation information of sDCI1 monitoring ” is supported by 3 companies and is related to RAN2 discussion, if any.

- “Base MCS”, “PMI”, “DMRS CS”, “RA/RV”, “SRS/CSI request”, “legacy PDSCH region” is supported by 1 company for each of them.

Proposed agreements

- An sPDSCH/sPUSCH is scheduled by a UE-specific sDCI1

  - Legacy DCI content is the starting point for sDCI1

  - Reduce payload size of sDCI1

    - Increase the granularity of resource block assignment by P times, e.g, P=2 or 3

    - Jointly indicate some of the information

  - Align the payload size for DL sDCI1 and UL sDCI1 for sPDSCH/sPUSCH scheduling

- sDCI1 for sPUSCH scheduling at least include the following information

  - HARQ process number

  - Redundancy version

  - UL DMRS position and presence(if supported) indication for 2-symbol based sPUSCH

  - UL DMRS comb, if supported for 2-symbol based sPUSCH

  - Flag to differentiate sDCI format for sPUSCH and sDCI format for sPDSCH

  - Carrier Indicator

  - Resource block assignment

  - TPC command for sPUSCH, if supported

  - Cyclic shift for DMRS

  - UL index or DAI at least for frame structure type 2

  - CSI request

  - SRS request

  - Resource allocation type

  - Modulation and coding scheme

  - NDI

  - Precoding information and number of layers if TM2 is configured

Note: Each information above is not necessary to be an individual field in sDCI1.

- sDCI1 for sPDSCH scheduling at least include the following information

  - Occupied sPDCCH resource indication

  - DL DMRS position and presence(if supported) indication for 2-symbol based sPDSCH

  - HARQ-ACK resource indication for sPUCCH, if supported

  - Flag to differentiate sDCI format for sPUSCH and sDCI format for sPDSCH

  - Carrier indicator

  - Resource allocation header

  - Resource block assignment

  - TPC command for sPUCCH, if supported

  - DAI at least for frame structure type 2

  - HARQ process number

  - MIMO related information if TM3/4/6/8/9/10 is configured

  - SRS request

  - Modulation and coding scheme

  - NDI

  - Redundancy version

Note: Each information above is not necessary to be an individual field in sDCI1.

- sDCI1 scheduling a single sPUSCH/sPDSCH is the baseline.

  - sDCI1 scheduling multiple sPUSCH(s) can be considered;

    - Multiple subframe scheduling for eLAA is the starting point

- eNB configures one of the sTTI scheduling methods to a UE by RRC signaling:

  - Single level scheduling: UE monitors sDCI1 in every sTTI.

  - Two-level scheduling: UE monitors sDCI1 in every sTTI and sDCI2 in legacy PDCCH region.

- sDCI2, if supported, at least include the following information

  - Aggregation level and/or candidates of sDCI1;

  - PRB set to sDCI1 monitoring;

  - FFS: Activation/deactivation information of sDCI1 monitoring and TPC command

- sDCI2, if supported, is transmitted on demand, while UE blindly detects sDCI2 in PDCCH in every non-DRX subframes.

- Send an LS to RAN2 asking if there is any conclusion on sTTI-specific DRX procedure.

# Appendix