3GPP TSG RAN WG1 Meeting #88	R1-1703348

Athens, Greece, 13th - 17th February 2017

Agenda Item:	8.1.2.5

Source:	Huawei, HiSilicon

Title:	Discussion on system-level calibration for NR-MIMO

Document for:	Discussion and decision

Introduction

In the previous meetings, NR MIMO calibration was proposed and in total three phased approach was agreed as below.

Phase 1: Calibration can be used to check the channel model and the basic beamforming behavior, e.g., by looking at the SNR/SINR distribution (aim to finish it in RAN1#86bis)

Phase 2: Start discussion on whether and how to establish the baseline.  Further discuss simulation assumptions for Phase 2 and Phase 3. Calibration can be used to check the link/system level performances, e.g., by looking at the BLER and spectrum efficiency (aim to finish it in RAN1#87)

Phase 3: Calibration can be used to check the UE movement/rotation/blockage (aim to finish it after RAN1#87)

In RAN1 NR AH meeting, the summary of email discussion [87-30] on NR-MIMO calibration was agreed in [1], where further clarification on phase2 calibration assumptions was also included. Therefore, in this contribution, we provide all sets of system level calibration results for phase2 with additional assumptions, together with detailed evaluation parameters.

System level calibration results

As mentioned above, in phase2 calibration more performance metrics are defined to facilitate the comparison of implementation and understanding among companies. To be specific, the new metrics agreed are listed in the following.

- The CDF of spectral efficiency is adopted as the metric of system level calibration. This spectral efficiency here equals that, taking into the overhead of RS, data throughput with beamforming is divided by its associated system bandwidth. - The outage is defined as the percentage of users that has zero throughput.

In the following, system level calibration results are attached for urban macro, indoor hotspot and dense urban scenarios.

Figure 1: System level calibration results of UMa

From Figure1 it is shown that the outage of urban macro at 30 GHz is about 21%.

Figure 2: System level calibration results of InH

Figure 3: System level calibration results of dense urban (macro layer)

Figure 4: System level calibration results of dense urban (micro layer)

Conclusion

In this contribution, we provided system level calibration results of NR MIMO phase2 with detailed evaluation parameters attached in the appendix. All proponents are encouraged to take these results and assumptions into account in the calibration campaign.

- References

[1] R1-1701517, “Summary of email discussion [87-30] on NR-MIMO calibration,” RAN1 NR AH, ZTE, ZTE Microelectronics, Spokane, USA, Jan. 16 - 20, 2017.

Appendix A

| Parameter | Urban Macro |
| --- | --- |
| Layout | Single layer Macro layer: Hex Grid |
| ISD | 500m |
| Carrier Frequency | 30GHz |
| System bandwidth and carrier spacing | 40 MHz(60 kHz/RE) |
| Channel model | 5GCM UMa |
| BS Tx power | 43dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,2,2) (dH,dV) = (0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0)λ a single TXRU is mapped per panel per polarization |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree 38.802: UE orientation for mobile device (Ω0,0, Θ0,0)=(U(0,360), 90) The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| BS antenna pattern |  |
| UE antenna pattern | Follow TR 38.802 |
| BS antenna height | 25m |
| UE antenna height | Follow TR36.873 |
| BS antenna element gain | 8dBi |
| UE antenna element gain | 5dBi |
| UE receiver noise figure | 10dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| O2I penetration loss | 50% low loss and 50% high loss |
| Analog beam set | Direction of BS beams cover [-60, 60] degrees in azimuth domain and [90, 160] degrees in zenith domain. -- Azimuth angle [-5*pi/16 -3*pi/16 -pi/16 pi/16 3*pi/16 5*pi/16] -- Zenith angle [5*pi/8 7*pi/8] Direction of UE beams cover [-90, 90] degrees in azimuth domain and [0, 180] degrees in zenith domain. -- Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8] -- Zenith angle [pi/4 3*pi/4] |
| UE attachment | Based on post-beamforming RSRP |
| Handover margin | 0dB |

| Parameter | Indoor hotspot |
| --- | --- |
| Layout | Single layer Indoor floor:(12BSs per 120m * 50m) TRP number:12 TR 38.900 |
| ISD | 20m |
| Carrier Frequency | 30GHz |
| System bandwidth and carrier spacing | 40 MHz (60 kHz/RE) |
| Channel model | 5GCM office |
| BS Tx power | 23dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,1,1). (dV,dH) = (0.5, 0.5)λ Boresight direction is perpendicular to the ceiling. Antenna model is taken from alternative 3 in R1-165850 (90 degree HPBW in Azimuth and zenith) a single TXRU is mapped per panel per polarization |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree 38.802: UE orientation for mobile device (Ω0,0, Θ0,0)=(U(0,360), 90) The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| BS antenna pattern |  |
| UE antenna pattern | Follow TR 38.802 |
| BS antenna height | 3m |
| UE antenna height | 1.5m |
| BS antenna element gain | 5dBi |
| UE antenna element gain | 5dBi |
| UE receiver noise figure | 10dB |
| UE distribution | 100% Indoor, 3km/h, 10 users per BS uniform dropping for indoor with minimum distance (2D) of 0 m |
| Analog beam set | Direction of BS beams cover [0, 180] degrees in azimuth domain and [0, 180] degrees in zenith domain. -Beam directions for TRP: -- Azimuth angle [pi/16 3*pi/16 5*pi/16 7*pi/16 9*pi/16 11*pi/16 13*pi/16 15*pi/16] -- Zenith angle [pi/8 3*pi/8 5*pi/8 7*pi/8] Direction of UE beams cover [-90, 90] degrees in azimuth domain and [0, 180] degrees in zenith domain. -- Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8] -- Zenith angle [pi/4 3*pi/4] |
| Handover margin | 0dB |

| Parameter | Dense Urban Macro layer |
| --- | --- |
| Layout | Single layer Macro layer: Hex Grid |
| ISD | 200m |
| Carrier Frequency | 4GHz |
| System bandwidth and carrier spacing | 20 MHz(15 kHz/RE) |
| Channel model | 3D UMa |
| BS Tx power | 44dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.8)λ Consider the following a TXRU to antenna elements mapping as examples 4GHz: the same as TR36.897 |
| UE Configuration | (M,N,P,Mg,Ng) = (1,1,2,1,1) Notes: the polarization angles are 0 and 90 |
| BS antenna pattern |  |
| BS antenna height | 25m |
| UE antenna height | Follow TR36.873 |
| UE antenna element gain | 0dBi |
| Noise figure for BS | 5dB |
| UE receiver noise figure | 9dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| O2I penetration loss | Follow TR 36.873 |
| Analog beam set | Direction of BS beams cover [-60, 60] degrees in azimuth domain and [90, 160] degrees in zenith domain. -- Zenith angle [9*pi/16 11*pi/16 13*pi/16] |
| Handover margin | 0dB |

| Parameter | Dense Urban Micro layer |
| --- | --- |
| Layout | Two layer Macro layer: Hex. Grid Micro layer: Random drop (All micro BSs are all outdoor, according to Figure A.2.1-3 and option1 in Table A.2.1-9 in TR 38.802) - 3 micro BSs per macro BS |
| ISD | Macro: 200m |
| Carrier Frequency | Micro:30GHz |
| System bandwidth and carrier spacing | Micro:40 MHz(60 kHz/RE) |
| Channel model | 5GCM UMi |
| BS Tx power | 33dBm |
| BS Antenna Configuration | Micro: (M,N,P,Mg,Ng) = (4,8,2,2,2) (dH,dV) = (0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0)λ |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern |  |
| UE antenna pattern (30GHz) |  |
| BS antenna height | 10m |
| UE antenna height | Follow TR36.873 |
| BS antenna element gain | 5dBi |
| UE antenna element gain | 4GHz:0, 30GHz:5dBi |
| UE receiver noise figure | 10dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| O2I penetration loss | 50% low loss and 50% high loss |
| Analog beam set | Direction of BS beams cover [-60, 60] degrees in azimuth domain and [35, 135] degrees in zenith domain. - Beam directions for TRP: -- Azimuth angle [-5*pi/16 -3*pi/16 -pi/16 pi/16 3*pi/16 5*pi/16] -- Zenith angle [3*pi/8 5*pi/8] Direction of UE beams cover [-90, 90] degrees in azimuth domain and [0, 180] degrees in zenith domain. -- Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8] -- Zenith angle [pi/4 3*pi/4] |
| UE attachment | Based on post-beamforming RSRP |
| Handover margin | 0dB |