3GPP TSG RAN WG1 Meeting #88bis	R1-1704321

Spokane, USA 3rd - 7th April 2017

Source:	Ericsson

Title:	Investigation of Dual-RM Performance

Agenda Item:	8.1.4.2.2

Document for:	Discussion and Decision

# Introduction

In RAN1#87, Polar codes were agreed to be the channel coding scheme regarding uplink and downlink control information for eMBB except for very small block size ‎[1]. For very small block size 12≤K≤22 a variation of (N=32, K=11)-Reed-Muller (RM) block codes, aka dual-RM code, is used in LTE ‎[2], by segmenting the code into two equal parts.

In this contribution, we compare the performance of three coding candidates, dual-RM code ‎[2] (using ML or FHT decoder), CA-Polar ‎[4] and PC-Polar [3] codes (using SCL decoder).  The LTE-RM performance for  is studied in the companion contribution [5].

# Dual-RM code

Channel coding of control information bits for more than 11 bits is done using dual-RM code ‎[2]. Using dual-RM scheme, the information bits O0,O1,…, OK-1   , where 12≤K≤22, are segmented into two parts, namely

O0, O1, …, OK/2-1

and

OK/2, OK/2+1, …, OK-1

The encode codewords are then linear combinations of the information sequences and the basis sequence of the LTE-RM code, denoted by :

and

The output bit sequence q0,q1,…, qK-1 to be transmitted over channel is obtained by the concatenation and circular repetition of the bit sequences  and  shown above.

In our simulation study, the received codeword is decoded using ML decoding or its low-complexity realization: FHT decoding algorithm.

# Performance comparison

The performance of three coding candidates are compared:

- dual-RM (using ML or FHT decoder);

- PC-Polar ‎[3] codes (using SCL decoder with list size L=8);

- CA-Polar ‎[4] codes (using SCL decoder with list size L=8);

Further simulation setup is summarized in Table 1. Also note that codeword length is chosen as N =48 as used in LTE PUCCH format 3 ‎[2]. For the CA-Polar code, a CRC outer code is concatenated with the original Polar code to check if any of the candidate paths in the list is correctly decoded.

In the following simulations, it is assumed that the information bits at the input to the encoder do not contains CRC bits. Thus, 3 CRC bits are added to the end of K information bits to be encoded using CA-Polar code. The PC-Polar decoder uses no CRC bits and just selects the list with the best path metric.

However, it is known that for the studied sizes of K>11 bits, the information bits at the input to the encoder is expected to contains CRC bits, for example, 8 CRC bits as in LTE UCI. In this case, CA-Polar can achieve better performance by using a CRC polynomial of (8+3)=11 bits, rather than assuming two independent, concatenated CRC vectors of length 8 and 3, respectively. Note, however, the 8 CRC bits carried with information bits are treated the same as normal information bits by dual-RM and PC-Polar codes, thus not affecting their performance.

The simulation results are shown in Figure 1 – Figure 3.

Table 1 – Simulation setup for various block codes

| Channel/Modulation | AWGN/QPSK |  |  |
| --- | --- | --- | --- |
| Code length N | 48 |  |  |
| Information length K | 12~22 |  |  |
| Targeted BLERs | 0.1, 0.01, 0.001 |  |  |
| Coding scheme | LTE-RM ‎[2] | PC-Polar ‎[3] | CA-Polar ‎[4] |
| Decoding algorithm | ML (or FHT) | PC-SCL List = 8 | SCL List = 8 |

From the simulation results, we have the following observations:

- Dual-RM code exhibit inferior performance as compared to CA-Polar and PC-Polar codes.

- CA-Polar code outperforms PC-Polar codes for 12≤K≤22.

It needs to be further studied whether CA-Polar or extended LTE-RM code should be used to cover 12≤K≤14, as LTE-RM has been proposed to cover up to K=14 bits.

Considering the comparison between the coding candidates, we propose

- Adopt CA-Polar codes for short control information bits of NR, at least when  15≤K≤22. FFS whether CA-Polar or extended LTE-RM code is used to cover 12≤K≤14.

Figure 1 – Required SNR vs. K for BLER = 0.1

Figure 2 – Required SNR vs. K for BLER = 0.01

Figure 3 – Required SNR vs. K for BLER = 0.001

# Conclusions

We have studied the performance of three coding candidates: dual RM, CA-Polar and PC-Polar codes for coding of information bits of size 12≤K≤22. Based on our simulation results, we have the following observation and proposal.

- Dual-RM code exhibit inferior performance as compared to CA-Polar and PC-Polar codes.

- CA-Polar code outperforms PC-Polar codes for 12≤K≤22.

- Adopt CA-Polar codes for short control information bits of NR, at least when  15≤K≤22. FFS whether CA-Polar or extended LTE-RM code is used to cover 12≤K≤14.

# References

Chairman’s notes RAN1#87

3GPP TS 36.212, Multiplexing and channel coding, V12.0.0, December 2013

R1-164040, On latency and complexity, Huawei, HiSilicon, RAN1#85, Nanjing, China, May 2016

R1-1701630, Design of CRC-assisted Polar code, Ericsson, RAN#188, Athens, Greece, February 2017

R1-1704320, “Block Codes for Very Short Control Information,” Ericsson.