3GPP TSG RAN WG1 Meeting #88bis                                     R1-1705027

Spokane, USA 3rd - 7th April 2017

Agenda item:	7.2.8.4

Source: 	Qualcomm Incorporated

Title: 	Handover results for aerial vehicles

Document for:     Discussion/Decision

# Introduction

Cellular connectivity will be key for coordinated operation and control of Unmanned Aerial Vehicles, more commonly referred to as drones, enabling a growing set of use cases within and beyond the operator’s visual line of sight. In this contribution, we present early results on mobility performance for considerations in aerial vehicles study.

# Mobility Analysis

Mobility simulations are performed to analyze handover performance and to compare handover and link reliability between ground and airborne drone UEs.

These simulations follow the downlink and uplink simulation results presented in [3], with the addition of UE mobility being simulated. Each UE modeled starts at a randomly selected location in the network, and an angular bearing angle is selected randomly and uniformly. The UE then moves at the assigned speed in a straight line for the duration of the simulation.

Table 1 describes the parameters used to model the handover algorithm for these simulations.

Table 1 Parameters used to model the handover algorithm

| Parameter | Value | Unit | Description |
| --- | --- | --- | --- |
| TimeToTrigger | 160 | ms | Time to trigger measurement report |
| A3Offset | 2.0 | dB | Offset between signal strength of serving and neighboring cells |
| MeasurementInterval | 40 | ms | Physical layer measurement interval |
| SamplesToAverage | 5 |  | Number of physical layer measurements averaged |
| L3RRMCoefficient | 1 |  | Filtering coefficient for layer 3 measurements |
| RMLInterval | 2 | s | Radio link monitoring interval |
| T310 | 1000 | ms | Timer to trigger radio link failure recovery |
| HOPreparationDelay | 50 | ms | Handover preparation delay |
| HOExecutionDelay | 40 | ms | Handover execution delay |
| RSRPError | 1.22 | dB | Standard deviation of RSRP measurement error |

The mobility simulations populate the network with 7 UEs per cell, on average. Simulations are performed with all UEs at ground, 50 m and 120 m altitude. Each of these are simulated with UE speeds of 0, 1.5, 3.0, 30, 60 and 120 km/h to gain an understanding of the handover performance dependency on speed.

## 2.1. Analysis of results

In this section, we evaluate the results of the handover (HO) simulations using six performance dimensions tabulated in Table 2, all are averages.

Table 2 HO simulations using six performance dimensions

| KPI | Unit | Figure | Description |
| --- | --- | --- | --- |
| Handover rate | HO/UE/sec | Figure 3 | Number of HOs over time |
| Radio Link Failure (RLF) rate | RLF/UE/sec | Figure 4 | Number of RLFs over time |
| Time in handoff | % | Figure 5 | Fraction of time a UE is in a HO procedure |
| Time in Qout | % | Figure 6 | Fraction of time a UE is in Qout state |
| Handover interruptions | % | Figure 7 | Percentage of HO procedures that start but fail |
| Re-establishment interruptions | % | Figure 8 | Percentage of HO re-establishment procedures that start but fail |

Figure 3 shows the average number of handovers per UE per second.

As shown, drone UEs experience a smaller handover rate than ground UEs. Even though the amount of strong interference for drone UEs increases due to better radio propagation at higher elevations, the radio conditions are also more stable, which leads to reduced handover rate.

The handover rate for ground UEs is impacted by significant variation of received signal strength due to shadowing, which has standard deviation of 10 dB and decorrelation distance of 30 m.

At UE speed of 0 km/h, drone UEs experience slightly higher handover rates, due to static radio conditions for all UEs and larger impact of measurement errors on drone UEs.

Figure 3 Handover rates

Observation 1: Drone UEs experience less handovers than ground based UEs.

In LTE, radio link failure events are expected to be infrequent as shown in Figure 4. A relatively stable radio environment for drone UEs translates to virtually no RLF events during the system simulation run, even for drone UE speed of 120 km/h.

Observation 2: Drone UEs experience less radio link failures than ground based UEs.

Figure 4 Radio link failure rates

In Figure 5 and Figure 6 we show the impact of handover of quality of service, measured by fraction of time communications with the UE is interrupted due to either handover events or radio link failures, respectively.

As seen in Figure 5, drone UEs experience significantly less time in handover than ground UEs for all speeds other than 0 km/h. The handover rates are UE speed of 0 km/h are very small and this speed is not of primary interest for drone UEs.

Figure 6 shows service interruption when signal quality drops below the Qout threshold, defined in 3GPP specs as SINR value of -8 dB. As shown, for all UE speeds whether they are on the ground or are drones at 50m and 120 m, these events are relatively infrequent.

Observation 3: Drone UEs experience less time in Qout and HO than ground based UEs.

Figure 5 Fraction of time in handover

Figure 6 Fraction of time in Qout state

In Figure 7 and Figure 8, we show another view on the service interruption statistics.

Figure 7 illustrates the fraction of time service interruption was caused by handover.

Figure 8 illustrates the fraction of time service interruption was caused by radio link failure event and therefore triggered radio link reestablishment procedure.

Observation 4: Drone UEs are less likely to suffer from handover interruption and re-establishment interruption than ground based UEs.

As shown, there is a noticeable fraction of interruptions caused by radio link failure events only at high speeds.

Figure 7 Likelihood of handover interruption

Figure 8 Likelihood of link re-establishment interruption

# Conclusion

In this contribution, we made the following observations on mobility performance of drone UEs:

Observation 1: Drone UEs experience less handovers than ground based UEs.

Observation 2: Drone UEs experience less radio link failures than ground based UEs.

Observation 3: Drone UEs experience less time in Qout and HO than ground based UEs.

Observation 4: Drone UEs are less likely to suffer from handover interruption and re-establishment interruption than ground based UEs.

# References

- RAN SID on Enhanced Support for Aerial Vehicles in RP-170779

- TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification

- R1-1705026, System level performance and interference mitigation techniques for aerial vehicles, Qualcomm Incorporated