3GPP TSG-RAN WG1 #88bis	R1-1705185

Spokane, USA, 3th – 7th April, 2017

Source:	Ericsson

Title:	NPRACH reliability for NB-IoT

Agenda Item:	7.2.7.3

Document for:	Discussion and Decision

# Introduction

In RAN#75, a working agreement on the work item on further enhanced NB-IoT was made. The objective of the work item is to introduce further support of enhanced features for NB-IoT, supporting standalone, guard-band, and in-band operation modes. One objective is to study NPRACH reliability and range enhancements listed below.

NPRACH reliability and range enhancements

- If found necessary, reduce false alarm probability for NPRACH detection due to inter-cell interference on NPRACH [RAN1, RAN2, RAN4]

- If found necessary, introduce at least additional cyclic prefixes for NPRACH to support cell radius of at least 100 km [RAN1, RAN2, RAN4]

In this contribution, we discuss how the existing NPRACH design can be used to achieve low false alarm probability for NPRACH detection in the presence of inter-cell interference on NPRACH.

# Discussion

As illustrated in Figure 1, NPRACH is a single subcarrier waveform of 3.75kHz with frequency hopping [1]. The basic unit of NPRACH preamble is symbol group that consists of a CP and 5 repetitive OFDM symbols. Four symbol groups are treated as a repetition unit. The repetition unit can be repeated 2j times for coverage extension, where j = 0, ..., 7. Accordingly, the length L of a preamble equals 4 × 2j symbol groups. Every preamble sequence takes a constant value “1”, i.e., u[m] = 1, m = 0, ..., L − 1, where u[m] denotes the symbol value in symbol group m.

A special hopping pattern is used in NPRACH. The hopping pattern of NPRACH consists of both inner layer fixed size hopping and outer layer pseudo-random hopping. Outer layer pseudo-random hopping is applied between groups of 4 symbol groups. Inner layer fixed size hopping is applied within every 4 symbol groups: A single-subcarrier hopping is used between the first and the second and between the third and the fourth symbol groups, while a six-subcarrier hopping is used between the second and the third symbol groups.

Figure 1: An illustration of NPRACH design

False alarm occurs if the post-processing statistic of a potential NPRACH preamble exceeds some pre-determined threshold when the NPRACH preamble is not present. With a false alarm, the consequence is that the eNB would respond a vain message (MSG2) that corresponds to the non-existent NPRACH preamble transmission. When setting the false alarm requirement, RAN4 assumes that the input to the receiver is a Gaussian noise signal.

NPRACH false alarms result in vain transmissions of MSG2 in random access procedure.

NPRACH false alarm requirement in RAN4 considers thermal noise.

From a network perspective, false alarm may occur because of intra-cell interference and/or inter-cell interference besides thermal noise. The interference may be due to NPUSCH and/or NPRACH. Since NPUSCH transmissions do not use frequency hopping, the intra-cell interference and/or inter-cell interference from NPUSCH to NPRACH is mitigated thanks to the special hopping pattern used in NPRACH.

The intra-cell interference and/or inter-cell interference from NPUSCH to NPRACH is mitigated thanks to the special hopping pattern used in NPRACH.

Next we consider the false alarm events due to intra-cell interference from NPRACH to NPRACH. Note that keeping the symbol value unchanged in a symbol group prevents inter-carrier interference and thus maintains the orthogonality of preamble transmissions on different subcarriers. For example, consider Figure 2, where the CP is 266.7 us, i.e., the same length as an OFDM symbol in NPRACH symbol group. On tone f1, the received symbols are (X1, X1, X1, X1, X1, X1). On tone f2, the received symbols are (X2, X2, X2, X2, X2, X2). Take the received symbols on tone f1 for example. The symbols received on tone f2 appearing on tone f1 equals 0 since

ICIf2→f1=0t1x2ej2πf2te-j2πf1tdt+ t1Tx2ej2πf2te-j2πf1tdt=0

If two UEs in the same cell choose the same starting subcarrier of NPRACH, the two transmissions will use the same frequency hopping signature, resulting in a collision. The collision can be resolved in the random access procedure. It should be noted that the collision is not a false alarm.

Figure 2: CP + 5 same OFDM symbols in NPRACH symbol group preserves the orthogonality of preamble transmissions on different subcarriers

With the symbol value unchanged in the same symbol group, existing NPRACH design preserves the orthogonality of preamble transmissions on different subcarriers, reducing false alarm probability due to intra-cell NPRACH interference.

We now consider the false alarm events due to inter-cell interference from NPRACH to NPRACH. If the same time-frequency resources of neighbouring cells are used for NPRACH, false alarm may arise due to NPRACH transmission in the neighbour cell. The pseudo-random hopping in NRACH is a function of cell ID and can help mitigate the inter-cell NPRACH interference. Since the pseudo-random hopping is applied between repetitions, the larger the number of repetitions, the better the interference randomization effect. For users in extreme coverage with 164 dB MCL, the NPRACH configuration is associated with a high number of repetitions (e.g., 32 or higher).

For short NPRACH with 1 or 2 repetitions configured for users is good coverage, there is no or limited inter-cell interference randomization. In this case, the network may configure non-overlapping (or at most partially overlapping) time-frequency resources for NPRACH across neighbouring cells. This is possible due to the flexible configuration capability of NPRACH configuration design with the following parameters.

- Periodicity (3 bits)

  - {40, 80, 160, 240, 320, 640, 1280, 2560} ms

- Starting time of period (3 bits)

  - {8, 16, 32, 64, 128, 256, 512, 1024} ms

- Number of repetitions (3 bits)

  - {1, 2, 4, 8, 16, 32, 64, 128}

- Frequency

  - Frequency location in subcarrier offset (3 bits)

    - {0, 12, 24, 36, 2, 18, 34}

  - Number of subcarriers (2 bits)

    - {12, 24, 36, 48}

In the Appendix, we give an example NPRACH configuration where NPRACH resources for different coverage classes in the cell are FDM. The same FDM may be used in the neighboring cells. To reduce inter-cell NPRACH interference, neighboring cells may use different starting times of periods to configure non-overlapping (or at most partially overlapping) NPRACH resources.

The pseudo-random hopping in NRACH is a function of cell ID and can help mitigate the inter-cell NPRACH interference, reducing false alarm probability due to inter-cell NPRACH interference especially when the number of NPRACH repetitions is large.

When the number of NPRACH repetitions is small, the false alarm probability due to inter-cell NPRACH interference may be reduced by configuring non-overlapping time-frequency NPRACH resources across neighbouring cells.

# Conclusions

In this contribution, we analyse the design of the existing NPRACH and made the following observations:

Observation 1	NPRACH false alarms result in vain transmissions of MSG2 in random access procedure.

Observation 2	NPRACH false alarm requirement in RAN4 considers thermal noise.

Observation 3	The intra-cell interference and/or inter-cell interference from NPUSCH to NPRACH is mitigated thanks to the special hopping pattern used in NPRACH.

Observation 4	With the symbol value unchanged in the same symbol group, existing NPRACH design preserves the orthogonality of preamble transmissions on different subcarriers, reducing false alarm probability due to intra-cell NPRACH interference.

Observation 5	The pseudo-random hopping in NRACH is a function of cell ID and can help mitigate the inter-cell NPRACH interference, reducing false alarm probability due to inter-cell NPRACH interference especially when the number of NPRACH repetitions is large.

Observation 6	When the number of NPRACH repetitions is small, the false alarm probability due to inter-cell NPRACH interference may be reduced by configuring non-overlapping time-frequency NPRACH resources across neighbouring cells.

# References

TS 36.211, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation.”

TS 36.104, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception.”

TR 45.820, “Technical Specification Group GSM/EDGE Radio Access Network; Cellular System; Support for Ultra Low Complexity and Low Throughput Internet of Things; (Release 13),”

TR 37.868, “RAN Improvements for Machine-type Communications.”

# Appendix

We use the most challenging scenario and traffic model in [3] to give an example NPRACH configuration. The coupling statistics and mean traffic arrival rate are summarized in Table 1. Based on the traffic model, the number of random access opportunities per second at 1% collision rate are computed in Table 1. Note that there are two definitions of collision defined in TR 37.868.

Table 1: Coupling loss statistics and traffic arrival rates [3]: Scenario 2 with 0.75 inter-site correlation coefficient; 52547 devices per sector

| Coupling loss | <144 dB | [144 dB, 154 dB] | >154 dB |
| --- | --- | --- | --- |
| Percentage | 88.42% | 9.01% | 2.57% |
| MAR mean arrival rate (/s) | 6.0229 | 0.6137 | 0.1751 |
| # RA opportunities per second at 1% collision rate (based on device’s perspective) | 599.27 | 61.06 | 17.48 |
| # RA opportunities per second at 1% collision rate (based on network’s perspective) | 130.93 | 13.34 | 3.81 |
| Note: See Annex B of TR 37.868 collision definition based on device’s perspective; See Section 6.3 of TR 37.868 for collision definition based on network’s perspective. |  |  |  |

The collision rate defined based on device’s perspective is much more demanding than the one based on network’s perspective. Figure 3 gives an example NPRACH configuration where NPRACH resources for different coverage classes in the cell are FDM to meet the 1% collision rate based on device’s perspective. The same FDM may be used in the neighboring cells. To reduce inter-cell NPRACH interference, neighboring cells may use different starting times of periods to configure non-overlapping (or at most partially overlapping) NPRACH resources.

Figure 3: An example NPRACH configuration