3GPP TSG RAN WG1 Meeting #88bis	R1-1705340

# Spokane, USA 3rd - 7th April 2017

Agenda item:	8.1.2.1.8

Source: 	Samsung

Title: 	Continuous precoding of NR DMRS in time domain

Document for:	Discussion and Decision

Introduction

In 3GPP RAN1#88 [3], following agreements on DMRS are agreed:

Agreements:

…

- Support DMRS bundling in time domain

  - At least time domain bundling with slot aggregation of DL-only slots is supported

  - DMRS pattern within the first slot is not impacted by the time domain DMRS bundling

  - FFS: Consider further overhead reduction of DMRS in case of bundling in time domain

…

In this contribution, we discuss continuous precoding of NR DMRS in time domain.

Discussions on precoding granularity in time domain

In LTE, DMRS precoding granularity in time domain is fixed to one subframe regardless of used precoding in data transmission. In Figure 1, DMRS channel estimation procedure in LTE is depicted as shown below:

Figure 1 Hybrid CSI-RS transmission for multiplexed services in NR

When reciprocity between uplink and downlink channel is not supported, eNB can realize preferred transmission layer and precoding by using UE’s periodic and/or triggered feedback. For periodic CSI reporting, it should be noted that information on precoding in eNB side does not change between CSI reporting instances. Considering such aspects, it is quite natural to use identical precoding between the CSI reporting instances for a given UE if UE is scheduled. As well as periodic CSI reporting case, UE which uses aperiodic CSI reporting experiences similar situation. Since information on precoding in eNB side only changes when eNB triggers, using identical precoding between UE reports would be natural. The situation is similar for channel reciprocity case since eNB can acquire channel information only when UE transmits SRS to eNB.

In order to support joint estimation of DMRS, specification support can be considered in NR. One possible alternative would be semi-static configuration of measurement window for DMRS channel estimation. In Figure 2, the example of joint DMRS estimation by using measurement window is provided.

Figure 2 DMRS joint estimation based on the semi-static measurement window

As shown in Figure 2, eNB can configure semi-static measurement window of DMRS to UE by using higher layer. Based on the configuration, UE can jointly estimate DMRS and enhance estimation performance. In this scheme, the performance can be degraded due to the restriction.

Another method to support joint estimation of DMRS would be dynamic indication of DMRS precoding set to UE. Figure 3 shows the exemplary operation of dynamic DMRS precoding set indication.

Figure 3 DMRS joint estimation based on the dynamic indication of precoding set

Observations:

- For both channel reciprocity and non-reciprocity case, CSI acquisition may not be done in every slots.

  - For periodic CSI reporting, acquisition of CSI is limited to UE reporting instance.

  - For aperiodic CSI reporting, CSI can be provided only when eNB triggered CSI.

  - For channel reciprocity case, channel information can be done only when UE transmits SRS.

- Considering availability of CSI, support of continuous precoding on DMRS in time domain can bring channel estimation accuracy enhancement.

Proposals:

- Support continuous precoding on DMRS in time domain.

- Consider following alternatives to support continuous precoding on DMRS in time domain:

  - Semi-static measurement window.

  - Dynamic indication of precoding sets.

Evaluation results

will be updated..

Conclusions

In this contribution, continuous precoding of NR DMRS in time domain is discussed. Based on the discussions, the following observations and proposals are provided:

Observations:

- For both channel reciprocity and non-reciprocity case, CSI acquisition may not be done in every slots.

  - For periodic CSI reporting, acquisition of CSI is limited to UE reporting instance.

  - For aperiodic CSI reporting, CSI can be provided only when eNB triggered CSI.

  - For channel reciprocity case, channel information can be done only when UE transmits SRS.

- Considering availability of CSI, support of continuous precoding on DMRS in time domain can bring channel estimation accuracy enhancement.

Proposals:

- Support continuous precoding on DMRS in time domain.

- Consider following alternatives to support continuous precoding on DMRS in time domain:

  - Semi-static measurement window.

  - Dynamic indication of precoding sets.

- References

- RP-170847, “New WID on New Radio Access Technology”, NTT DOCOMO

- 3GPP TSG RAN1#88, Chairman’s Note.