3GPP TSG RAN WG1 Meeting #88bis                            		   R1-1705511

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	8.1.2.4.1

Source:	InterDigital Communications

Title:	On CSI-RS Design for CSI Acquisition

Document for:	Discussion

# Introduction

In RAN #75, the NR WI has been approved and following agreements related to CSI-RS captured in TR 38.912 [1] should be specified based on the NR WID [2].

NR supports periodic, aperiodic, and semi-persistent transmissions of CSI-RS as follows.

-	Semi-persistent transmission

-	Activation(s)/de-activation(s) of CSI-RS resource is triggered dynamically

-	Preconfigured CSI-RS resources can be activated or de-activated

-	Periodic transmission

-	Periodic transmission can be configured by higher layer signaling

-	Periodic CSI-RS transmissions are semi-statically configured/re-configured.

NR CSI-RS pattern with at least the following properties is supported.

-	CSI-RS mapped in one or multiple symbols

The following configurations of NR CSI-RS are supported.

-	UE-specific configuration to support

-	Wideband CSI-RS, i.e. from UE perspective, the full bandwidth the UE is configured to operate with

-	Partial-band CSI-RS, i.e. from UE perspective, part of the bandwidth the UE is configured to operate with

…

The RE pattern for an X-port CSI-RS resource spans N ≥ 1 OFDM symbols in the same slot and is comprised of one or multiple component CSI-RS RE patterns where a component CSI-RS RE pattern is defined within a single PRB as Y adjacent REs in the frequency domain and Z adjacent REs in the time domain. Note that, depending on the density reduction approach, the Y REs of a component CSI-RS RE pattern may be non-adjacent in the frequency domain. The multiple component CSI-RS RE patterns can be extended across the frequency domain within the configured CSI-RS bandwidth. At least, the numbers of OFDM symbols for a CSI-RS resource, N = {1, 2, 4}, are supported. The N OFDM symbols can be adjacent/non-adjacent. OFDM symbol(s) can be configured to contain CSI-RS only. Density per port in terms of RE per port per PRB is configurable.

In this contribution, we discuss on the details of CSI-RS design for CSI acquisition

# Discussions

The configurable CSI-RS density has been agreed as the RS overhead gets increased for a higher number of CSI-RS ports with a UE-specific CSI-RS configuration. Therefore, reducing the RS overhead while increasing CSI-RS pattern reuse to support a larger number of UEs may help the system performance, although it slightly degrades the CSI reporting accuracy.

In LTE, the CSI-RS density of 1RE/port/RB has been supported as a maximum and the density can be further reduced by nulling of CSI-RS in a PRB level. It has been observed that there is a marginal performance difference between 1RE/port/RB and a higher CSI-RS density. This is mainly due to implicit CSI feedback has been used in LTE. Since two CSI feedback types are supported and Type II CSI requires high resolution, a higher density than 1RE/port/RB may be considered and carefully investigated at least for Type II CSI feedback in terms of system performance and RS overhead.

As the PRB-level nulling of CSI-RS is evaluated and supported in LTE already, at least PRB-level nulling of CSI-RS to support the configurable CSI-RS density should be supported. The configurable CSI-RS density in time domain can be further studied.

Proposal-1: support at least PRB-level nulling of CSI-RS to configure the density

Multiple OFDM symbols can be used to multiplex antenna ports in a slot, however, a code spreading has to be used across antenna ports located in a different OFDM symbol to support a full power transmission. For example, orthogonal cover code (OCC) in time has been used for the CSI-RS antenna ports so that all antenna ports are multiplexed in FDM and CDM within an OFDM symbol, thus allowing a full power transmission.

For better coverage of CSI-RS, the full power transmission should be considered as one of design criteria for CSI-RS pattern design.

Proposal-2: CSI-RS design should support full power transmission always

A CSI-RS resource pool is used in NR and a UE may be configured and/or indicated to measure one or more CSI-RS resources in the CSI-RS resource pool. Since the configuration and indication is a UE-specific, the active aperiodic and/or semi-persistent NZP-CSI-RS set may be dynamically changed from a slot to another. A UE scheduled for NR-PDSCH reception with and without aperiodic and/or semi-persistent NZP-CSI-RS may need to know the active set of aperiodic and/or semi-persistent NZP-CSI-RS configurations in the slot for rate-matching purpose.

Since the active set of aperiodic and/or semi-persistent NZP-CSI-RS configurations are common for all UEs scheduled in the slot, it can be dynamically indicated in a common DCI.

Proposal-3: active set of aperiodic and/or semi-persistent ZP-CSI-RS is indicated in each slot

# Summary

This contribution discussed CSI-RS design considerations in NR. Based on the discussions, we propose followings:

Proposal-1: support at least PRB-level nulling of CSI-RS to configure the density

Proposal-2: CSI-RS design should support full power transmission always

Proposal-3: active set of aperiodic and/or semi-persistent ZP-CSI-RS is indicated in each slot

# References

TR 38.912, “Study on New Radio (NR) Access Technology”

RP-170847, “New WID on New Radio Access Technology”, NTT DOCOMO Inc.