3GPP TSG RAN WG1 Meeting #88bis                                     R1-1705728

Spokane, USA 3rd - 7th April 2017

Source:	NTT DOCOMO

Title:	Phase-2 LLS MIMO Calibration Results

Agenda Item:	8.1.2.6

Document for: 	Discussion

Introduction

At the RAN1 86, 87, NR AH meetings, issues about NR MIMO link-level calibration were discussed [1]-[4]. General guideline for calibrations was agreed as follows.

Agreements: RAN1 #86 - Phase 1: Calibration can be used to check the channel model and the basic beamforming behavior, e.g., by looking at the SNR/SINR distribution (aim to finish it in RAN1#86bis) - Phase 2: Start discussion on whether and how to establish the baseline. Further discuss simulation assumptions for Phase 2 and Phase 3. Calibration can be used to check the link/system level performances, e.g., by looking at the BLER and spectrum efficiency (aim to finish it in RAN1#87) - Phase 3: Calibration can be used to check the UE movement/rotation/blockage (aim to finish it after RAN1#87) NR AH meeting - For updating phase-1 results: Until Feb. 6 - Companies are encouraged to update results (particularly for the micro layer of dense urban scenario) - If further clarification of simulation assumptions is needed, it can also be discussed under this email thread - For phase-2 results: Until Mar. 31 - For phase-3 results: Until Apr. 28

In this contribution, we present our link-level simulation results of NR MIMO phase-2 calibration, according to the agreed evaluation assumptions.

Simulation Assumptions

The basic simulation assumptions of Phase 2 calibration are summarized as follows for reference.

Channel Model

According to the email discussion, both CDL-A and CDL-B with delay spread=100ns and UE speed=3km/h need to be calibrated. For 30GHz, the subcarrier spacing is 60kHz for 80MHz bandwidth; for 4GHz, the subcarrier spacing is 15kHz for 20MHz bandwidth.

The angles associated with the TRP, i.e., AoD, ZoD, are uniformly distributed within [-60°, 60°] degrees in azimuth and [90°, 135°] degrees in zenith, while the angles associated with the UE, i.e., AoA, ZoA, are uniformly distributed within [-180°, 180°] degrees in azimuth and [45°, 90°] in zenith. These are implemented by making uniform-distribution of desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly.

TXRU mapping to antenna elements & TXRU mapping weights

For 30GHz, a single TXRU per panel per polarization is used and 2D TXRU virtualization weights for each panel is obtained by Kronecker product between vertical and horizontal DFT.

For 4GHz, TXRU virtualization is applied only in vertical dimension (i.e. 1D virtualization) using DFT vectors.

Beam sweeping approach

For beam selection, the best analog beam pair is selected among the limited set of DFT beams based on the criteria of maximizing receive power.

The beam direction set covers [-90°, 90°] degrees in azimuth domain and [0°, 180°] degrees in zenith domain.

- Beam directions@30GHz:

  - Azimuth angle [-7*pi/16 -5*pi/16 -3*pi/16 -pi/16 pi/16 3*pi/16 5*pi/16 7*pi/16]

  - Zenith angle [pi/8  3*pi/8  5*pi/8  7*pi/8]

- Beam directions@4GHz:

  - Zenith angle [pi/8  3*pi/8  5*pi/8  7*pi/8]

- Beam directions for UE @30GHz：

  - Azimuth angle [-3*pi/8  -pi/8  pi/8  3*pi/8];

  - Zenith angle [pi/4  3*pi/4];

Digital beamforming and RS pattern

SVD-based (i.e., H=USV) digital beamforming is agreed to be involved in calibration. In our implementation, the U and V vector from SVD decomposition are used for UE and TRP side respectively, to map data from one layer to multiple TXRUs and vice versa. The SVD-based beamforming is based on ideal channel estimation at the first symbol of each PRB and then used for the rest symbols of the PRB.

The CRS and CSI-RS are not taken into consideration since ideal CSI feedback is assumed. But channel estimation for demodulation is assumed to be based on one port DMRS (12 REs).

Simulation Results

According to the agreed simulation assumptions, some comparison results are provided for analysis.

30GHz case

The calibration results for 30GHz have been presented in Figure 1.

Figure 1 BLER at 30 GHZ

4GHz case

The calibration results for 4GHz have been presented in Figure 2.

Figure 2 BLER at 4 GHZ

Conclusion

This contribution presents link level simulation results for NR-MIMO Phase 2 calibration.

- References

- 3GPP, R1-168487, “WF on NR MIMO calibration”, ZTE Corporation, Oct. 2016

- 3GPP, R1-1608661, “Summary of [86-20] email discussion on MIMO calibration for NR”, ZTE, Oct. 2016

- 3GPP, R1-1613741, “Way forward on Phase 1 and Phase 2 MIMO calibration for NR”, ZTE, Nov. 2016

- 3GPP, R1-1701516, “WF on MIMO Calibration”, NTT DOCOMO, Jan. 2017

- Appendix

Table 1: Other Simulation Assumptions for Link-level MIMO Calibration

| Parameter | Values |
| --- | --- |
| Data allocation | 8 RBs - Note: Error free PDCCH decoding is assumed. - First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| BS antenna configurations | 30GHz: (M,N,P,Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ 4GHz: (M,N,P,Mg,Ng) = (4,4,2,1,1). (dV,dH) = (0.5, 0.5)λ |
| BS antenna pattern | HPBW=65°, Max Antenna Gain = 8dBi, SLAV=Am=30 polarization angles are -45° and +45° |
| UE antenna configurations | 30GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 Notes: introduce (Ωmg,ng, Θmg,ng) for orientation of the panel (mg, ng), 0≤mg<Mg, 0≤ng<Ng, where the orientation of the first panel (Ω0,0, Θ0,0) is the same as UE orientation, Ωmg,ng is the array bearing angle and Θmg,ng is the array downtilt angle defined in [TR 36.873]. The panel with the best receive SNR is chosen for output metric. i.e. no combining is done between panels. 4GHz: (M,N,P,Mg,Ng) = (1,1,2,1,1) |
| BS array orientation | azimuth 0 degree; mechanic downtilt 0 degree; |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UE antenna pattern | 30 GHZ: HPBW=90°, Max Antenna Gain = 5dBi, SLAV=Am=25 4 GHZ: Omnidirectional polarization angles are 0° and +90° |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD |
| MIMO mode | SU-MIMO with rank=1 |
| UE receiver type | MMSE-IRC |
| MCS | LTE MCS = 1 |
| Metrics | BLER w/ beamforming as a function of transmission SNR ranging from -20 dB to 0dB |