3GPP TSG RAN WG1 Meeting #88bis							R1-1705749

Spokane, USA 3rd - 7th April 2017

Source:	NTT DOCOMO, INC.

Title:	Views on HARQ enhancements for NR

Agenda Item:	8.1.3.3.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1 January AH NR meeting and RAN1 #88 meeting, following agreements and working assumption were achieved [1] – [2]:

Agreements: - HARQ-ACK feedback with one bit per TB is supported. Agreements: - RAN1 will down select following options to utilize HARQ-ACK feedback with more than one bit per TB until the next meeting - Option 1: CB-group based re-transmission (Samsung) - Option 2: Decoder state information feedback (Nokia) - Option 3: CB-level outer erasure code (Qualcomm) - Option 4: Any combination of Option 1-3 - Other options are not precluded - Note that if RAN1 will not reach consensus in the next meeting, no support of utilization HARQ-ACK feedback with more than one bit per TB in Rel-15 Working assumption: - CBG-based transmission with single/multi-bit HARQ-ACK feedback is supported in Rel-15, which shall have the following characteristics: - Only allow CBG based (re)-transmission for the same TB of a HARQ process - CBG can include all CB of a TB regardless of the size of the TB – In the such case, UE reports single HARQ ACK bits for the TB - CBG can include one CB - CBG granularity is configurable

In this contribution, we discuss enhanced HARQ mechanisms for NR.

- HARQ enhancements

  - More than one HARQ-ACK bits for one transport block

There are some simulation results demonstrating throughput performance improvement by supporting more than one HARQ-ACK bits for one transport block [3] – [4]. In addition, RAN1 agreed to support puncturing of eMBB downlink data by a short URLLC downlink data, which is likely to result in decoding failure of a subset of CBs for a given TB. Considering these situations, support of more than one HARQ-ACK bits for one transport block can be a good approach to enable efficient HARQ re-transmission of eMBB for NR. Besides, based on the working assumption, HARQ-ACK feedback with one bit per TB can also be supported by configuring the CBG granularity is all CBs of a TB regardless of the size of the TB. Therefore, the working assumption should be confirmed.

Proposal 1:

- Following working assumption should be confirmed:

  - CBG-based transmission with single/multi-bit HARQ-ACK feedback is supported in Rel-15, which shall have the following characteristics:

    - Only allow CBG based (re)-transmission for the same TB of a HARQ process

    - CBG can include all CB of a TB regardless of the size of the TB – In the such case, UE reports single HARQ ACK bits for the TB

    - CBG can include one CB

    - CBG granularity is configurable

To realize the CBG-based (re)-transmission, UE needs to know which CBGs are re-transmitted. One simple way is to indicate which CBGs are re-transmitted by the scheduling DCI. For re-transmission, following two options are considered:

Option 2.1-1: Only error CBGs are re-transmitted over a slot

The HARQ process ID of the TB will not be released as long as all the CBGs are correctly decoded. For the re-transmission, only CBGs that need to be re-transmitted are transmitted as shown in Fig. 1(a). The gNB can schedule PRBs or adjust MCS such that the amount of scheduled resources is appropriate for the re-transmission of error CBGs. It is also possible to schedule large amount of PRBs even if the amount of re-transmitted data is small, so that different RVs or repetitions can be applied in the single re-transmission of error CBGs.

Option 2.1-2: In addition to error CBGs, new CBGs are transmitted over a slot

For the re-transmission of some CBGs, new data CBGs are multiplexed in the same transmission. For this, TB-based HARQ assignment may not work well. New HARQ mechanisms would be necessary for this option.

(a) Option 2.1-1				(b) Option 2.1-2

Fig. 1	CB/CBG-based re-transmission over a slot.

According to our understanding, Option 2.1-2 is difficult to be realized based on the current working assumption as it only allows CBG based (re)-transmission for the same TB of a HARQ process. Besides, it requires new HARQ mechanisms including MAC and PHY layers. Hence, it would not be feasible to be specified at the first release of NR.

  - HARQ process sharing between slot-level data and mini-slot level data

Re-transmission of some CBG(s) may not require huge amount of resources compared to initial transmission of a TB. Therefore, instead of re-transmitting CBG(s) using one whole slot, it should also be possible to schedule re-transmission using a mini-slot. This enables further efficient and flexible radio resource utilization and reduces the latency. For the mini-slot-level re-transmission, following two options can be considered.

Option 2.2-1: TB scheduled over a slot is re-transmitted over a mini-slot (not CBG-based re-transmission)

For this, it is expected that the re-transmission may require larger amount of frequency resources compared to the initial transmission using slot, while reducing the amount of time-domain resources. Therefore, remaining time duration can be used for transmitting other data.

Option 2.2-2: CBGs in a TB scheduled over a slot for initial transmission is re-transmitted over a mini-slot

For this, gNB can schedule re-transmission of CBG(s) over a mini-slot with a certain amount of frequency resources. If higher coding gain is preferable, larger amount of frequency resources should be scheduled for the re-transmission. If efficient resource utilization is preferable, appropriate amount of resources can be scheduled. In either option, duration of the re-transmission is much shorter than the initial transmission. Therefore, remaining time duration can be used for transmitting other data.

In case of option 2.2-2, further flexible HARQ operation can be realized. Example 1 of Fig. 3 shows the case where initial TB transmission includes 4 CBGs, and re-transmission for two or them is done by using mini-slots. For the remaining time resource, additional two CBGs are scheduled by using mini-slots. Comparing the example 1 of Fig. 3 with option 2.1-2, same resource utilization is realized. In other words, equivalent to option 2.1-2 can be realized by option 2.2-2 without changing basic HARQ mechanisms. Other ways of resource utilization are also possible as seen in examples 2 and 3 of Fig. 3.

For this operation, it is necessary to enable HARQ process ID sharing between slot-level scheduling and mini-slot-level scheduling. In other words, based on this function, it is possible to realize equivalent operation to option 2.1-2 in section 2.1 (see example 1 of Fig. 3).

(a) Option 2.2-1				(b) Option 2.2-2

Fig. 2	CB/CBG-based re-transmission over a mini-slot.

Fig. 3	Mini-slot based re-transmission for slot based initial transmission.

Proposal 2:

- To support CB/CBG-based HARQ re-transmission.

  - In case of TB-level scheduling, CB/CBG-index needs to be included in the scheduling DCI.

- HARQ process sharing between slot-level and mini-slot-level scheduling should be also considered.

  - This enables TB/CB/CBG-based HARQ re-transmission by using mini-slot.

RAN1 so far agreed to support slot-aggregation. Whether one TB spans multiple slots or each TB spans each slot of aggregated slots is not decided yet. In either case, the above mechanism should be scaled for slot-aggregation; part of data scheduled over aggregated slots should be able to be re-transmitted using one slot.

Fig. 4	Slot based re-transmission for aggregated slots based initial transmission.

Proposal 3:

- Mini-slot based TB/CB/CBG-level HARQ re-transmission for slot-level initial transmission should be scaled to slot based TB/CB/CBG-level HARQ re-transmission for aggregated-slots-level initial transmission.

- Conclusion

In this contribution, we discussed HARQ enhancements for NR and proposed the following.

Proposal 1:

- Following working assumption should be confirmed:

  - CBG-based transmission with single/multi-bit HARQ-ACK feedback is supported in Rel-15, which shall have the following characteristics:

    - Only allow CBG based (re)-transmission for the same TB of a HARQ process

    - CBG can include all CB of a TB regardless of the size of the TB – In the such case, UE reports single HARQ ACK bits for the TB

    - CBG can include one CB

    - CBG granularity is configurable

Proposal 2:

- To support CB/CBG-based HARQ re-transmission.

  - In case of TB-level scheduling, CB/CBG-index needs to be included in the scheduling DCI.

- HARQ process sharing between slot-level and mini-slot-level scheduling should be also considered.

  - This enables TB/CB/CBG-based HARQ re-transmission by using mini-slot.

Proposal 3:

- Mini-slot based TB/CB/CBG-level HARQ re-transmission for slot-level initial transmission should be scaled to slot based TB/CB/CBG-level HARQ re-transmission for aggregated-slots-level initial transmission.

References

- 3GPP RAN1 January NR AH Chairman’s note.

- 3GPP RAN1 #88 meeting, Chairman’s note.

- R1-1700165, ‘DL/UL data scheduling and HARQ procedure,’ MediaTek.

- R1-1700960, ‘Evaluation results of partial retransmission for eMBB,’ Samsung