3GPP TSG-RAN WG1 RAN1#88b	                                  	                  R1-1706771

Spokane, Washington, April 3-7 2017

Source:	Mitsubishi Electric, []

Title:	Summary of AI 8.1.2.1.5 UL transmit diversity

Agenda Item:	8.1.2.1.5

Document for: 	Discussion/Decision

# Introduction

At the RAN1 NR AH meeting in January 2017 [1] the following was decided:

- Support at least the following UL transmission schemes for data in NR

  - Scheme A: Codebook based UL transmission

      - […]

  - Scheme B: Non-codebook based UL transmission

    - […]

  - Diversity-based transmission schemes

    - FFS: Whether the scheme has specification impact or not

  - FFS: Merging of the schemes

Moreover, in RAN1#88 [2] it was more specifically decided that:

- For DFT-S-OFDM, following schemes can be candidates for UL diversity schemes for UL data: CDD, precoder cycling, antenna port switching, SFBC, and STBC.

  - RAN1 should down select them in WI phase.

  - FFS the corresponding spec impact (if any)

At the RAN1 NR AH#1 meeting the following was decided [1] concerning the waveform and transmission scheme for long duration PUCCH:

- For PUCCH in long-duration, DFT-s-OFDM waveform is supported.

- For PUCCH in long-duration, transmit antenna diversity is supported.

Based on the submitted contributions in RAN1#88bis for this agenda item, the following candidates are identified as UL transmit diversity candidates.

# UL transmit diversity candidates for DFTsOFDM

For DFTsOFDM, the PAPR property imposes some further precautions than for CP-OFDM. The following candidates were proposed:

- Low PAPR Alamouti-based transmit diversity: Mitsubishi Electric (PAPR preserving SFBC for both PUSCH and PUCCH), Qualcomm (split symbol STBC for PUCCH), Huawei (STBC), Apple (STBC)

- Short delay CDD: Qualcomm (for PUSCH only)

- Time domain precoder beam cycling: CATT

- Transparent TxD: Ericsson

PUCCH-related transmit diversity will be addressed in 8.1.3.

# UL transmit diversity for CP-OFDM

The following non-mutually excluding candidate families were proposed:

- Transparent DM-RS based: Intel, ZTE, Ericsson

- Non-transparent DM-RS based: ZTE

- Beam and/or precoder cycling: LGE, Samsung, Nokia

- SFBC: Huawei, Nokia

- Semi open-loop MIMO for at least uplink control channel: ZTE

Several companies are targeting a unified framework with DL transmission scheme 2 (Intel, Samsung, Qualcomm)

# General comments

Some UL transmit diversity proposals for PUCCH are also discussed in agenda items (8.1.3.2.1 Structure of PUCCH in short-duration and 8.1.3.2.2 Structure of PUCCH in long-duration).

UL-transmit diversity to mitigate large scale fading (e.g. blocking) also needs to be addressed in NR. Some mechanism to compensate blockage, e.g., gripping, can be considered in addition to diversity scheme(s) regardless of waveform properties, e.g., panel selection (NTT DOCOMO).

- For > 6 GHz and for low cost single transceiver terminals < 6 GHz panel/antenna switched diversity can be considered: Sony

- For < 6 GHz also dominant eigenmode transmission can be considered. :Sony

Ericsson points out that the majority of evaluations so far are at link level using models reflecting fixed ray angle of arrival and angle spread statistics.  This implies fixed antenna correlation. Since TxD schemes can be sensitive to antenna correlation, models taking into account variable AoA and angle spread should be used to provide better measures of relative performance.

Ericsson suggests that system level simulations with spatial channel models be taken into account to assess performance of TxD gains. The availability of other diversity sources (e.g. frequency hopping, HARQ, number of eNB antennas) should also be considered.

# Current list of related WFs

R1-170XXX 		WF on transmit diversity for DFTsOFDM, Mitsubishi Electric, Qualcomm, Interdigital, Intel, Ericsson, CATT, NTT DOCOMO

# Annex: Proposals summary

## ZTE

Proposal 1：

- DMRS based open-loop MIMO scheme(s) should be supported.

- Both transparent and non-transparent ways should be studied and evaluated for supporting diversity schemes.

Proposal 2:

- Further study Semi open-loop MIMO for at least uplink control channel

## CATT

Proposal: consider time domain precoder/beam cycling, including per DFT-S-OFDM and per UL slot cycling, as an option for UL diversity scheme and further evaluate the performance, overhead, PAPR etc.

## Intel

Proposal 1. Adopt transparent DMRS based uplink diversity-based transmission scheme at least for rank-1 and rank-2 scheme where transparent DMRS transmission scheme refers the scheme utilizes identical precoder for DRMS and data.

Proposal 2. UE can be configured to transmit precoded SRS.

- Configuration includes number of ports,  transmission scheme (reciprocity based or implementation specific)

- For precoded SRS transmission, higher number of MIMO layer transmission precoder contains lower number of MIMO layer transmission precoder

- UE uses same precoder for data transmission

Proposal 3. The beam cycling should be taken into account for multi-beam operation, and both FDMed/SDMed based and TDMed based beam cycling should be studied.

## Mitsubishi Electric

Proposal 1: Support Alamouti-based transmit diversity schemes for DFTsOFDM.

Proposal 2: Consider PAPR preserving SC-SFBC as transmit diversity scheme for DFTsOFDM.

## LGE

Proposal 1: Beam/precoder cycling transmission technique should be considered in NR for UL.

Proposal 2: The set of cycling beams/precoders can be narrowed down based on long-term channel information (e.g., W1), panel calibration capability at UE side, or gNB indication (e.g., codebook subset restricton).

Proposal 3: The set of cycling beams/precoders can be implicitly indicated to UE by using precoded SRS with bundling.

Proposal 4. Rank 1 transparent scheme can be used for both DL and UL.

## Huawei

Proposal 1:  NR should support STBC for uplink data channel at least for DFT-S-OFDM.

Proposal 2: NR should support SFBC for uplink data channel at least for CP-OFDM.

## Sony

Proposal 1: NR need to define scheduling approach, SRS resource allocation and SRS transmission strategies in order to enable UL/DL diversity for Massive MIMO below 6 GHz.

Proposal 2: NR need to support both high performance and low cost UEs. Therefore both dominant eigenmode and switched diversity based transmissions shall be concidered.

Proposal 3: SRS transmission capability from all UE antennas needs to be concidered by NR. It is essential from both diversity and higher rank operation perspectives and impacts on both UL and DL performance.

## Samsung

Proposals:

- Common framework between transmission scheme 2 in DL and diversity based transmission scheme in UL should be supported.

- For UL diversity based transmission scheme, precoder cycling should be supported.

## Qualcomm

Proposal 1: For PUCCH, adopt STBC as described in [R1-1705578] for the transmission diversity scheme.

- Virtual split of symbols can be considered if symbol pairing is not available.

Proposal 2: For PUSCH with DFT-S-OFDM, consider small delay CDD as UL transmission diversity scheme.

- FFS: whether to adopt proposal 1 for PUSCH with small RB allocation.

Proposal 3: For PUSCH with CP-OFDM waveform, adopt the same transmit diversity scheme as PDSCH

## Ericsson

Proposals:

- At least transparent TxD supported in NR

  - TxD designs should take into account other diversity sources such as frequency hopping

- UEs with Multi-Tx antenna capability can be configured for single antenna port transmission

- Single antenna transmission is a fall back mode for UL SU-MIMO

- Provide robust spatially multiplexed PUSCH using single codeword transmission

## Nokia

Proposal 1:	UL diversity based transmission schemes shall be considered for both DFT-S-OFDM and CP-OFDM.

Proposal 2:	For DFT-S-OFDM, evaluate small-delay CDD, and further study other diversity-based transmission schemes.

Proposal 3:	At least for rank-1, consider to support SFBC and precoder cycling scheme for CP-OFDM diversity based transmission.

# References

### Chairman’s notes, RAN1 NR AH 01, Spokane, Washington, January 2017.

### Chairman’s notes, RAN1 #88, Athens, Greece, February 2017.

### R1-1704394,Diversity based uplink transmission schemes, ZTE, ZTE Microelectronics

### R1-1704548, On UL diversity transmission schemes, CATT

### R1-1704721, On Diversity based UL Transmission	Intel Corporation

### R1-1704814, UL diversity transmission for DFTsOFDM	Mitsubishi Electric

### R1-1704875, Discussion on diversity transmission for UL	LG Electronics

### R1-1705078, Diversity-based transmission for UL	Huawei, HiSilicon

### R1-1705214, Consideration on both UL and DL Diversity for NR Massive MIMO	Sony

### R1-1705337, Diversity transmission for UL	Samsung

### R1-1705578, On UL diversity transmission scheme	Qualcomm Incorporated

### R1-170588, Robust Transmission Scheme for PUSCH	Ericsson

### R1-1705956, UL diversity schemes	Nokia, Alcatel-Lucent Shanghai Bell.