3GPP TSG RAN WG1 Meeting #89						          R1-1707156

Hangzhou, China, 15th - 19th May 2017

Title: 			Further evaluation results on PDCCH diversity scheme

Source:  		ZTE

Agenda Item: 	7.1.3.1.1

Document for: 	Discussion and Decision

Introduction

RAN1 has discussed the transmission diversity schemes and related number of RS ports. Basically, those transmit diversity scheme will be used in case when the channel is not well measured.

The evaluation is yet to confirm some cases for lower aggregation levels. This contribution further compared transmit diversity schemes 2port SFBC and 1 port precoder cycling. 5MHz bandwidth with smaller aggregation levels and 20MHz bandwidth larger aggregation level are all under evaluation. Simulation results are provided based on the agreed parameters in previous meetings. We also assumed bundle sizes from WA to facilitate the comparison.

TX diversity and bundle sizes

A WA about PDCCH transmit scheme were reached at RAN1 #88b meeting:

Working assumption:

One-port transmit diversity scheme with REG bundling per CCE is used for NR-PDCCH

  - FFS the bundling size

  - FFS: REG bundling is also for localized mapping in time and/or frequency-domain

Companies are encouraged to provide evaluation results for 10 MHz and 20 MHz for larger aggregation levels and 5 MHz and 10 MHz for smaller aggregation levels.

The above bundle sizes are applicable for distributed mapping and important for channel estimation in frequency domain. For localized mapping it might not be applicable since it can utilize good channel information. And, the transmit diversity scheme may not be used for localized mapping.

Proposal 1: The bundle size in frequency domain should be always enabled for distributed mapping in NR PDCCH.

Figure 1 Distributed CCE to REG mapping with bundle size

For distributed mapping, channel estimation may have different performance with different bundle size. The bundle size will affect the frequency diversity gain. The bundle sizes have not been down-selected from REG size 2 and 3. The selection of bundle sizes is discussed in our companion contribution [2]. We give the comparison between SFBC and 1 port precoder cycling and assuming bundle size is 3 in frequency domain. Besides, it is shown that in our companion contribution [2] that bundling size 3 has better performance in most  of the cases.

We also assume 6 REGs per CCE are used as working assumption from the last meeting. At least in the comparing cases, we did not see the impact of the CCE definition on CORESET size, CCE aggregation levels, data resource allocation granularity, etc. Thus, the CCE and bundle size definition is kept constant for different aggregation levels.

In our simulation, we use distributed mapping and frequency first mapping. For a certain AL, all bundled REG sets are uniformly distributed in the whole CORESET bandwidth as shown in Figure 1. Other simulation parameters are shown in Appendix.

The PDCCH occupy only one OFDM symbol as the agreed evaluation assumption. This will be the case more essential for the CORESET transmitting the common control. To let the two schemes compared under different bundle size and different aggregation level, we evaluate AL=1/AL=2 with around 5MHz bandwidth and AL=4/AL=8 with around 20MHz bandwidth.

Evaluation of transmit diversity

The simulation results for small aggregation level (AL = 1,2) are shown in Figure 2, where 5MHz CORESET bandwidth is assumed. It can be observed that precoder cycling outperform SFBC in most of cases except for large payload (60bits) with AL = 1. This is because SFBC can achieve un-coded diversity gain which benefits the performance of higher code rate. Precoder cycling can only have coded diversity gain, which results in greater performance advantage in the case of lower code rate.

Figure 2 Small Aggregation Level

The simulation results for large aggregation level are shown in Figure 3, where 20MHz CORESET bandwidth is assumed. It can be observed that precoder cycling outperforms SFBC in all of cases thanks to the coded diversity gain achieved by precoder cycling in the case of lower coding rate.

Figure 3 Large Aggregation Level

According to above simulation results, it can be concluded that precoder cycling outperforms SFBC in most of cases. Moreover, large code rate (e.g. AL = 1 with larger payload size), where SFBC can shows its advantages, is usually used in the scenario with good channel information. At this time, the gain of SFBC over precoder cycling seems not so critical. So, it should be confirmed that single port diversity scheme is adopted as transmit diversity scheme of NR-PDCCH as more transparent TX scheme for PDCCH.

Proposal 2: Transmit diversity scheme with single-port should be accepted for different aggregation levels regardless of system bandwidths.

Conclusion

In this contribution, we present further simulation on TX diversity scheme and analyzed the results. Based on the analysis given above, we have the following proposals:

Proposal 1: The bundle size in frequency domain should be always enabled for distributed mapping in NR PDCCH.

Proposal 2: Transmit diversity scheme with single-port should be accepted for different aggregation levels regardless of system bandwidths.

References

- 3GPP RAN1 meeting #88bis Chairman's Notes.

- R1-1707157, “Analysis and simulation of REG bundling”, ZTE

- Appendix

Table 1 Simulation Assumptions

| Item | Value |
| --- | --- |
| Channel | TDL-A, DS = 300ns, Speed = 3kmph |
| Subcarrier spacing | 15kHz |
| RS overhead | 33% |
| Antenna | 2*2 |
| Numerology | 1)BW = 20Mhz, SCS = 15kHz, 100 RB 2)BW = 5Mhz, SCS = 15kHz, 25 RB |
| Aggregation level (AL) | 1)4,8 for 20MHz bandwidth 2)1,2 for 5MHz bandwidth |
| CCE | 6 REG/CCE |
| PDCCH payload | 20/60+16(CRC) |
| Number of OFDM symbols for PDCCH | 1 |
| coding | TBCC |
| Modulation | QPSK |
| Frequency | 4GHz |
| Receiver | MMSE |