3GPP TSG RAN WG1 Meeting #89	R1-1708490

Hangzhou, China 15th - 19th May 2017

Source:	NTT DOCOMO

Title:	Performance evaluation of sequence design for Polar codes

Agenda Item:	7.1.4.2.1.2

Document for: 	Discussion

Introduction

In RAN1#88bis meeting, it was concluded that the following issues should be considered when designing Polar codes [1].

Conclusions: At least the factors relevant for evaluating the sequences are: - Performance - Information granularity - Compatibility with rate matching - Complexity - Latency

The structure of Polar codes is shown in Fig. 1. In [2], we have shown more details on our single nested sequence design. In this contribution, we will show the BLER performance with different sequences together with different rate matching schemes.

Fig. 1 The structure of Polar codes for NR

The following notations are used in this contribution.

K:		information block size

M:		codelength

N:		mother codelength,

K1:		number of input bits of the basic encoder

ui:		input bits of the basic encoder

xi:		coded bits of the basic encoder

QN:		ordered sequence of codelength N

Performance evaluation

In [3], it is shown that the performance of sequence is related to the rate matching schemes. In this Section, we show the BLER performance of our sequence and compare it with other sequence design. The evaluation assumptions are shown in Table 2 and the evaluation results are shown in Section 2.1 and 2.2, respectively.

Table 1 Evaluation assumptions

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK |
| Sequence design | Design 1 [4], Design 2 [3], Proposed |
| Code rate (K+16)/M | 1/6, 1/3, 1/2, 2/3 |
| Rate matching | Bit-reversal, Block |
| Decoding algorithm | List SC with list size of 8 |
| Info. block length (bits w/o CRC) | 48, 64 |

  - Sequence with bit-reversal rate matching

Fig. 2 The performance of the different sequence designs with bit-reversal rate matching

Fig. 3 The performance of the proposed sequence design with bit-reversal rate matching and K = 48

Fig. 4 The performance of the proposed sequence design with bit-reversal rate matching and K = 64

It can be seen from the evaluation, Polar codes with the proposed sequence have competitive performance with sequence design 1.

Observation 1: Polar codes with the proposed sequence and bit-reversal rate matching have competitive performance with other sequence designs.

  - Sequence with block rate matching

Fig. 5 The performance of the different sequence designs with block rate matching

Fig. 6 The performance of the proposed sequence design with bit-reversal rate matching and K = 48

Fig. 7 The performance of the proposed sequence design with bit-reversal rate matching and K = 64

It can be seen from the evaluation, Polar codes with the proposed sequence have competitive performance with sequence design 2.

Observation 2: Polar codes with the proposed sequence and block rate matching have competitive performance with other sequence designs.

Summary

In this contribution, we evaluated different sequence designs together with different rate matching schemes. Based on the evaluation results, we have the following Observations.

Observation 1: Polar codes with the proposed sequence and bit-reversal rate matching have competitive performance with other sequence designs.

Observation 2: Polar codes with the proposed sequence and block rate matching have competitive performance with other sequence designs.

- References

- 3GPP, Chairman's Notes, RAN1 #88bis, Spokane, USA.

- 3GPP, R1-1708489, “Sequence design of Polar codes”, NTT DOCOMO, RAN1 #89, Hangzhou, China.

- 3GPP, R1-1706674, “FRANK polar construction: nested extension design of polar codes based on mutual information”, Qualcomm Incorporated, RAN1 #88bis, Spokane, USA.

- 3GPP, R1-167209, “Polar code design and rate matching”, Huawei, HiSilicon, RAN1 #86, Gothenburg, Sweden.