3GPP TSG RAN WG1 Meeting #89		      R1-1708951

Hangzhou, P.R. China 15th - 19th May 2017

Agenda Item:	7.1.1.1.1

Source: 	CATR

Title: 	Consideration on details of NR-PSS and NR-SSS

Document for:	Discussion and Decision

Introduction

In last RAN1 NR Ad hoc meeting, some agreements have been made on the PSS/SSS sequence bandwidth and sequence design:

- Working assumption: Number of PSS sequences: 3

  - PSS sequence details:

    - Frequency domain-based pure BPSK M sequence (fixing the time/freq. offset ambiguity)

      - 1 polynomial: Decimal 145 (i.e. g(x) = x7 + x4 + 1)

      - In freq. domain 3 cyclic shifts (0, 43, 86) to get the 3 PSS signals

      - Initial poly shift register value: 1110110

    - FFS modified ZC: 2 ZC sequences concatenation or interleaving in time or freq., 4 ZC sequences concatenation in time

Conclusions:

- For NR-SS,

- Following remaining issues need to be finalized in the next meeting

  - Confirmation of WA of NR-PSS

  - NR-SSS M-sequence parameters and scrambling details

  - Mapping from Cell IDs to sequences

  - Maximum number of SS blocks in an SS burst set

  - Mapping of NR-PSS, NR-SSS and NR-PBCH to SS block of SS burst set configurations

- Followings remaining issues need to be finalized by Nov. meeting

  - Remaining details of SS burst set configurations

    - Mapping of SS blocks to bursts

    - Mapping of SS bursts to burst sets

    - Indication of actually used SS blocks

    - Note that all RRC related aspects need to be finalized by Oct. meeting

In this contribution, we firstly give our further evaluation results and analysis considering the comparison of ZC sequence PSS, M sequence PSS and the modified ZC sequence, and then we give our consideration and evaluation results of NR-SSS M sequence parameters and scrambling details.

The NR-PSS sequence

In last meeting, a working assumption is agreed for NR PSS, which is Frequency domain based pure BPSK M sequence is adopted as PSS sequence, in this meeting, it is targeted to confirm the working assumption. we give our initial evaluation of several ZC sequence and M sequence as PSS sequence.

The number of NR-PSS sequence for evaluation is 3, The generation of m sequence follows the working assumption made at last meeting, the root of ZC sequence is 42,44 and 85, respectively. We choose the low frequency band scenario below 6GHz(4GHz), the subcarrier spacing is 15kHz, we give our evaluation assuming relatively larger frequency offset , the detailed simulation assumptions are given in appendix.

As shown in Figure 1, when frequency offset is larger, the joint detection probability of ZC sequence and M sequence are similar. And as shown in Figure 2 and Figure 3, both the residual timing offset and residual frequency offset of ZC sequence and M sequence are also similar.

Figure 1the joint detection probability of M sequence and ZC sequence(larger frequency offset)

Figure 2 the residual timing offset of M sequence and ZC sequence(larger frequency offset)

Figure 3 the residual frequency offset of M sequence and ZC sequence (larger frequency offset)

Observation 1: the performance of M sequence and ZC sequence are similar with relatively larger frequency offset.

The modified ZC sequence

In our evaluation, we give the following options to evaluate, as shown in table 1. we simulate modified ZC sequence of 2 ZC sequences concatenation in time, 2 ZC sequences interleaving in time, 2 ZC sequence concatenation in frequency, 2 ZC sequences interleaving in frequency and 4 ZC sequences concatenation in time

table 1 modified ZC sequence option

|  | - Sequence design |  |  | - Modified ZC sequence |
| --- | --- | --- | --- | --- |
|  | - number | - length | - root |  |
| - Opt1 | - 1 | - 126 | - 61 | - ZC sequence in frequency |
| - Opt2 | - 1 | - 126 | - 29 | - 2 ZC sequences interleaving in frequency |
| - Opt3 | - 1 | - 126 | - (25,29) | - 2 ZC sequences concatenation in frequency |
| - Opt4 | - 1 | - 128 | - 61 | - ZC sequence in time |
| - Opt5 | - 1 | - 128 | - 29 | - 2 ZC sequences interleaving in time |
| - Opt6 | - 1 | - 127 | - (1,30) | - 4 ZC sequences concatenation in time |

Figure 4 gives the Joint detection probability performance, and figure 5 gives the timing residual offset of modified ZC sequence, and from the results, we can see that the ZC sequence in time is better than ZC sequence in frequency.

Figure 4 the joint detection probability of modified ZC sequence

Figure 5 the residual timing offset of modified ZC sequence

Observation 2: the performance of ZC sequence in time is similar with ZC sequence in frequency with interleaving and concatenation.

NR-SSS M-sequence parameters and scrambling

In our evaluation assumptions ,the carrier frequency is lower than 6GHz (4GHz), the subcarrier is 15kHz. we simulation two options of SSS sequence scrambling:

Option 1: Interleaved concatenation between two different cyclic shifts of a length-63 m-sequence, scrambled by the scrambling sequence given by PSS and mapped on consecutive subcarriers

Option 2: Different cyclic shifts of 3 length-127 m-sequences with generator polynomials x7 + x + 1, x7 + x3 + 1 and x7 + x4 + 1, scrambled by the PSS indices and mapped on consecutive subcarriers without special DC tone treatment.

The joint detection probability of 1 TRP and 3 TRPs are given in figure 6 and figure 7, from the results ,we can see that the two options are similar in 1 TRP, and option 1 is better than option 3 in 3 TRPs scenarios.

Figure 6 the joint PSS/SSS detection of two NR-SSS options(1TRP)

Figure 7 the joint PSS/SSS detection of two NR-SSS options(3TRP)

Observation 3: The performance of two NR-SSS design options are similar in 1 TRP, and option 1 is better than option 2 in 3 TRPs scenarios

Conclusion

In this contribution, we we firstly give our further evaluation results and analysis considering the comparison of ZC sequence PSS, M sequence PSS and the modified ZC sequence, and then we give our consideration and evaluation results of NR-SSS M sequence parameters and scrambling details, from the evaluation results, the following observations are given:

Observation 1: the performance of M sequence and ZC sequence are similar with relatively larger frequency offset.

Observation 2: the performance of ZC sequence in time is similar with ZC sequence in frequency with interleaving and concatenation.

Observation 3: The performance of two NR-SSS design options are similar in 1 TRP, and option 1 is better than option 2 in 3 TRPs scenarios

References

[1] Chairman notes of RAN1 88bis meeting, 2017.4

[2]R1-1706156 	NR PSS and SSS design	Intel Corporation

[3]R1-1706187 	Synchronization signals for NR	Huawei, HiSilicon

Appendix simulation assumptions

Table1. Link-level Evaluation Assumptions

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 4GHz |
| Subcarrier spacing | 15kHz/30kHz |
| Channel model | CDL-C with delay scaling values of 100ns |
| UE speed | 3km/h |
| Antenna configuration at the TRP | (1,1,2) with omni-directional antenna element |
| Antenna configuration at the UE | (1,1,2) with omni-directional antenna element |
| Target received baseband SNR | -6dB |
| Frequency Offset | TRP: uniform distribution +/-0.05 ppm UE: uniform distribution +/- 5 ppm |
| - Number of interfering TRPs | - 0 TRP: mandatory - 2 interfering TRPs (1st SIR = 0dB and 2nd SIR = -3dB, where SIR is defined as the ratio of power between a reference cell and interfered cell; The time delays of 2 interfering TRPs are uniformly distributed from -3 to 3 us). |