3GPP TSG-RAN WG1 #90	R1-1713469

21st – 25th August 2017

Prague, Czech Republic

Agenda item:	6.1.4.2.2

Source: 	Qualcomm Incorporated

Title: 	Evaluation of the sequence for Polar codes

Document for:	Discussion/Decision

# Introduction

In RAN1 NR Ad-Hoc #2 meeting [1], the following agreement was made in terms of determining the sequence design for Polar code.

Decision procedure:

Candidate sequences shall have the property of simple nestedness, i.e. one sequence of length N/2 is nested with the sequence of length N

- Presence or absence of any other property (including symmetry, arithmetic describability, down-nestedness (i.e. a sequence of length N/2 is nested within the lower half of the sequence of length N), up-and-down-nestedness (i.e. a sequence of length N/2 is nested in both the upper and lower halves of the sequence of length N)) shall not be used as a decision criterion.

- Performance metric

  - SNR to achieve 10-2 and 10-3 BLER

- Simulation assumptions

  - Evaluate the block error rate (BLER) performance versus SNR

| Channel | AWGN Channel |
| --- | --- |
| Modulation | QPSK |
| Info. Block length (=K bits w/o CRC) | K = 8:1:min⁡(200,Kmax,N), where Kmax,N=5N/6-J+J' K = min⁡(200,Kmax,N):24:Kmax,N, where Kmax,N=5N/6-J+J' excluding any code rates below 1/8 |
| Codeword length (=N) | {64, 128, 256, 512, 1024} |
| Decoding algorithm | List-X with LLR-based min-sum |
| List sizes | 1,2,4,8,16 (pruned to 8 best paths for CRC check) |
| Code construction for evaluation | CA polar |
| Number of (J+J’) bits | 19 bits (0b10100010101101111001 where the last bit is d19) |

PerfThresh_K = 0.1dB for lower range of K, 0.3dB for higher range of K

PerfThresh_L = 0.4dB for L=1, 0.2dB for L=2, 0.1dB otherwise.

PerfThresh = max (PerfThresh_K, PerfThresh_L)

Each company selects a winning sequence by the following algorithm:

  - For sequence A,

    - compare with sequence Bat each simulation case.

      - For each simulation case:

        - if A’s performance is worse than B – PerfThresh, increment FailCount_AB

        - if A’s performance is better than B + PerfThresh, increment WinCount_AB

      - If (FailCount_AB – WinCountAB) / Total number of simulation cases > 2%, increment OverallFail_A

    - compare with sequence Cat each simulation case.

      - For each simulation case:

        - if A’s performance is worse than C – PerfThresh, increment FailCount_AC

        - if A’s performance is better than C + PerfThresh, increment WinCount_AC

      - If (FailCount_AC – WinCountAC) / Total number of simulation cases > 2%, increment OverallFail_A

    - repeat for sequences D…N

  - For sequence B,

    - compare with sequence A at each simulation case

    - etc

  - …

  - For sequence N,

    - compare with sequence A

    - etc

  - Select sequence with smallest OverallFail

If multiple sequences A to M have the same smallest OverallFail,

  - For sequence A,

    - compare with sequence Bat each simulation case.

      - For each simulation case, if A’s performance is better than B + PerfThresh, increment WinCount_AB

    - compare with sequence C at each simulation case.

      - For each simulation case, if A’s performance is better than C + PerfThresh, increment WinCount_AC

    - WinCount A = ∑WinCount_AB…AM

    - repeat for sequences up to M

  - Repeat for sequences B to M.

  - Select sequence with highest WinCount, referred to as sequence W.

  - If any WinCount_xW > WinCount Wx, then sequence(s) x is/are also selected.

If more than 1 sequence is selected by at least one company, then the final sequence will be chosen from the sequences that were selected by at least one company according to the largest support in RAN1#90.

# Simulation results on sequence comparison

According to the agreed decision procedure, there are in total 5710 test cases.  Seven companies submitted their sequences: Ericsson, Huawei, LG, MediaTek, Qualcomm, Samsung, ZTE. Among them, the one submitted by Samsung and Ericsson are designed jointly by Docomo, Ericsson, Intel, Samsung, and Qualcomm.

The exact sequence is presented in the appendix. In Table 2 and Table 1 we summarize the simulation results regarding the win-loss count. The 1e-2 and 1e-3 BLER achieving SNR is obtained by collecting 500 packet errors with at each SNR at 0.1dB step size.

Table 1 Pair-wise win-loss count for sequence comparison

|  | A:B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B | ER | HW | LG | MTK | QC | SS | ZTE |
| A |  |  |  |  |  |  |  |
| ER |  | 1:0 | 60:1 | 80:1 | 18:2 | 24:0 | 67:1 |
| HW | 0:1 |  | 66:2 | 83:1 | 19:3 | 26:1 | 83:2 |
| LG | 1:60 | 2:66 |  | 16:11 | 5:43 | 3:34 | 30:16 |
| MTK | 1:80 | 1:83 | 11:16 |  | 2:46 | 8:42 | 29:24 |
| QC | 2:18 | 3:19 | 43:5 | 46:2 |  | 15:6 | 56:0 |
| SS | 0:24 | 1:26 | 34:3 | 42:8 | 6:15 |  | 52:7 |
| ZTE | 1:67 | 2:83 | 16:30 | 24:29 | 0:56 | 7:52 |  |

From Table 1, it is clear that none of the pair-wise comparison between sequence A and sequence B satisfies the initial disqualifying criteria of

(FailCount_AB – WinCountAB) / Total number of simulation cases > 2%

and thus all the sequence should enter second round of comparison, where total win-count across all sequences should be taken into account as the decision criteria, which is summarized in Table 2.

Table 2 Number of cases Sequence A is better than Sequence B

| B | ER | HW | LG | MTK | QC | SS | ZTE | Total Win count |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |  |  |
| ER |  | 1 | 60 | 80 | 18 | 24 | 67 | 250 |
| HW |  |  | 66 | 83 | 19 | 26 | 83 | 277 |
| LG | 1 | 2 |  | 16 | 5 | 3 | 30 | 57 |
| MTK | 1 | 1 | 11 |  | 2 | 8 | 29 | 52 |
| QC | 2 | 3 | 43 | 46 |  | 15 | 56 | 165 |
| SS |  | 1 | 34 | 42 | 6 |  | 52 | 135 |
| ZTE | 1 | 2 | 16 | 24 |  | 7 |  | 50 |
| Total loss count | 5 | 10 | 230 | 291 | 50 | 83 | 317 |  |

According to the agreed sequence determination criteria below, it is clear from the results in  Table 2 that the sequence submitted from Huawei has the highest WinCount and should be accepted as wining sequence.

    - For sequence A,

      - compare with sequence Bat each simulation case.

        - For each simulation case, if A’s performance is better than B + PerfThresh, increment WinCount_AB

      - compare with sequence C at each simulation case.

        - For each simulation case, if A’s performance is better than C + PerfThresh, increment WinCount_AC

      - WinCount A = ∑WinCount_AB…AM

      - repeat for sequences up to M

    - Repeat for sequences B to M.

    - Select sequence with highest WinCount, referred to as sequence W.

Additionally, according to the agreed criteria below, the sequence from Ericsson is also selected as winning sequence since the WinCount comparison between Ericsson and Huawei is 1:0.

    - If any WinCount_xW > WinCount Wx, then sequence(s) x is/are also selected.

To summarize, following the agreed Polar code sequence decision procedure in the RAN NR Ad-Hoc #2 meeting, the following agreement should be made:

Outcome of the decision process: At least two sequences are selected as winner: Huawei and Ericsson (joint design).

Also note that since the following is agreed in the last meeting, any of the winners that are identified from other companies should also be chosen as the winner sequence.

If more than 1 sequence is selected by at least one company, then the final sequence will be chosen from the sequences that were selected by at least one company according to the largest support in RAN1#90.

Also, since more than one sequences are selected from the procedure, other factors should be considered in the decision process, such as:

- Implementation benefits of the proposed sequence

- Openness to incorporate good properties from other companies' sequence proposal

- Merged design vs. single company design

One potential implementation benefit of the MI-DE based construction is elaborated in [2].

# Conclusion

Proposal 1: In compliance with the decision procedure that is agreed in RAN1 NR Ad-Hoc#2 meeting,  at least two sequences are selected as winner: Huawei and Ericsson (joint design);

- To restate the agreement from RAN1 NR Ad-Hoc#2 meeting, any of the winner sequences that are identified from other companies are also selected as winner.

Proposal 2: Since more than 1 sequence is chosen as the winner sequence, consider the following factors in the final decision process

- Implementation benefits of the proposed sequence

- Openness to incorporate good properties from other companies' sequence proposal

- Merged design vs. single company design

# References

- RAN1 NR Ad-Hoc #2 Chairman’s Notes

- R1-1713468 “Sequence construction of Polar codes for control channel”, Qualcomm Inc.

# Appendix: Sequence submitted by seven companies

Samsung

[1023,1022,1021,1019,1015,1007,1020,991,1018,1017,1014,959,1013,1006,1011,1005,895,990,1003,989,1016,999,987,958,1012,767,957,983,1010,1004,955,1009,894,975,1002,951,893,988,1001,998,511,891,986,943,997,985,766,956,887,982,995,765,927,954,981,1008,879,974,763,953,979,950,892,973,1000,759,949,863,510,890,971,942,996,947,509,889,984,751,941,886,967,994,507,764,831,926,885,939,980,993,878,735,925,762,952,503,883,978,935,877,972,761,923,977,758,948,862,495,875,970,703,757,919,946,861,508,888,969,750,940,871,966,755,945,479,859,506,749,830,911,639,884,505,938,965,734,829,502,747,855,992,924,937,882,963,934,876,733,447,760,827,922,501,847,494,743,933,881,823,499,731,860,702,918,931,493,976,874,815,870,921,383,756,701,858,917,478,727,968,754,638,491,873,964,915,910,857,477,699,799,869,504,828,748,944,909,487,854,936,753,719,475,695,907,867,746,446,637,853,732,500,687,903,826,962,255,635,932,742,745,846,851,825,845,730,822,445,741,471,961,729,821,880,498,843,920,726,930,739,492,631,443,814,700,497,819,872,725,382,916,929,671,813,463,839,698,490,439,718,868,914,723,623,697,381,476,811,489,717,856,908,694,486,798,866,913,379,474,693,752,797,431,906,485,715,636,852,865,807,254,607,686,905,691,253,634,744,795,375,850,473,444,685,902,824,711,483,633,844,496,415,849,740,470,630,901,251,820,791,683,842,728,442,670,629,738,367,575,469,351,462,818,841,441,669,438,899,724,737,622,679,812,627,838,817,467,783,247,621,380,461,667,722,488,810,837,960,696,437,239,716,459,378,721,319,619,809,692,430,606,663,484,796,835,714,455,806,928,435,377,605,429,690,713,615,374,794,252,805,472,414,223,684,710,373,689,603,912,655,793,427,632,482,682,709,366,250,440,413,790,468,574,803,466,864,904,423,371,599,628,481,573,681,249,789,668,707,411,678,460,350,626,365,246,848,900,571,782,191,666,787,407,363,677,458,620,436,625,591,399,781,665,434,349,898,618,245,840,465,359,662,567,675,318,457,428,736,238,376,347,617,779,454,816,836,661,897,604,243,426,614,237,127,372,720,602,808,834,654,559,613,453,433,222,412,659,343,451,775,317,235,601,364,653,422,221,712,833,804,598,611,315,370,425,410,335,572,480,248,792,231,597,688,651,802,421,708,543,369,311,570,219,348,190,788,801,590,406,419,595,680,706,464,569,362,303,358,647,589,409,566,244,189,786,346,705,676,405,215,456,242,126,565,361,780,452,624,587,785,664,403,398,345,674,187,287,357,558,316,236,432,778,563,616,660,673,397,342,241,557,207,583,183,234,125,355,424,777,341,395,612,334,658,774,450,220,555,391,314,313,175,123,600,652,542,610,657,230,420,233,408,229,368,773,339,310,218,541,650,119,333,596,609,551,404,217,302,449,188,331,214,649,896,771,309,227,539,159,396,594,360,186,418,301,646,568,213,111,327,588,593,417,286,832,402,307,356,645,344,535,564,240,586,206,185,299,401,211,354,182,562,585,800,124,643,394,340,285,205,95,203,295,556,353,582,561,527,283,232,181,122,393,174,312,448,554,581,704,784,332,228,179,121,390,308,279,338,553,330,216,540,173,579,118,199,389,550,63,776,672,271,416,326,337,538,226,158,549,329,117,171,110,656,306,157,212,387,115,772,537,300,225,167,305,298,534,547,109,184,155,400,608,648,770,210,533,94,204,284,325,297,323,107,209,180,151,294,592,644,769,526,93,531,202,392,352,120,282,525,178,103,642,584,293,91,281,336,278,62,201,172,143,198,560,177,580,523,641,291,388,116,61,170,87,197,270,156,552,114,578,304,59,277,169,269,328,519,386,224,195,166,577,108,548,79,154,113,55,275,165,536,324,106,153,546,267,385,208,296,150,92,163,47,532,545,105,102,263,149,292,90,322,200,530,142,101,280,321,176,147,524,89,99,529,60,31,141,196,86,276,290,58,522,168,85,289,139,194,78,112,521,274,518,57,268,83,54,768,193,164,135,77,384,517,152,104,162,53,266,75,640,273,46,148,262,515,100,161,51,88,45,146,576,71,265,320,98,261,140,30,43,145,84,29,138,544,259,97,56,137,82,39,288,134,528,27,76,272,81,133,52,74,520,23,192,50,73,44,264,70,131,15,516,49,160,42,69,28,514,260,96,41,144,67,38,26,513,258,80,37,136,25,22,35,257,72,132,21,14,48,130,68,19,129,13,66,11,40,7,36,24,65,256,34,20,33,512,18,12,17,128,10,9,64,6,5,3,32,16,8,4,2,1,0]

Huawei

[1023,1022,1021,1019,1015,1007,1020,991,1018,1017,1014,1006,895,1013,1011,959,1005,990,1003,989,767,1016,999,1012,987,958,983,957,1010,1004,955,1009,894,975,893,1002,951,1001,988,511,766,998,891,943,986,997,985,887,956,765,995,927,982,981,879,954,974,763,953,979,510,1008,759,863,950,892,1000,973,949,509,890,971,996,942,751,984,889,507,947,831,886,967,941,764,926,980,994,939,885,993,735,878,925,503,762,883,978,935,703,495,952,877,761,972,923,977,948,758,862,875,919,970,757,861,508,969,750,946,479,888,639,871,911,830,940,859,755,966,945,749,506,884,938,965,829,734,924,855,505,747,963,937,882,934,827,733,447,992,847,876,501,921,702,494,881,760,743,933,502,918,874,922,823,731,499,860,756,931,701,873,493,727,917,870,976,815,910,383,968,478,858,754,699,491,869,944,748,638,915,477,719,909,964,255,799,504,857,854,753,828,746,695,487,907,637,867,853,475,936,962,446,732,826,745,846,500,825,903,687,932,635,471,445,742,880,498,730,851,822,382,920,845,741,443,700,729,631,492,872,961,726,821,930,497,381,843,463,916,739,671,623,490,929,439,814,819,868,752,914,698,725,839,856,476,813,718,908,486,723,866,489,607,431,697,379,811,798,913,575,717,254,694,636,474,807,715,906,797,693,865,960,852,744,634,473,795,905,485,415,483,470,444,375,850,740,686,902,824,691,253,711,633,844,685,630,901,367,791,928,728,820,849,783,670,899,738,842,683,247,469,441,442,462,251,737,438,467,351,629,841,724,679,669,496,461,818,380,437,627,622,459,378,239,488,667,838,430,484,812,621,319,817,435,377,696,722,912,606,810,864,716,837,721,714,809,796,455,472,619,835,692,663,223,414,904,427,806,482,632,713,690,848,605,373,252,794,429,710,684,615,805,900,655,468,366,603,413,574,481,371,250,793,466,423,374,689,628,440,365,709,789,803,411,573,682,249,460,790,668,599,350,707,246,681,465,571,626,436,407,782,191,127,363,620,666,458,245,349,677,434,678,591,787,399,457,359,238,625,840,567,736,665,428,376,781,898,618,675,318,454,662,243,897,347,836,816,720,433,604,617,779,808,661,834,712,804,833,559,237,453,426,222,317,775,372,343,412,235,543,614,451,425,422,613,370,221,315,480,335,659,654,364,190,369,248,653,688,231,410,602,611,802,792,421,651,601,598,708,311,219,572,597,788,570,409,590,362,801,680,464,406,419,348,647,786,215,589,706,361,676,566,189,595,244,569,303,405,358,456,346,398,565,242,126,705,780,587,624,664,236,187,357,432,785,558,674,207,403,397,452,345,563,778,241,316,342,616,660,557,125,234,183,287,355,583,673,395,424,314,220,777,341,612,658,123,175,774,555,233,334,542,450,313,391,230,652,368,218,339,600,119,333,657,610,773,541,310,420,159,229,650,551,596,609,408,217,449,188,309,214,331,111,539,360,771,649,302,418,594,896,227,404,646,186,588,832,568,213,417,301,307,356,402,800,564,327,95,206,240,535,593,645,586,344,396,185,401,211,354,299,585,286,562,643,182,205,124,232,285,295,181,556,582,527,394,340,63,203,561,353,448,122,283,393,581,554,174,390,704,312,338,228,179,784,199,553,121,173,389,540,579,332,118,672,550,337,158,279,271,416,216,308,387,538,549,226,330,776,171,212,117,110,329,656,157,772,306,326,225,167,115,537,534,184,109,300,547,305,210,155,533,325,352,608,400,298,204,94,648,284,209,151,180,107,770,297,392,323,592,202,644,93,294,178,103,143,282,62,336,201,120,172,198,769,584,91,388,293,177,526,278,281,642,525,531,61,170,116,197,87,156,277,114,560,169,59,291,580,275,523,641,270,195,552,519,166,224,578,108,269,79,154,113,548,577,536,328,55,106,165,153,150,386,208,324,546,385,267,47,92,163,296,304,105,102,149,263,532,322,292,545,90,200,31,321,530,142,176,147,101,141,196,524,529,290,89,280,60,86,99,139,168,58,522,276,85,194,289,78,135,112,521,57,83,54,518,274,268,768,164,77,152,193,53,162,104,517,273,266,75,46,148,51,640,100,45,576,161,265,262,71,146,30,140,88,515,98,43,29,261,145,138,84,259,39,97,27,56,82,137,76,384,134,23,52,133,320,15,73,50,81,131,44,70,544,192,528,288,520,160,272,74,49,516,42,69,28,144,41,67,96,514,38,264,260,136,22,25,37,80,513,26,258,35,132,21,257,72,14,48,13,19,130,68,40,11,512,66,129,7,36,24,34,256,20,65,33,12,128,18,10,17,6,9,64,5,3,32,16,8,4,2,1,0]

ZTE

[1023,1022,1021,1019,1015,1007,1020,991,1018,1017,1014,959,1013,1006,1011,1005,895,990,1003,1016,999,767,989,1012,957,1010,987,1004,1009,955,983,894,958,975,1002,511,893,951,988,1001,998,943,985,891,982,997,766,887,956,927,995,765,986,974,879,981,953,763,979,950,973,1008,954,892,759,863,510,971,949,509,890,942,1000,947,751,941,996,967,886,507,889,831,984,926,994,939,885,764,503,925,735,980,878,993,935,762,978,952,923,875,972,883,761,495,862,758,948,970,919,703,877,977,946,861,757,750,911,966,969,871,940,508,945,479,755,830,859,888,938,965,506,749,829,884,639,505,963,855,924,734,934,747,502,937,882,733,827,922,447,847,876,933,743,501,823,921,881,874,731,494,702,931,918,760,499,917,860,727,873,992,493,756,701,870,383,815,478,858,910,869,491,754,976,699,719,915,857,854,748,909,504,753,487,255,867,638,799,828,695,746,968,907,853,637,826,944,745,964,500,475,446,846,742,732,825,635,903,851,687,730,498,477,845,445,936,962,741,822,729,932,961,821,631,843,726,739,814,497,492,471,382,880,671,463,700,920,930,443,725,490,839,698,929,813,916,872,819,718,439,623,381,723,697,489,868,914,798,474,811,694,908,486,717,913,431,476,379,797,866,856,485,607,636,807,906,686,865,715,852,473,752,254,375,691,470,795,634,905,444,685,253,850,483,415,744,633,711,902,824,693,630,462,367,442,791,844,849,740,469,575,901,683,670,251,842,820,629,441,728,467,738,461,622,841,818,496,669,899,783,437,737,679,724,380,247,351,621,627,838,667,817,812,459,722,430,488,438,378,837,810,696,716,619,455,239,663,377,606,721,484,809,429,435,714,796,835,806,692,374,960,319,605,472,690,713,482,615,414,794,655,805,710,427,366,252,684,413,603,468,689,928,793,481,223,373,709,803,574,371,250,632,423,682,790,573,440,466,681,912,789,628,249,365,599,707,460,411,678,668,246,571,626,350,864,436,465,407,191,782,363,666,904,787,591,620,458,625,349,781,245,677,848,900,434,665,238,457,618,567,399,359,454,428,662,243,779,675,433,840,617,898,237,376,604,661,453,318,426,127,347,559,816,317,614,602,836,736,613,654,372,897,659,222,775,451,343,412,425,235,601,808,370,422,315,834,720,410,653,335,611,598,543,364,248,231,409,572,421,597,804,651,369,833,712,362,311,419,570,792,590,802,406,688,244,215,189,595,242,221,569,708,405,647,589,361,348,788,480,303,358,801,566,680,706,346,398,236,403,187,219,464,786,587,565,676,397,345,705,357,558,780,241,785,126,190,342,456,207,624,316,287,664,674,233,563,125,355,395,557,314,432,583,778,660,452,673,616,234,183,777,341,220,334,339,391,230,175,555,658,424,774,542,612,313,217,123,333,450,541,652,657,214,310,420,229,773,449,551,600,610,119,331,309,368,596,650,408,418,188,218,227,539,159,609,771,301,649,302,327,360,417,206,404,111,594,646,307,186,568,213,588,535,185,593,402,645,356,211,286,564,586,896,240,396,299,344,205,124,401,95,285,354,181,585,643,562,527,174,394,295,353,340,582,122,232,203,182,561,556,393,338,832,283,121,228,581,554,179,390,332,199,312,173,337,118,800,553,389,158,63,579,540,308,550,330,216,279,704,226,171,784,306,157,329,117,538,549,387,212,271,110,225,167,326,537,672,300,115,776,210,109,325,184,305,534,547,155,204,94,298,533,656,772,297,209,448,202,107,180,323,151,284,608,526,648,178,93,294,770,531,103,282,293,416,525,172,198,143,120,592,281,644,769,201,116,62,170,197,291,177,523,400,278,584,114,91,642,61,156,277,195,169,352,560,392,580,641,154,270,165,108,113,87,59,79,552,336,519,578,275,269,388,153,166,105,150,328,548,577,55,386,102,163,267,224,47,536,304,546,324,92,385,149,106,89,142,208,532,545,263,296,101,322,530,141,86,292,200,60,321,31,147,524,99,176,90,280,290,529,196,139,58,83,194,78,522,135,289,168,276,85,274,57,164,518,521,112,193,54,152,75,268,162,273,77,266,104,53,517,46,148,161,515,265,100,768,51,71,88,262,45,140,146,30,98,145,261,640,138,84,43,29,259,97,576,56,39,137,384,134,76,81,27,544,74,52,320,23,133,82,528,50,44,131,73,288,69,49,15,192,272,520,70,42,28,516,160,67,41,264,514,144,38,26,37,96,260,513,25,136,258,80,22,132,35,257,21,72,130,48,68,14,19,13,40,129,66,11,36,24,7,65,34,20,33,18,12,512,17,256,10,128,9,64,6,5,32,3,16,8,4,2,1,0]

LG

[1023,1022,1021,1019,1015,1007,1020,991,1018,1017,1014,959,895,1013,1006,1011,1005,990,1003,989,958,1016,767,999,511,987,894,1012,957,983,1010,1004,1009,955,893,975,1002,766,988,951,1001,998,891,943,986,997,956,510,887,765,985,982,927,995,981,879,954,1008,763,974,892,953,979,509,950,973,863,759,1000,949,971,890,942,996,751,889,507,947,984,967,886,764,941,994,926,831,980,735,503,939,885,993,925,878,762,978,952,883,495,935,761,972,877,923,977,758,862,508,948,970,875,703,919,861,946,969,479,639,830,940,966,757,888,945,506,871,911,750,965,859,938,755,505,884,924,829,749,992,937,502,855,734,934,882,747,963,447,876,922,733,881,933,383,827,702,976,847,760,921,501,918,874,743,931,860,494,731,968,823,638,917,499,873,756,870,944,493,815,701,910,858,727,478,964,869,754,915,491,828,748,909,857,477,753,936,854,699,446,799,637,907,867,504,719,962,853,746,255,932,826,732,880,487,695,920,846,745,475,903,825,742,382,851,635,930,822,500,687,845,730,445,961,872,741,471,916,700,821,843,498,631,929,729,492,814,868,726,497,914,671,819,443,739,856,908,381,725,839,623,463,813,698,913,490,752,866,254,798,718,906,476,636,852,697,439,811,723,905,489,694,865,486,717,797,607,379,850,902,431,744,824,693,474,807,715,485,844,634,960,444,686,795,375,740,849,253,633,901,691,473,820,728,470,842,711,415,791,630,483,685,928,469,738,818,670,367,841,575,442,812,724,496,838,683,251,899,629,462,817,737,380,669,783,441,622,837,467,722,912,438,696,627,461,810,351,679,796,247,716,864,488,621,667,437,809,721,459,806,378,606,904,430,692,714,484,835,619,239,794,377,663,605,435,805,690,472,848,455,713,574,374,900,684,252,793,429,710,632,482,790,414,689,615,319,803,468,655,840,223,709,427,603,789,373,682,481,628,816,413,668,366,573,250,898,736,466,681,440,782,599,371,836,423,678,249,626,787,460,707,365,620,411,781,666,465,625,246,677,350,808,571,436,720,458,779,363,591,191,665,407,834,662,349,618,434,897,457,675,127,604,318,804,712,428,567,454,661,245,792,376,617,433,614,359,775,399,688,559,238,654,453,802,347,602,426,708,243,833,372,613,412,659,317,572,237,788,480,653,425,601,343,680,222,801,422,598,370,543,235,651,451,786,364,611,410,706,221,597,369,780,421,676,315,570,190,624,464,335,785,248,664,409,590,406,647,362,778,231,569,419,595,348,219,674,456,566,311,896,126,589,405,705,361,244,616,358,777,432,660,303,565,189,587,398,346,673,215,774,452,558,357,242,612,403,658,316,563,236,397,832,345,600,241,652,424,342,583,187,557,287,125,657,395,773,355,610,207,450,542,341,650,234,420,596,314,555,220,368,800,183,649,408,334,609,233,541,391,313,594,230,646,339,123,771,418,310,568,175,551,333,218,588,449,360,704,229,539,404,188,784,309,331,593,119,417,645,217,564,302,356,586,214,402,535,159,307,672,227,344,776,396,562,213,327,585,111,301,556,186,401,582,240,354,643,286,206,561,394,124,527,340,185,581,299,211,772,393,182,656,353,205,285,554,95,338,540,390,232,608,312,181,553,295,203,550,332,122,448,648,174,283,579,228,337,770,121,389,179,538,308,216,549,330,199,279,592,118,173,644,416,226,306,537,158,329,534,63,300,212,547,326,171,387,584,117,305,225,533,157,271,560,110,325,210,769,400,184,115,642,298,167,284,526,204,352,109,580,155,297,209,531,294,94,392,525,180,202,323,107,552,282,151,93,523,293,178,336,578,201,62,388,172,641,281,198,120,548,278,177,103,291,536,143,328,197,91,519,277,170,116,304,546,156,61,386,224,577,169,270,87,324,532,166,114,275,195,545,108,269,154,113,768,165,296,59,208,267,530,79,153,322,150,524,106,385,163,92,292,529,200,55,149,280,105,522,102,263,176,47,142,290,640,90,196,321,101,147,521,60,276,141,89,168,289,518,86,31,139,274,99,194,576,85,268,164,58,112,273,152,517,78,135,266,57,83,162,54,544,384,77,193,104,265,148,53,75,515,161,262,46,100,146,51,320,88,528,140,71,45,145,261,98,30,138,84,520,43,288,137,97,29,82,134,259,56,39,76,516,192,27,272,81,133,52,74,23,160,264,50,73,514,44,70,131,49,15,69,260,144,42,28,96,41,38,136,67,513,258,26,37,80,132,25,22,35,72,21,48,257,130,14,68,19,13,40,11,66,129,36,512,24,7,34,65,20,33,256,18,12,17,10,128,9,6,64,5,32,3,16,8,4,2,1,0]

Ericsson

[1023,1022,1021,1019,1015,1007,1020,991,1018,1017,1014,959,1013,1006,1011,1005,895,990,1003,989,1016,999,987,958,1012,767,957,983,1010,1004,955,1009,894,975,1002,951,893,988,1001,998,511,891,986,943,997,985,766,956,887,982,995,765,927,954,981,1008,879,974,763,953,979,950,892,973,1000,759,949,863,510,890,971,942,996,947,509,889,984,751,941,886,967,994,507,764,831,926,885,939,980,993,878,735,925,762,952,503,883,978,935,877,972,761,923,977,758,948,862,495,875,970,703,757,919,946,861,508,888,969,750,940,871,966,755,945,479,859,506,749,830,911,639,884,505,938,965,734,829,502,747,855,992,924,937,882,963,934,876,733,447,760,827,922,501,847,494,743,933,881,823,499,731,860,702,918,931,493,976,874,815,870,921,383,756,701,858,917,478,727,968,754,638,491,873,964,915,910,857,477,699,799,869,504,828,748,944,909,487,854,936,753,719,475,695,907,867,746,446,637,853,732,500,687,903,826,962,255,635,932,742,445,745,846,851,825,845,730,822,471,741,961,729,821,880,498,843,920,726,930,739,492,631,443,814,700,497,819,872,725,382,916,929,671,813,463,839,698,490,439,718,868,914,723,623,697,381,476,811,489,717,856,908,694,486,798,866,913,379,474,693,752,797,431,906,485,715,636,852,865,807,254,607,686,905,691,253,634,744,795,375,850,473,444,685,902,824,711,483,633,844,496,415,849,740,470,630,901,251,820,791,683,842,728,442,670,629,738,367,575,469,351,462,818,841,441,669,438,899,724,737,622,679,812,627,838,817,467,783,247,621,380,461,667,722,488,810,837,960,696,437,239,716,459,378,721,319,619,809,692,430,606,663,435,796,835,714,484,806,928,223,429,605,455,690,713,615,377,794,472,805,374,414,252,684,710,373,689,603,912,655,793,427,632,482,682,709,191,468,366,413,790,466,574,803,423,864,904,250,481,599,628,371,573,681,440,789,668,707,365,678,411,460,626,249,350,848,900,571,782,246,666,787,436,407,677,349,620,363,625,591,458,781,665,399,434,898,618,465,840,238,127,662,567,675,245,359,318,736,457,428,376,617,779,347,816,836,661,897,604,454,243,614,426,237,372,720,602,808,834,654,559,613,453,433,222,412,659,343,451,775,317,235,601,364,653,422,221,712,833,804,598,611,315,370,425,410,335,572,480,231,792,190,597,688,651,802,369,708,543,421,311,570,409,219,248,788,801,590,348,406,595,680,706,419,569,464,362,303,647,589,358,566,244,189,786,215,705,676,405,456,242,126,361,565,398,780,346,624,587,785,664,187,236,403,674,357,316,432,558,207,287,397,778,563,616,660,673,452,342,345,557,241,583,125,234,355,424,395,777,314,183,612,220,658,774,123,233,555,341,450,391,334,175,600,652,542,610,657,420,230,313,218,339,119,773,229,368,310,541,650,408,333,596,609,551,404,159,214,217,302,449,111,649,896,771,188,331,539,309,227,594,360,418,186,213,646,568,307,396,301,588,593,327,417,832,286,402,356,645,344,535,564,95,586,206,240,185,299,401,211,354,562,585,800,182,643,124,285,205,203,295,232,181,556,63,582,561,527,174,122,228,179,283,394,199,340,554,581,704,784,353,121,173,393,338,390,216,158,553,118,279,540,226,579,117,171,312,550,332,776,672,337,389,308,212,538,110,157,549,330,225,115,167,656,329,271,306,387,184,772,537,204,109,155,326,300,534,547,305,210,325,94,608,648,770,180,533,107,298,151,209,297,202,93,323,284,143,178,592,644,769,526,62,531,172,103,294,448,201,525,282,61,642,584,198,120,177,91,293,281,416,291,170,197,560,87,580,523,641,278,277,116,156,195,114,169,59,108,552,400,578,166,352,270,392,275,224,519,79,154,113,165,577,269,548,55,336,153,106,388,163,536,150,267,92,546,208,47,102,149,200,386,105,328,532,545,90,304,142,31,101,176,147,324,530,385,263,89,141,296,196,524,86,60,529,99,322,139,292,168,85,321,58,522,280,290,194,78,135,112,164,521,57,518,83,54,77,276,768,289,152,193,53,274,517,46,162,104,75,268,148,640,51,273,266,100,515,71,45,161,30,146,140,576,88,43,265,262,98,29,145,138,84,39,261,97,544,259,27,137,82,56,76,23,134,528,192,133,384,52,15,81,74,520,320,50,44,131,73,70,160,49,42,516,69,288,28,144,41,514,67,26,38,272,96,136,37,513,264,25,22,132,35,260,80,21,258,14,72,130,48,19,13,257,68,40,129,11,7,66,36,24,65,34,20,33,18,512,12,17,10,256,128,9,6,5,64,3,32,16,8,4,2,1,0]

MTK

[1023,1022,1021,1019,1015,1007,991,1020,1018,1017,959,1014,1013,1006,1011,895,1005,990,1003,989,999,987,1016,958,767,957,1012,983,1010,1004,955,894,1009,975,1002,951,893,988,511,1001,998,891,986,943,766,997,985,956,995,887,765,982,927,954,981,974,763,953,879,979,950,1008,892,973,510,759,949,1000,890,971,863,942,509,996,947,889,984,941,967,751,886,507,994,764,831,926,980,885,939,878,503,993,762,925,978,735,883,952,935,877,972,761,923,977,948,758,862,495,875,703,479,970,946,919,757,861,969,888,966,750,940,871,508,755,506,945,859,965,938,830,749,911,884,505,639,924,937,882,963,829,734,855,934,447,747,876,922,881,733,502,933,760,992,501,827,874,743,494,921,847,731,756,931,918,702,499,823,976,873,860,383,870,917,754,701,727,968,493,748,910,858,478,944,869,753,815,491,915,699,909,964,857,746,477,638,828,487,936,719,867,854,475,695,745,732,637,907,962,826,853,799,504,446,932,742,255,961,500,880,846,825,730,920,903,851,741,635,687,445,471,822,498,845,930,872,729,492,916,700,821,443,739,726,631,382,843,929,914,814,497,463,698,725,490,671,819,868,908,856,439,752,476,913,813,839,718,381,697,489,623,723,486,866,906,694,798,717,852,379,865,811,636,474,431,254,744,693,905,797,485,473,850,824,634,444,715,607,483,375,902,807,844,253,686,740,470,691,795,633,849,415,442,901,820,469,728,842,630,711,685,738,462,251,441,367,818,841,467,438,791,724,629,899,575,670,496,737,838,380,683,812,960,461,817,247,622,722,669,437,837,488,627,810,696,783,459,716,378,679,351,621,721,430,928,484,809,667,435,377,796,692,835,714,806,472,606,912,429,455,239,374,619,482,794,690,713,864,805,663,605,223,319,414,710,468,684,373,427,632,366,793,689,615,904,803,848,790,481,709,252,413,682,603,655,466,250,371,628,574,440,423,365,789,681,460,900,249,668,411,707,840,465,573,626,678,599,436,191,782,350,898,736,363,787,620,666,246,677,816,625,571,836,781,458,434,407,245,349,457,665,618,591,376,897,454,808,675,662,238,428,359,720,243,567,779,617,433,347,453,661,426,604,318,614,834,804,127,399,372,712,654,602,412,613,833,425,775,659,792,559,237,370,451,317,222,343,688,422,802,653,601,235,708,364,611,598,410,369,221,572,421,788,231,480,680,801,315,219,651,597,362,570,409,543,335,706,248,190,786,419,676,406,189,311,590,569,361,464,595,705,780,647,624,348,405,664,358,566,244,589,785,215,674,242,456,236,398,565,778,346,660,432,357,187,616,587,303,673,403,126,397,345,452,558,777,241,658,563,207,316,234,424,355,612,774,342,652,583,557,600,183,220,125,395,233,657,314,610,450,341,287,230,773,420,650,542,555,449,596,368,334,313,609,123,408,391,339,218,175,229,310,333,541,896,649,418,594,360,771,551,217,646,568,404,188,309,588,227,119,214,539,331,593,417,645,402,302,159,586,564,186,832,213,307,356,396,344,206,401,327,301,585,535,562,185,111,800,211,182,643,582,354,556,394,240,286,95,340,299,704,561,205,353,581,181,232,554,393,527,285,338,203,784,390,124,122,312,295,332,672,553,174,228,179,283,579,121,337,540,550,389,308,216,330,173,199,63,776,656,538,226,549,306,329,279,118,158,387,212,772,117,171,326,110,300,537,448,305,225,608,157,547,210,534,648,325,115,298,184,271,167,109,204,770,155,533,416,297,209,592,644,284,180,323,94,294,107,202,526,531,769,584,400,282,178,151,293,93,352,525,201,642,103,560,198,172,281,91,177,580,392,291,278,120,197,170,62,523,336,641,143,552,116,277,169,388,578,156,195,61,87,328,114,166,270,548,386,275,519,108,154,59,577,165,113,224,536,304,269,324,79,153,106,163,546,385,150,296,55,92,267,532,208,105,149,102,545,322,292,200,142,90,530,176,47,263,101,321,280,147,524,141,89,290,196,529,31,99,86,276,168,522,139,289,60,194,58,85,521,274,164,193,78,57,83,518,54,268,135,152,112,77,273,162,768,53,517,266,104,148,75,161,146,265,46,515,51,100,262,140,640,88,45,71,145,98,261,576,138,30,84,43,97,384,137,29,82,259,134,544,39,27,76,56,81,133,320,52,74,528,288,23,50,73,131,70,520,44,49,192,69,15,42,272,516,160,28,41,67,38,264,514,144,26,37,513,260,96,25,136,35,22,258,80,21,132,257,72,14,130,19,48,13,68,129,40,11,66,36,65,24,7,34,20,33,18,12,17,10,512,9,6,256,5,128,3,64,32,16,8,4,2,1,0]

Qualcomm

[1023,1022,1021,1019,1015,1007,991,1020,1018,959,1017,1014,1013,1006,1011,1005,990,1003,895,989,999,958,987,1016,767,957,983,1012,955,894,1010,511,975,1004,951,1009,1002,893,766,988,943,1001,998,986,891,997,956,927,765,510,985,887,982,995,954,981,879,974,763,953,950,863,979,509,1008,973,759,949,892,942,971,890,1000,751,947,507,941,831,967,996,926,735,889,984,939,886,503,764,994,925,885,980,935,762,878,495,993,952,883,703,923,978,877,972,479,761,948,862,919,977,758,875,508,970,861,757,946,940,871,750,506,911,830,969,966,755,859,447,945,888,749,938,829,965,505,734,924,855,747,937,884,502,934,639,827,963,882,501,733,992,847,743,922,876,494,933,702,921,823,499,731,918,881,760,976,493,874,860,701,815,931,478,727,873,917,756,491,870,910,699,858,383,968,638,869,828,754,477,944,799,719,487,915,857,748,909,446,695,854,964,753,475,867,936,746,826,504,853,637,907,255,732,846,962,687,445,825,745,932,471,822,742,851,500,903,730,880,635,443,961,741,845,382,821,700,920,498,671,814,930,463,729,843,631,492,872,726,916,439,819,739,813,497,698,929,490,725,839,623,381,868,914,718,254,798,697,856,476,694,908,431,607,811,723,489,866,752,486,913,717,797,474,693,852,379,906,636,485,807,686,865,444,715,905,415,634,824,253,744,795,691,473,850,685,375,575,711,470,902,740,483,844,670,442,820,633,469,960,728,367,791,683,849,462,630,842,251,738,901,441,669,438,629,818,351,724,812,467,679,783,496,622,928,461,737,841,247,696,838,437,627,380,817,667,430,722,810,459,621,899,378,716,488,837,692,239,796,606,435,663,429,809,721,319,806,619,455,912,484,714,605,414,835,223,690,864,377,684,615,472,794,427,655,805,374,574,252,904,713,482,710,413,793,373,603,790,468,689,423,632,848,250,682,366,573,481,803,709,440,789,371,668,599,191,900,411,681,466,628,365,782,678,460,840,249,571,436,707,816,350,626,407,736,465,787,246,591,363,666,781,458,620,349,677,245,836,434,665,428,567,898,359,662,238,625,399,808,318,720,457,618,454,779,243,604,347,559,433,834,376,675,617,237,661,426,804,317,614,453,654,222,775,412,602,343,712,235,833,425,613,372,572,422,688,127,792,315,543,451,659,370,221,480,802,335,601,231,653,410,598,364,788,421,708,190,611,409,897,311,680,219,570,406,597,369,801,248,651,464,590,362,706,348,786,189,303,569,419,676,215,566,361,780,405,595,244,647,358,624,398,785,187,705,346,456,589,126,565,357,664,316,778,242,558,432,674,287,207,587,403,777,345,616,236,660,397,183,563,342,557,452,774,241,355,673,424,583,234,612,314,658,341,125,832,395,542,220,600,334,450,652,175,555,313,233,773,420,610,310,230,657,339,541,391,596,218,368,650,123,800,449,229,551,333,609,309,188,649,408,568,159,302,896,418,217,539,331,594,119,360,646,214,404,588,307,564,227,301,704,186,417,771,213,535,327,111,593,356,402,586,206,645,286,185,562,344,182,556,396,95,784,299,211,527,354,240,672,401,205,585,285,181,582,340,561,394,124,554,295,174,776,353,203,643,393,122,312,581,232,283,540,179,338,553,173,63,550,199,390,656,228,332,579,158,308,121,608,448,216,772,279,171,538,337,118,549,330,226,389,648,157,117,537,306,212,300,534,167,271,592,110,416,225,547,329,184,533,326,115,305,644,155,210,298,770,109,387,526,204,325,180,584,284,94,151,560,297,209,294,531,107,400,202,525,93,323,178,580,352,172,103,642,282,143,293,552,62,392,201,523,198,281,91,278,578,177,120,336,548,170,61,291,197,519,277,156,87,769,388,169,116,270,577,166,328,59,195,536,304,114,224,275,546,79,154,269,532,108,165,324,153,55,641,386,150,113,296,545,208,106,267,92,530,47,322,163,105,149,524,292,102,142,263,529,90,200,321,101,60,522,176,280,31,147,290,89,521,141,86,276,196,99,518,385,168,58,85,289,139,576,78,194,274,57,164,54,517,268,83,135,112,273,193,544,77,53,152,266,46,162,75,768,265,104,148,51,45,262,161,640,71,100,146,320,30,88,140,43,515,261,98,145,29,84,138,288,39,528,97,137,56,384,27,82,134,76,52,192,520,259,23,81,74,133,50,44,272,15,160,73,516,70,49,42,264,131,69,28,41,38,144,67,96,260,514,26,37,136,25,22,80,35,21,132,258,14,72,48,19,13,68,513,130,40,11,66,36,7,257,65,24,34,20,129,33,18,12,17,10,9,6,64,5,32,512,256,128,3,16,8,4,2,1,0]