3GPP TSG RAN WG1 Meeting #90	      R1-1714167

Prague, Czech Republic, 21st – 25th August 2017

Agenda Item:	6.1.4.1.2

Source:	InterDigital Inc.

Title:	Code Block Segmentation for Data Channel

Document for:	Discussion and Decision

# 1	Introduction

The detailed design of LDPC codes for eMBB data channel has been discussed, and several agreements and working assumptions have been achieved [1]-[4].

The two base graphs of LDPC codes were agreed and provided in [5]. The dimension of the first base matrix is 46 x 68 and the dimension of the second base matrix is 42 x 52. The largest shift size of the first base matrix is 384 and the largest shift size of the second base matrix is 256.

Besides the LDPC base graph design, the coding chain for eMBB data channel has been discussed [2]-[4]. One important component in the coding chain for eMBB data channel is code block (CB) segmentation. CB segmentation determines the number and the size(s) of CBs per TB. Since LDPC encoding will be applied to each CB, CB segmentation is related to the shift size selection of LDPC code. It was agreed [4] that for all the CBs segmented from a TB, the resulting shift size is of the same value.

In this contribution, we discuss the two options of code block segmentation for eMBB data channel.

# 2	Discussion

Two LDPC base matrices will be used for eMBB data channel. The first base matrix is of dimension 46 x 68, which implies the maximum information column blocks in the base matrix is Kb, max1=22 and the lowest code rate is 1/3. The set of shift sizes for the first base graph is denoted as z1. The set z1 is composed of values of a∙2j, with a={2, 3, 5, 7, 9, 11, 13, 15}, j ={0, …, 7}, and the largest shift size is 384. Hence, the supported information block length of the first base matrix is K1=Kb,max1∙z1, and the largest information block length is KCB,max1=8448 bits.

The second base matrix is of dimension 42 x 52, which implies the lowest code rate is 1/5. Depending on the CB sizes, the possible maximum information column blocks in the base matrix Kb, max2 could be 10, 9, 8, 6. The set of shift sizes for the second base graph is denoted as z2. It is a working assumption that the largest shift size is 256. The supported information block length of the second base matrix is K2=Kb,max2∙z2, and the largest information block length is KCB,  max2=2560 bits.

It was agreed [4] that the base graph 1 is used for the initial transmission and subsequent re-transmissions of the same transport block (TB), when code block size (CBS) > X or code rate of the initial transmission is larger than Y. Otherwise, the base graph 2 is used. It is a working assumption that X is 2560 and Y is 2/3. Note that the CBS may depend on the CB segmentation from a TB, as the largest information block length used in the CB segmentation process is associated with the base graph selection. Specifically, KCB,max is set as 8448 bits if base graph 1 is selected; KCB,max is set as 2560 bits if base graph 2 is selected. In [6], we propose a way of selecting base graph, based on code rate and transport block size (TBS), rather than CBS. In this contribution, we simply assume that KCB,max is already determined.

Determine the number of code blocks

Once the KCB,max is determined, the number of CBs per TB could be calculated based on TB size. Suppose a TB is of size B bits. If B is larger than a threshold, then a 24-bit TB level CRC is attached to the TB. Otherwise, the TB level CRC length could be less than 24 bits. Let CRCTB be the number of CRC bits attached to the TB. The number of CBs in a TB is calculated as

C=B+CRCTBKCB,max-CRCCB,                              (1)

where CRCCB is the CB level CRC length and x is the smallest integer larger than or equal to x. The exact value of CRCCB is to be determined.

Proposal 1: Apply equation (1) to determine the number of CBs in a TB.

Determine the code block sizes

Once the number of CBs per TB is determined, the segmentation of a TB to CBs could be processed by one of the following two options.

Option 1: Equally partition a TB for all but the last CBs.

Each of the first (C-1) CBs could contain B+CRCTBC source bits (i.e., TB plus TB level CRC), while the last CB could contain (B+CRCTB)- (C-1)B+CRCTBC source bits. We could pad CB+CRCTBC-(B+CRCTB) zeros to the last CB so that it has the same length as the other CBs. The selection of the shift size Z in z could be such that Kb,max∙Z is the smallest value larger than or equal to K=B+CRCTBC+CRCCB bits. Additional filler bits will be appended to each CB to match the selected value of Z. Note that the difference between padding bits and filler bits is that the padding bits will be sent over the air together with the source bits, while the filler bits will be removed after LDPC encoding.

One advantage of this option is that all the CBs sent over the air are of the same length. This simplifies the operations at the receiver side. This option is shown in Figure 1.

Figure 1: Example of CB generation for Option 1

Option 2: Approximate equally partition a TB for all the CBs

Define two sizes K+=B+CRCTB+C∙CRCCBC and K-=B+CRCTB+C∙CRCCBC. Let the first C+ CBs contain K+-CRCCB source bits, while the remaining CBs contain K+-CRCCB source bits. Find a supported information block length K' in the set K of supported information block length, such that K' is the minimum value larger than K+. For each of the first C+ CBs, insert K'-K+ filler bits. For the remaining CBs, insert K'-K- filler bits. One disadvantage of this option is that the CBs sent over the air may be of different lengths. This may introduce additional operations at the receiver side. This option is shown in Figure 2.

Figure 2: Example of CB generation for Option 2

It is expected that the BLER performance of all CBs will be similar between Option 1 and Option 2. Option 1 may be selected due to its simpler receiver processing. Hence, we have the following proposal.

Proposal 2: Adopt Option 1 for code block segmentation.

# 3	Conclusion

In this contribution, we have the following proposals for code block segmentation in the LDPC coding chain design:

Proposal 1: Apply equation (1) to determine the number of CBs in a TB.

Proposal 2: Adopt Option 1 for code block segmentation.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 #88 Meeting, Athens, Greece, Feb. 2017.

- Chairman’s Notes, 3GPP TSG RAN WG1 #88bis Meeting, Spokane, USA, Apr. 2017.

- Chairman’s Notes, 3GPP TSG RAN WG1 #89 Meeting, Hangzhou, China, May 2017.

- Chairman’s Notes, 3GPP TSG RAN WG1 NR Ad-Hoc #2, Qingdao, China, Jun. 2017.

- R1-1711982, “WF on LDPC parity check matrices,” Nokia, Jun. 2017.

- R1-170xxxx, “On LDPC base graph selection”, InterDigital, Aug. 2017.