3GPP TSG RAN WG1 Meeting #90	              			     R1-1715041

Prague, Czech Republic, 21th – 25th August 2017

Title:	LS on RAN3 signaling support for LTE-NR coexistence feature

Response to:	-

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN3

Cc:	-

Contact Person:

#### Name:	Ralf Bendlin

E-mail Address:	ralf.bendlin@att.com

1. Overall Description:

In order to enable coordination between LTE and NR for LTE-NR coexistence in overlapping and adjacent spectrum, RAN1 reached the following agreement:

Agreements: - For LTE-NR coexistence in overlapping spectrum, - Send an LS to RAN3 to specify the Xn interface and enhanced X2 interface messages that enable coordination between LTE and NR, including - LTE cell on/off configuration with details up to RAN3 - LTE MBSFN subframe configuration - DL and/or UL carrier center frequency (ARFCN) - Carrier bandwidth - Signaling related to timing synchronization and SFN - Note: this does not require the network to be synchronized and/or SFN aligned and/or radio frame boundary aligned - Note: It is up to RAN3 if this requires new procedures in addition to signaling support - Indication of semi-statically used resources (to avoid collisions with, e.g., CSI-RS, SRS, PRACH, PUCCH, DRS, PSS/SSS, PBCH, …) - Indication of slots/PRBs not intended for transmissions by the eNB and gNB, respectively - For LTE-NR coexistence in adjacent spectrum, - Send an LS to RAN3 to specify the Xn interface and enhanced X2 interface messages that enable coordination between LTE and NR, including - Signaling related to timing synchronization and SFN - Note: this does not require the network to be synchronized and/or SFN aligned and/or radio frame boundary aligned - Note: It is up to RAN3 if this requires new procedures in addition to signaling support - TDD UL/DL configuration in case of LTE and NR and special subframe configuration in case of LTE

2. Actions:

To RAN3 WG:

RAN1 kindly requests RAN3 to take the above RAN1 agreements into account.

3. Dates of next TSG-RAN WG1 Meetings:

RAN1 NR Ad-Hoc#3	18th – 21th September 2017	Nagoya, Japan

RAN1 #90bis	9th – 13th October 2017	Prague, Czech Republic