3GPP TSG RAN WG1 Meeting #90		R1-1715149

Czechia, 21th – 25th August 2017

Title:	New configuration and UE capabilities for NB-IoT

# Response to:

# Release:	Rel-14

Work Item:	NB_IOTenh-Core

Source:	RAN1

To:	RAN2

Copy:

# Contact Person:

# Name:	Alberto Rico

E-mail Address:	albertor@qti.qualcomm.com

Attachments:	None

# 1. Overall Description:

RAN1 discussed the issue of interference randomization for NB-IoT. RAN1 decided to adopt an interference randomization scheme for NPDSCH and NPDCCH as an essential correction to Rel-14.

In particular, RAN1 made the following agreements:

- The interference randomization technique for NPDCCH is applied in the following cases:

  - For Type-2 and Type1-CSS, in non-anchor carriers.

  - For USS, is enabled by RRC configuration.

  - For Type-1A and Type-2A, always.

- The interference randomization technique for NPDSCH is applied in the following cases:

  - For PDSCH scrambled by RA-RNTI, TC-RNTI and P-RNTI, in non-anchor carriers.

  - For C-RNTI, is enabled by RRC configuration.

  - For G-RNTI and SC-RNTI, always.

For this new interference randomization scheme, the following signalling needs to be introduced in RAN2 specification:

- UE capability indicating support of interference randomization (e.g. new IE intRandomization)

- UE-specific configuration to enable interference randomization (e.g. new IE enableIntRandomization)

# 2. Actions to RAN2:

RAN1 respectfully requests RAN2 to implement the signalling indicated above.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #90-Bis	Oct. 9-13,  2017 		      Prague (CZ)