3GPP TSG RAN WG1 Meeting 90bis 		               R1-1718453

Prague, CZ, 9th – 13th, October 2017

Source:	Ericsson

Title:	Antenna port mapping tables

Agenda Item:	7.2.3.8

Document for:	Discussion and Decision

# Introduction

In this contribution, we show more details on antenna port mapping tables for single front loaded symbol

The single symbol front loaded table proposal for Type 1 DMRS is:

| Value | #Layers | Port (+1000) | DMRS FDM with data | k' for PTRS mapping | MU-MIMO | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | No | 0 | No | Comb#0, SU-MIMO |
| 1 | 1 | 0 | No | 0 | Yes | Comb#0, MU-MIMO |
| 2 | 1 | 0 | Yes | 0 | No | Comb#0, FDM with data |
| 3 | 1 | 1 | No | 1 | Yes | Comb#0, MU-MIMO |
| 4 | 1 | 2 | No | 0 | Yes | Comb#1, MU-MIMO |
| 5 | 1 | 3 | No | 1 | Yes | Comb#1, MU-MIMO |
| 6 | 2 | 0,1 | No | 0 | No | Comb#0, FD-OCC, SU-MIMO |
| 7 | 2 | 0,1 | No | 0 | Yes | Comb#0, FD-OCC, MU-MIMO |
| 8 | 2 | 0,1 | Yes | 0 | No | Comb#0, FD-OCC, FDM with data |
| 9 | 2 | 2,3 | No | 0 | Yes | Comb#1, FD-OCC, MU-MIMO |
| 10 | 2 | 0,3 | - | 0 | No | Comb#0 and Comb#1, FDM, SU-MIMO |
| 11 | 3 | 0,1,2 | - | 0 | No | Comb#0 and Comb#1, SU-MIMO |
| 12 | 3 | 0,1,2 | - | 0 | Yes | Comb#0 and Comb#1, MU-MIMO |
| 13 | 4 | 0,1,2,3 | - | 0 | No | Comb#0 and Comb#1, SU-MIMO |
| 14-15 | Reserved |  |  |  |  |  |

And for Configuration Type 2 DMRS with single CW transmission. The case of two CW transmission and single front loaded symbol does not need to be supported since the maximum number of  SU-MIMO layers is bounded by 6 only.

| Value | #Layers | Port (+1000) | DMRS FDM with data | k' for PTRS mapping | MU-MIMO | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | Yes | 0 | No | FDM-group#0, SU-MIMO, FDM with data |
| 1 | 1 | 0 | No | 0 | Yes | FDM-group#0, MU-MIMO |
| 2 | 1 | 1 | No | 1 | Yes | FDM-group#0, MU-MIMO |
| 3 | 1 | 2 | No | 0 | Yes | FDM-group#1, MU-MIMO |
| 4 | 1 | 3 | No | 1 | Yes | FDM-group#1, MU-MIMO |
| 5 | 1 | 4 | No | 0 | Yes | FDM-group#2, MU-MIMO |
| 6 | 1 | 5 | No | 1 | Yes | FDM-group#2, MU-MIMO |
| 7 | 2 | 0,1 | No | 0 | No | FDM-group#0, FD-OCC, SU-MIMO, FDM with data |
| 8 | 2 | 0,1 | No | 0 | Yes | FDM-group#0, FD-OCC, MU-MIMO |
| 9 | 2 | 0,2 | Yes | 0 | No | FDM-group#0+FDM-group#1, FDM, SU-MIMO, FDM with data |
| 10 | 2 | 2,3 | Yes | 0 | Yes | FDM-group#1, MU-MIMO |
| 11 | 2 | 4,5 | Yes | 0 | Yes | FDM-group#2, MU-MIMO |
| 12 | 3 | 0,1,2 | Yes | 0 | No | FDM-group#0+FDM-group#1, SU-MIMO,FDM with data |
| 13 | 3 | 0,1,2 | No | 0 | Yes | FDM-group#0+FDM-group#1, MU-MIMO |
| 14 | 4 | 0,1,2,3 | No | 0 | No | FDM-group#0+FDM-group#1, FD-OCC, SU-MIMO |
| 15 | 4 | 0,1,2,3 | No | 0 | Yes | FDM-group#0+FDM-group#1, FD-OCC, MU-MIMO |