3GPP TSG RAN WG1 Meeting 91 									R1-1720117

Reno, USA, November 27th – December 1st, 2017

Agenda Item:	7.2.2.3

Source:	Apple Inc.

Title:	Discussion on beam measurement and reporting

Document for:	Discussion/Decision

# Introduction

NR, as a new Radio Access Technology (RAT), is designed to provide better services including high data rate, low latency, high reliability and high connection density, etc. Among all those performance metrics, achieving high data rate for eMBB service is very important. To improve the date rate, it is crucial to design a system that can work at higher frequency band (above 6GHz or mmW), where large bandwidth can be available. High frequency band communications are typically limited by the analog beam forming due to RF constraint as well as the coverage requirement, therefore, beam measurement and reporting is one of the most important topics for high frequency band communication design.

So far, in 3GPP RAN1 meeting, many progresses have been made regarding the beam measurement and reporting. Firstly, general DL beam management procedures (P1, P2 and P3) and UL beam management procedures (U1, U2 and U3) have been agreed in RAN1 #86 and #86b [1][2], respectively.

Agreements: - The following DL L1/L2 beam management procedures are supported within one or multiple TRPs: - P-1: is used to enable UE measurement on different TRP Tx beams to support selection of TRP Tx beams/UE Rx beam(s) - For beamforming at TRP, it typically includes a intra/inter-TRP Tx beam sweep from a set of different beams - For beamforming at UE, it typically includes a UE Rx beam sweep from a set of different beams - FFS: TRP Tx beam and UE Rx beam can be determined jointly or sequentially - P-2: is used to enable UE measurement on different TRP Tx beams to possibly change inter/intra-TRP Tx beam(s) - From a possibly smaller set of beams for beam refinement than in P-1 - Note: P-2 can be a special case of P-1 - P-3: is used to enable UE measurement on the same TRP Tx beam to change UE Rx beam in the case UE uses beamforming Agreements: - UL beam management is to be further studied in NR - Similar procedures can be defined as DL beam management with details FFS, e.g.: - U-1: is used to enable TRP measurement on different UE Tx beams to support selection of UE Tx beams/TRP Rx beam(s) - Note: this is not necessarily useful in all cases - U-2: is used to enable TRP measurement on different TRP Rx beams to possibly change/select inter/intra-TRP Rx beam(s) - U-3: is used to enable TRP measurement on the same TRP Rx beam to change UE Tx beam in the case UE uses beamforming

So far, for all three types of beam measurement and reporting, i.e. periodic, semi-persistent and aperiodic, the agreement focuses on the basic NW controlled behavior. In other words, NW configures periodic or semi-persistent resources for UE to measure and UE reports periodically or semi-persistently, or NW can configure aperiodic resource for UE to measure and report. We list some important agreement in RAN1 so far regarding the beam measurement and reporting [3][4]

Agreement: - Support configuration of SSB for a UE to measure and report one or more L1-RSRP(s) Agreement: - For non-grouping based beam reporting, support the following reports parameters: - Maximal number of configured Tx beams for beam measurement: K equals 64 - Maximal number of configured Tx beams to be reported in one instance: N_max = 2, 4 where a subset of N (N<=N_max where N = 1, 2, 4) beams can be selected by the gNB and indicated to the UE (FFS signaling mechanism) - Reporting differential L1-RSRP when multiple beams are reported in one reporting instance. Reference is the largest L1-RSRP in that reporting instance. FFS other reference for differential reporting. - FFS applicable reporting channels and number of beams, and associated reporting contents - FFS: the UE adjusts the L1-RSRP of multiple RS resources according to the power offset between them - Bit-width: 7bit for L1-RSRP ranging from -140dBm to -44dBm with 1dB stepping size (analogous with LTE) and 4bit for differential L1-RSRP - FFS stepping size of differential quantization Agreement: NR supports the following configurations for beam management where a resource set is formed from multiple beam management CSI-RS resources and is contained within a resource setting: Single resource set with repetition = “OFF” UE reports CSI-RS resource indicator(s) within this resource set for CRI feedback Single resource set with repetition = “ON” UE does not report CRI FFS: Further support additional configuration by down selection from the following two alternatives: (a) Multiple resource sets, all with repetition = “ON” UE reports CSI-RS resource set indicator(s) for CRI feedback - FFS: Whether set ID(s) are local within a resource setting or global across all resource settings (b) Multiple equal-size resource sets, all with repetition = “OFF” UE reports distinct local CSI-RS resource indicator(s) within one or more resource sets. The UE can assume that the gNB applies the same Tx beams in the same order for each of the sets

The main purpose of beam measurement and reporting is to track the beam change due to various reasons, such as UE movement, UE rotation, obstacle passing by, etc. Therefore, beam measurement and reporting has the following requirement

- Minimize the overhead for both DL/UL reference signals and corresponding UL/DL measurement report

- Fast response to various mobility situations

With the above requirements in mind, we discuss some of the techniques that could be used to improve the beam measurement/reporting response time and/or reduce the overhead of reference signals/UE reporting.

# UE Event Based Aperiodic Beam Measurement Reporting

When periodic or semi-persistent reference signals (CSI-RS, or SSB) are configured, the simplest solution is for UE to make measurement report periodically. However, this approach has the following limitation,

- It is hard for the gNB to optimally configure the periodicity of reference signals as gNB has less knowledge about the actual mobility situation compared to UE

- To ensure robust beam management, frequent reference signals and reports are needed which can cause unnecessary overhead.

- Frequent periodic measurement reporting can also negatively impact the UE power consumption

On the other side, event based measurement report mechanism has been proved to effectively maintain good mobility service for LTE/UMTS etc. and thus deserved to be visited for beam mobility maintenance. While previous RATs, such as UMTS and LTE, focuses on the cell level mobility management, beam management can be viewed as the beam level mobility where similar concept can be applied in order to reduce the overhead while ensure robust mobility performance.

For event based aperiodic beam measurement reporting, UE only triggers mobility event report when certain predefined condition is met. Below are a few examples of the mobility even used in LTE and UMTS

LTE

- Event A1: Serving cell becomes better than threshold

- Event A2: Serving cell becomes worse than threshold

- Event A3: Neighbor cell becomes offset better than PCell

- Event A4: Neighbor cell becomes better than threshold

UMTS

- Event 1a: a cell enters the reporting range, i.e. add a cell to active set.

- Event 1b: a cell leaves the reporting range, i.e. removed a cell from active set.

- Event 1d: Neighbor cell becomes offset better than PCell.

Figure 1: UE Event Based Aperiodic Beam Measurement Reporting

Figure 1 illustrates the event based measurement reporting compared to periodic report. Assuming NW configures periodic measurement resources (SSB or CSI-RS). The baseline operation is for UE to make the periodic measurement report in order to track the beam change. Note that, due to the un-predictableness of the beam change, a frequent beam reporting is needed in order to ensure the robustness of the beam management as shown by red arrows in Figure 1. Assuming UE desirable receive beam changes due to, e.g., device rotation, UE can detect the change via periodic measurement. Only when the beam change is detected, or certain event is detected, UE is required to report the measurement as shown by green arrows. Clearly, event based measurement can reduce the reporting overhead while maintaining the same robustness of the beam management. Therefore, we propose the following

Proposal 1: NR to support UE event based aperiodic beam measurement reporting.

FFS: the detailed events and configurations

# UE Assisted Beam Management Procedure

One important role of periodic/aperiodic beam measurement report is to allow gNB to (1) monitor beam pair(s) quality and (2) adjust beam pair(s) by triggering beam management procedures when needed. So far, the triggering and configuration of the beam management procedure, i.e. P1/P2/P3 or U1/U2/U3, is a gNB decision without much UE direct input on which procedure is preferred. Beam changes typically are resulted from UE movement, rotation, blockage (e.g., hand, body or surrounding). UE can utilize various reference signals, measurements or even sensors to detect beam changes, and may figure out causes and preferable actions of such changes. Therefore, it is beneficial to allow UE assisting gNB to configure beam management procedure via, e.g., beam measurement report or other events.

Figure 2 illustrates the usefulness of the UE assisted beam management procedure. Assuming that due to the device rotation, the existing Rx beam quality deteriorates and the desirable Rx beam changes and requires an adjustment. From the gNB side, gNB may only observe the Rx beam quality deterioration from the UE measurement report. In order for gNB to discover the new optimal beam pair, gNB may configure P2 operation as shown in Figure 2. This will cause unnecessary delay and increase overhead in the beam management. On the other side, UE may quickly detect Rx beam degradation as the cause, e.g., switching to wide Rx beam width, and then UE can feed back this relevant information to help gNB decide a P3 procedure.

Figure 2 UE Request Beam Management Procedure

Similar as the measurement report, such UE report to assist the beam measurement procedure can be event triggered with predefined triggering conditions. One distinct aspect is that these UE reports should have implications on UE preferable beam management procedures.

Proposal 2: NR to support UE report to assist gNB configuring beam manage procedure (P1/P2/P3/U1/U2/U3)

FFS: (1) Detailed channels/mechanism to send such report from UE and (2) detailed events and triggering conditions

For both P1 and P3 procedure, UE needs to performance Rx beam sweep. The number of Rx beams that UE need to sweep may not be known at the gNB or may change dynamically. There can be multiple reasons for UE to prefer sweeping different number of Rx beam, for example, some of the antenna panel could be blocked by the hand, etc. Similarly, for UL beam management, UE Tx beam sweep is required for U1/U3 procedure. It is beneficial for UE to request the number of resource for P3/U3 (P1/U1) beam management procedure. Therefore, we propose

Proposal 3: NR to support UE request for the number of resources for P3/U3 (P1/U1) beam management procedure

# UE Request to Change the Configurations of Periodic and Aperiodic Beam Measurement/Reporting

As we discussed before, for periodic and semi-persistent beam measurement, periodic and semi-persistent resources should be configured. In order to ensure robust beam management and react to the beam change promptly, gNB has to configure frequent enough measurement resources.

Typically, UE has better knowledge about its mobility, channel condition, etc. Allowing UE to request beam measurement and reporting configurations can potentially improve the robust of the beam management while minimizing the overhead of the reference signals and measurement report. It is up to gNB to modify/activate/deactivate measurement configurations upon reception of UE request

For periodic and semi-persistent measurement, UE can request to change the periodicity as illustrated in Figure 3. In this example, UE is initially configured with beam measurement resource with short periodicity. The short periodicity is needed when UE is in high mobility, while longer periodicity is enough when UE has low mobility, or semi-static. Generally speaking, device is able to detect the mobility status change faster than gNB. Therefore, UE can request to change the periodicity of the reference signals, i.e. make it longer, in order to save the overhead as well as to reduce the measurement reports as shown in Figure 3.

Figure 3 UE request to change the periodicity of the periodic/semi-persistent beam measurement

For aperiodic event triggered measurement reporting, UE can request to activate and deactivate of certain events, as well as the configuration (TTT, hysteresis, offset etc.) of the event. Figure 4 illustrates that UE can request to active the new event.

Figure 4 UE request to activate/deactivate event triggered measurement reporting and its configurations

Proposal 4: NR to consider supporting of UE to request the preferred configuration of periodic and Aperiodic beam measurement/reporting

# Conclusions

Beam measurement and reporting is one the most important steps of beam measurement procedure. In order to ensure the robust and efficient beam management, beam measurement and reporting has the following requirement

- Minimize the overhead for both DL/UL reference signals and corresponding UL/DL measurement report

- Fast response to the mobility situation

With the above requirements in mind, this contribution discusses some of the techniques that could be used to improve the beam measurement/reporting response time and/or reduce the overhead of reference signals/UE reporting. We have the following proposals

Proposal 1: NR to support UE event based aperiodic beam measurement reporting.

FFS: the detailed events and configurations

Proposal 2: NR to support UE report to assist gNB configuring beam manage procedure (P1/P2/P3/U1/U2/U3)

FFS: (1) Detailed channels/mechanism to send such report from UE and (2) detailed events and triggering conditions

Proposal 3: NR to support UE request for the number of resources for P3/U3 (P1/U1) beam management procedure

Proposal 4: NR to consider supporting of UE to request the preferred configuration of periodic and Aperiodic beam measurement/reporting

# References

- 3GPP RAN1 #86 Meeting Chairman Notes, Gothenburg, Sweden, 22nd – 26th August 2016

- 3GPP RAN1 #86b Meeting Chairman Notes, Lisbon, Portugal, 10th – 14th October 2016

- 3GPP RAN1 NR#3 Meeting Chairman Notes, Nagoya, Japan, 18th – 21st, September 2017

- 3GPP RAN1 #90b Meeting Chairman Notes, Prague, Czech, 9th – 13th Oct 2017