3GPP TSG-RAN WG1 #91	                                  		                   R1-1720148

Reno, Nevada, USA, Nov. 27th, Dec. 1st 2017

Source:	Mitsubishi Electric

Title:	Low PAPR SFBC evaluations for PSSCH

Agenda Item:	6.2.3.3.2 Evaluation Results

Document for: 	Discussion/Decision

# Introduction

In the RAN1#90 meeting [1] the following working assumption was reached for PSSCH:

- For designing PSSCH, RAN1 assumes the use of two-port non-transparent transmit diversity

  - The use of non-transparent transmit diversity is configured.

  - Details, including diversity scheme, are FFS

  - Support of transmission and/or reception up to UE capability

    - Note: It is RAN1 understanding that requirements on capabilities can be set at regional level and are outside 3GPP scope

  - Send LS to RAN4 to ask their opinion about when non-transparent scheme for transmit diversity is used by Rel-15 UEs:

    - Impact on Rel-14 UEs of PSSCH-RSRP measurement accuracy

    - MPR for Rel-15 UEs

- Non-transparent Transmit diversity is not used in the following cases:

  - When communicating with Rel-14 UEs

  - When there is a high probability of resource collision with Rel-14 UEs

- Note: Some companies observe that the performance of MMSE-IRC receiver degrades when a non-transparent Transmit diversity scheme is used in interference limited scenarios with a dominant interferer

In RAN1#88b [2], simulation assumption agreements have been made to set up the simulation scenarios for transmit diversity candidates for PSSCH and PSCCH.

- For the design and feasibility of TxD schemes in Rel-15 PC5 operation, the CM increase per antenna over single antenna port transmission of Rel-14 is considered.

- For link level simulation (SNR vs. BLER) to investigate TxD gains for performance of V2X is applied for PSSCH and PSCCH.

| Parameters | Value |
| --- | --- |
| Carrier frequency | 6GHz |
| Antenna number | 2 x 2 |
| Channel model | LOS/NLOS in TR36.885 (linear polarization, half-lambda spacing) |
| Vehicle speed (absolute) | 15 km/h, 140km/h, 250km/h, 60 km/h optional |
| MCS | QPSK ½, 16QAM ½ |
| Payload size for PSSCH | 300 bytes, 190 bytes |

- Frequency offset modelled as in TR36.885

This contribution provides link level simulations showing the benefit of using PAPR-preserving SC-SFBC [3] for sidelink communications. Our companion contribution [4] provides detailed description of candidate schemes.

# Performance evaluations

In the contribution we investigate the following candidate techniques for sidelink communication: PAPR preserving SFBC, pre-DFT STBC [7], short delay CDD, long delay CDD, PVS with different precoders, SIMO reference. In [5], thorough performance comparison among Alamouti-based transmit diversity (PAPR preserving SFBC, split symbol STBC), short delay CDD, and different types of time-domain PVS (e.g. antenna port switching) is conducted in NR context in a large number of scenarios.

## Low PAPR SC-SFBC brief review

PAPR-preserving SFBC has already been discussed in the literature [3] and thoroughly described in our companion contribution [4]. It is also currently under investigation in an NR context for both PUSCH [5] and PUCCH [6] based on DFTsOFDM. Alamouti precoding is applied at subcarrier level, after DFT precoding, as it can be seen in Figure 1. Alamouti precoded symbols can be mapped onto non-adjacent subcarriersand  in such a way that PAPR is preserved, as in Figure 2. M is the number of allocated subcarriers (DFT precoder size) and p is an even integer, usually chosen as closest possible to M/2 to minimize the distance between subcarriers carrying Alamouti precoded pairs. This PAPR-preserving SFBC is called SC-SFBC [3]. Thorough description can be found in the companion contribution [4], where it is also shown that this scheme preserves the PAPR of DFTsOFDM.

Data block x(i)

Figure 1 - SFBC for DFTsOFDM

Figure 2 - Mapping of Alamouti pairs to subcarriers for PAPR preserving SFBC in DFTsOFDM

SC-SFBC has the following advantages:

- PAPR preserving for all constellation types, including BPSK pi/2 with or without frequency domain pulse shaping

- operating on single symbol DFTsOFDM,

- full diversity achieving,

- with low-complexity frequency-domain decoding at subcarrier level.

## PSSCH evaluation

In the following, SFBC is only given as a FER performance reference and is not proposed to be used for DFTsOFDM as it engenders around 1dB PAPR loss at CCDF target 10-4.

Figure 3 presents comparative performance of PAPR preserving SFBC, pre-DFT STBC, short delay CDD, long delay CDD, PVS with different precoders and SIMO transmission in a simulation setup in conformity with the current V2X agreements (see Annex), for QPSK ½ and 16QAM ½.

PAPR preserving SC-SFBC is implemented as described in [4]. A classical SFBC MMSE receiver is applied at the receiver side, since the only difference with classical SFBC is the subcarrier mapping order.

For pre-DFT STBC, Alamouti is applied within one single DFTsOFDM symbol before the DFT, among the two symbol halves. Both transmitter and receiver are more complex than in the SFBC case. The simulations were performed in an optimistic scenario for this scheme: the channel is considered static during the transmission of a DFTsOFDM symbol and Doppler effect is only visible between successive DFTsOFDM symbols. Therefore, orthogonality loss due to the channel change between the first and second symbol halves containing Alamouti pairs is neglected.

Short delay CDD considers a cyclic shift of 2 samples, and long delay CDD is considered with ½ OFDM symbol shift.

PVS with transparent DMRS (equivalent to antenna switching) has obvious drawbacks in high speed scenarios as explained in [4] and confirmed through simulation in [5], and will not be further investigated here.

We considered different time domain precoders for PVS with non-transparent DMRS, where channel tracking can be made at the receiver side.

| - QPSK ½, 1000ns, 10RB, 60kmph | - 16QAM ½, 300ns, 8RB, 60kmpk |
| --- | --- |

Figure 3 - Fc=6GHz, ITU UMi NLOS CDL, 0.1 PPM for TX and -0.1 PPM for RX w.r.t. UE’s sync reference

We have investigated the proposed schemes in a large number of scenarios. An example is given in Figure 3 and full results are summarized in Table 1and Table 2. Table 1and Table 2 summarize the gain of SC-SFBC (in dB, understood as SNR gain at target FER=10-1) over the other transmit diversity schemes for QPSK and 16QAM respectively. The color code in the tables is used to indicate:

- in orange, the cases where competing scheme has performance close to SC-SFBC (less than 0.5dB difference)

- in red, the cases where competing scheme has performance significantly worse than SC-SFBC

- in green, the cases where competing scheme has performance gain over SC-SFBC

Table 1 SNR gain at FER=10-1 of SC-SFBC over other diversity schemes for QPSK modulation

|  |  |  | pre-DFT-STBC | SFBC* | SD-CDD | LD-CDD | PVS 1 | PVS 2 | PVS 3 | SIMO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 130ns | 15kmph | 10RB | 0dB | -0.1dB | 1.5dB | 1dB | 1.5dB | 1dB | 1.1dB | 1.5dB |
|  |  | 16RB | -0.3dB | -0.1dB | 0.8dB | 1dB | 1.4dB | 0.9dB | 1.2dB | 1.6dB |
|  |  | 44RB | -0.6dB | -0.2dB | 0.1dB | 0.6dB | 0.9dB | 0.8dB | 0.8dB | 0.8dB |
|  | 60kmph | 10RB | -0.1dB | 0dB | 1.1dB | 1dB | 1.4dB | 0.8dB | 1.1dB | 1.6dB |
|  |  | 16RB | -0.3dB | -0.1dB | 0.5dB | 0.7dB | 1.3dB | 0.6dB | 1dB | 1.2dB |
|  |  | 44RB | -0.5dB | -0.2dB | 0dB | 0.5dB | 1.1dB | 0.6dB | 0.9dB | 1dB |
|  | 120kmph | 10RB | -0.1dB | 0.1dB | 1.5dB | 1.4dB | 1.8dB | 1.5dB | 1.4dB | 1.6dB |
|  |  | 16RB | -0.2dB | -0.1dB | 0.7dB | 1.4dB | 1.5dB | 1.2dB | 1.4dB | 1.6dB |
|  |  | 44RB | -0.6dB | -0.3dB | 0.6dB | 1.1dB | 1.6dB | 1.2dB | 1.3dB | 2.2dB |
| 300ns | 15kmph | 10RB | 0dB | 0dB | 1.3dB | 0.9dB | 1.1dB | 0.8dB | 1dB | 1.2dB |
|  |  | 16RB | -0.2dB | -0.1dB | 0.7dB | 0.6dB | 1dB | 0.5dB | 0.8dB | 1.1dB |
|  |  | 44RB | -0.6dB | -0.4dB | -0.2dB | 0.6dB | 1dB | 0.8dB | 0.8dB | 0.7dB |
|  | 60kmph | 10RB | 0dB | 0dB | 1dB | 0.8dB | 1.1dB | 0.9dB | 1dB | 0.8dB |
|  |  | 16RB | -0.1dB | -0.1dB | 0.4dB | 0.6dB | 1dB | 0.6dB | 0.7dB | 1dB |
|  |  | 44RB | -0.5dB | -0.3dB | 0.1dB | 0.5dB | 0.8dB | 0.6dB | 0.6dB | 0.8dB |
|  | 120kmph | 10RB | -0.1dB | -0.1dB | 1dB | 1.1dB | 1.3dB | 1dB | 1dB | 1.1dB |
|  |  | 16RB | -0.2dB | -0.1dB | 0.5dB | 1.1dB | 1.5dB | 0.9dB | 1.2dB | 1.3dB |
|  |  | 44RB | -0.4dB | -0.3dB | 0.4dB | 1.1dB | 1.4dB | 1.2dB | 1.2dB | 1.6dB |
| 1000ns | 15kmph | 10RB | 0dB | -0.1dB | 1dB | 0.8dB | 1.2dB | 0.7dB | 0.9dB | 1dB |
|  |  | 16RB | -0.3dB | -0.2dB | 0.5dB | 0.7dB | 1.1dB | 0.6dB | 0.8dB | 1.2dB |
|  |  | 44RB | -0.6dB | -0.6dB | -0.2dB | 0.6dB | 0.9dB | 0.8dB | 0.8dB | 0.6dB |
|  | 60kmph | 10RB | 0.1dB | 0dB | 1dB | 1dB | 1.3dB | 0.9dB | 1dB | 1dB |
|  |  | 16RB | -0.3dB | -0.3dB | 0.5dB | 0.6dB | 0.9dB | 0.5dB | 0.6dB | 0.7dB |
|  |  | 44RB | -0.5dB | -0.4dB | 0.1dB | 0.7dB | 0.8dB | 0.7dB | 0.7dB | 0.7dB |
|  | 120kmph | 10RB | 0.1dB | 0dB | 1dB | 1.4dB | 1.6dB | 1.2dB | 1.2dB | 1.3dB |
|  |  | 16RB | -0.2dB | -0.2dB | 0.4dB | 1dB | 1.2dB | 1dB | 1dB | 1.2dB |
|  |  | 44RB | -0.4dB | -0.4dB | 0.4dB | 0.8dB | 1.2dB | 1dB | 0.8dB | 1.2dB |

*SFBC has a PAPR loss not taken into account in the table here-above

Table 2 SNR gain at FER=10-1 of SC-SFBC over other diversity schemes for 16QAM modulation

|  |  |  | pre-DFT STBC | SFBC* | SD-CDD | LD-CDD | PVS 1 | PVS 2 | PVS 3 | SIMO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 130ns | 15kmph | 5RB | 0.14 dB | -0.11dB | 2.1 dB | 1.99 dB | 1.76 dB | 1.12 dB | 1.4 dB | 1.69 dB |
|  |  | 8RB | 0.21 dB | 0.09 dB | 1.74 dB | 1.79 dB | 2.17 dB | 0.86 dB | 1.8 dB | 2.41 dB |
|  |  | 22RB | 0.05 dB | -0.3 dB | 0.94 dB | 1.44 dB | 1.61 dB | 1.46 dB | 1.46 dB | 1.92 dB |
|  | 60kmph | 5RB | 0.29 dB | 0.1 dB | 2.02 dB | 1.98 dB | 1.84 dB | 1.05 dB | 1.66 dB | 1.95 dB |
|  |  | 8RB | 0.17 dB | -0.12dB | 2.07 dB | 1.87 dB | 2.29 dB | 1.24 dB | 1.78 dB | 2.41 dB |
|  |  | 22RB | 0.07 dB | -0.34dB | 1.54 dB | 1.99 dB | 2.03 dB | 1.43 dB | 1.49 dB | 2.79 dB |
|  | 120kmph | 5RB | 0.35 dB | -0.1 dB | >6dB | >6dB | >6dB | >6dB | >6dB | >6dB |
|  |  | 8RB | 0.7 dB | 0 dB | >6dB | >6dB | >6dB | >6dB | >6dB | >6dB |
|  |  | 22RB | - | - | - | - | - | - | - | - |
| 300ns | 15kmph | 5RB | 0.65 dB | -0.09dB | 1.9 dB | 1.51 dB | 1.45 dB | 0.82 dB | 1.35 dB | 1.8 dB |
|  |  | 8RB | 0.62 dB | -0.15dB | 2 dB | 1.7 dB | 1.62 dB | 1.04 dB | 1.65 dB | 1.84 dB |
|  |  | 22RB | 0.1 dB | -0.4 dB | 0.68 dB | 0.96 dB | 1.36 dB | 1 dB | 1.22 dB | 1.66 dB |
|  | 60kmph | 5RB | 0.64 dB | -0.04dB | 2.1 dB | 1.69 dB | 1.53 dB | 1.06 dB | 1.31 dB | 1.84 dB |
|  |  | 8RB | 0.91 dB | -0.19dB | 1.73 dB | 1.84 dB | 1.92 dB | 1.32 dB | 1.64 dB | 1.94 dB |
|  |  | 22RB | 0.27 dB | -0.82dB | 1.16 dB | 1.42 dB | 1.73 dB | 1.14 dB | 1.31 dB | 2.03 dB |
|  | 120kmph | 5RB | 1.41 dB | 0 dB | >6dB | >6dB | >6dB | >6dB | >6dB | >6dB |
|  |  | 8RB | 2.19 dB | -0.39dB | >6dB | >6dB | >6dB | >6dB | >6dB | >6dB |
|  |  | 22RB | - | - | - | - | - | - | - | - |
| 1000ns | 15kmph | 5RB | 1.01 dB | -0.09dB | 1.77 dB | 1.48 dB | 1.72 dB | 0.79 dB | 1.38 dB | 2.1 dB |
|  |  | 8RB | 0.45 dB | -0.25dB | 1.4 dB | 1.32 dB | 1.53 dB | 0.74 dB | 1.21 dB | 1.73 dB |
|  |  | 22RB | 0.1 dB | -0.55dB | 0.69 dB | 0.93 dB | 1.24 dB | 0.92 dB | 1.11 dB | 1.35 dB |
|  | 60kmph | 5RB | 1.32 dB | 0.04 dB | 2.14 dB | 1.8 dB | 1.84 dB | 1.07 dB | 1.56 dB | 1.9 dB |
|  |  | 8RB | 1.34 dB | -0.27dB | 1.7 dB | 1.69 dB | 1.92 dB | 1.72 dB | 1.32 dB | 1.93 dB |
|  |  | 22RB | 0 dB | -0.7 dB | 1.1 dB | 1.41 dB | 1.77 dB | 1.32 dB | 1.23 dB | 1.99 dB |
|  | 120kmph | 5RB | 5.6 dB | -0.1 dB | >6dB | >6dB | >6dB | >6dB | >6dB | >6dB |
|  |  | 8RB | 4.38 dB | -0.46dB | >6dB | >6dB | >6dB | >6dB | >6dB | >6dB |
|  |  | 22RB | - | - | - | - | - | - | - | - |

*SFBC has a PAPR loss not taken into account in the table here-above

CDD is easy to implement and can be transparent to the receiver in the case of short delay CDD (SD-CDD). Nevertheless, CDD has very variable performance depending on the applied delay, and an optimized delay (depending on the channel profile, allocation size, modcod) cannot be applied in an open loop scenario. Also, since CDD recovers frequency diversity through coding, CDD performance degrades with less strong coding and in scenarios where enough diversity is already present (frequency selective channels/frequency hopping transmission where the extra diversity obtained through CDD is limited, high speed where the extra diversity wrt to already present time diversity has limited gain, etc).

SC-SFBC has similar performance with classical SFBC, the penalty due to precoding onto non-adjacent subcarriers being small even on very dispersive channels and with large band allocations. Considering the fact that SC-SFBC is PAPR preserving, while SFBC brings up to 1dB of PAPR loss, SC-SFBC is preferred.

Pre-DFT STBC has better performance than the non-Alamouti schemes. Pre-DFT STBC performance is close to SC-SFBC and may even have a slight advantage when QPSK is used, but displays performance loss (significant, even up to several dBs in some cases) when 16QAM is used. This scheme is rather sensitive to long channels and medium/high modulation orders because of the interference between the two symbol halves involved in Alamouti precoding. Given that pre-DFT STBC is more complex at both receiver and transmitter side without providing any significant gain over SC-SFBC, SC-SFBC is preferred.

Observation: Full diversity Alamouti schemes are more robust than CDD and PVS and provide significantly better performance..

Observation: The PAPR preserving SC-SFBC has close FER performance with SFBC without any PAPR loss and is thus preferred to SFBC.

Observation: The PAPR preserving SC-SFBC is less complex than pre-DFT STBC and has similar (QPSK) or better (16QAM) performance.

# Conclusion

Observation: Full diversity Alamouti schemes are more robust than CDD and PVS and provide significantly better performance.

Observation: The PAPR preserving SC-SFBC has close FER performance with SFBC without any PAPR loss and is thus preferred to SFBC.

Observation: The PAPR preserving SC-SFBC is less complex than pre-DFT STBC and has similar (QPSK) or better (16QAM) performance.

Proposal: SC-SFBC Alamouti-based transmit diversity is supported for PSSCH with DFTsOFDM

# References

### Chairman’s notes, RAN1 #90, Prague, Czech Republic, August 2017.

### Chairman’s notes, RAN1 #88b, Spokane, April 2017

### C. Ciochina, D. Castelain, D. Mottier and H. Sari, “A Space-Frequency Block Code for Single-Carrier FDMA,” Electronics Letters 44(11):690 - 691 February 2008

### R1-1720147, Low PAPR SFBC for V2X transmit diversity, Mitsubishi Electric, RAN1#91, Reno, November 2017.

### R1-1715555, UL diversity transmission for DFTsOFDM, Mitsubishi Electric, RAN1 NR AH#03, Nagoya, September 2017.

### R1-1716237, Transmit diversity for DFTsOFDM-based PUCCH in long duration, Mitsubishi Electric RCE, RAN1 NR AH#03, Nagoya, September 2017.

### R1-1713389, On UL diversity transmission scheme, RAN1#90, Prague, August 2017.

# Annex B: Simulation parameters

| Assumptions , | Value |  |  |
| --- | --- | --- | --- |
| Carrier frequency | 6 GHz |  |  |
| Number of antennas | 2x2 |  |  |
| Numerology | BW=10MHz, N_FFT=1024, Δf=15kHz, Fs=15.36MHz |  |  |
| Data bandwidth | 18RB for QPSK, 12RB for 16QAM (payload 300bytes) |  |  |
| Payload | 190 bytes | 300 bytes | 800 bytes |
| MCS /allocation | QPSK ½ - 10RB 16QAM ½ - 5RB | QPSK ½ - 16RB 16QAM ½ - 8RB | QPSK ½ - 44RB 16QAM ½ - 22RB |
| Channel coding | LTE Turbo code |  |  |
| HPA | Rapp model, , IBO=-11.6dB |  |  |
| Channel estimation | Realistic, LTE pilots |  |  |
| Channel model | ITU UMi NLOS CDL 129ns with delay spread scaled to 130ns/300ns/1000ns |  |  |
| CFO | Case 1 + Case B: - +0.1 PPM for TX and -0.1 PPM for RX w.r.t. UE’s sync reference. - +0.05 PPM for TX’s reference and -0.05 PPM for RX’s reference w.r.t. the absolute frequency. |  |  |
| Speed | 15/60/120 kmph |  |  |
| Receiver type | L-MMSE |  |  |
| SD-CDD delay | 2 samples |  |  |
| PVS vectors | [[1 1] [1 -1]], [[1 1] [-1 -1]], [[1 1 1 1] [1 j -1 -j]] |  |  |