3GPP TSG RAN WG1 Meeting #91 		          R1-1720165

Reno, USA, November 27th – December 1st, 2017

Source:	CATT

Title:	Discussion on shorten TTI in PC5

Agenda Item:	6.2.3.6

Document for:	Discussion and Decision

Introduction

In RAN #77 meeting [1], the following conclusion was achieved:

Conclusion: guidance from RAN to RAN1: short TTI work is deprioritized but not precluded, look into other means of latency reduction than short TTI.

Based on the agreements of RAN1 #89 for sTTI scheme evaluations [2], the companion contribution [3] is presented to provide the simulation performance gain of Rel-14 and Rel-15 UEs with sTTI scheme in the mixed scenarios. In this contribution, the potential solutions of latency reduction and other key technical issues for shorten TTI in PC5 are discussed.

This is a re-submission of R1-1717795 [4].

Discussion

Impact on LTE R14 V2V sensing and resource selection

As discussed in the CATT’s contribution in RAN1 #90 [5], the following potential solution is suggested as the baseline solution for supporting sTTI scheme in the same resource pool of Rel-14 UEs:

Figure 1 The suggested solution for supporting sTTI mechanism

Proposal 1: Rel-15 UE transmitting PSCCH with only sTTI SA is suggested as the baseline sTTI mechanism.

AGC Schemes

Because of the unacceptable PRR performance with puncturing the AGC and GP as two symbols in one slot [6], the legacy AGC and GP scheme cannot be directly applied to the Rel-15 sTTI.

In order to decrease the overhead of the Rel-15 sTTI scheme and solve the near-far problem of AGC, the Shared AGC [7] and AGC/GP in one symbol [4] solutions has been proposed. In [7], it can be observed that Shared AGC and AGC/GP in one symbol both have similar PRR performance for Rel-14 and Rel-15 UEs in low density and high density scenarios. Considering the clipping noise and satuature is more sensitive to the receiving power, Shared AGC scheme is considered as the AGC scheme in system level simulation in the companion contribution [3].

Proposal 2: Considering the overhead and the performance of the system level simulation, shared AGC scheme is considered as the AGC scheme.

sTTI gain

Because the granularity of the transmitting time is assumed as one slot in sTTI and configured as one subframe for TTI, for one transmission opportunity, the transmitting time of sTTI UE is shorter than TTI UE. Thus, the performance loss due to half-duplex is reduced and the accuracy of the sensing results is improved by sTTI scheme.

Proposal 3: Because the transmitting time of sTTI UE is shorter than TTI UE, the performance loss due to half-duplex is reduced and the accuracy of the sensing results is improved by sTTI scheme.

Figure 2 The receiving power from Rel-15 UE data and Rel-14 UE data with same message size

As shown in Figure 2, with the Rel-15 UE with only sTTI SA, power density of Rel-15 UE is lower than the Rel-14 UE at the same receiving distance with the message size and MCS configurations. Thus, the co-channel interference from Rel-15 is lower than from Rel-14. Meanwhile, IBE is relatively lower than the co-channel interference and have less impact on the system performance.

Based on the system level simulation results of companion contribution [3], with the increasing proportion of the Rel-15 sTTI UEs in Mixed Scenario 2, the PRR of legacy Rel-14 UE can also be improved with the replacement of the lower interference from the Rel-15 sTTI UEs not the legacy Rel-14 UEs. Meanwhile, from the system view, the PRR of Rel-15 sTTI UE is also improved by the alleviation of the co-channel interference from the Rel-15 UE.

Proposal 4:

- With the increasing proportion of the Rel-15 sTTI UEs in Mixed Scenario 2, the PRR of Rel-15 sTTI UE is improved by the alleviation of the interference from the Rel-14 UE.

- Meanwhile, with the replacement of the lower interference from the Rel-15 sTTI UEs not the legacy Rel-14 UEs, the PRR of legacy Rel-14 UE in Mixed Scenario 2 can also be improved.

Conclusion

In this contribution, the design of shorten TTI in PC5 are discussed. Particularly, we have following observations and proposals:

Proposal 1: Rel-15 UE transmitting PSCCH with only sTTI SA is suggested as the baseline sTTI mechanism.

Proposal 2: Considering the overhead and the performance of the system level simulation, shared AGC scheme is considered as the AGC scheme.

Proposal 3: Because the transmitting time of sTTI UE is shorter than TTI UE, the performance loss due to half-duplex is reduced and the accuracy of the sensing results is improved by sTTI scheme.

Proposal 4:

- With the increasing proportion of the Rel-15 sTTI UEs in Mixed Scenario 2, the PRR of Rel-15 sTTI UE is improved by the alleviation of the interference from the Rel-14 UE.

- Meanwhile, with the replacement of the lower interference from the Rel-15 sTTI UEs not the legacy Rel-14 UEs, the PRR of legacy Rel-14 UE in Mixed Scenario 2 can also be improved.

References

  - Draft Meeting Report, 3GPP TSG RAN Meeting #77, RAN Chairman’s Notes;

  - Draft Minutes 3GPP TSG RAN WG1 Meeting #89, RAN1 Chairman’s Notes;

  - R1-1720166, “Evaluations for shorten TTI in PC5”, 3GPP RAN1 Meeting #91, CATT;

  - R1-1717795, “Discussion on shorten TTI in PC5”, 3GPP RAN1 Meeting #90bis, CATT;

  - R1-1712345, “Discussion on shorten TTI in PC5”, 3GPP RAN1 Meeting #90, CATT;

  - R1-1714597, “Evaluations for shorten TTI in PC5”, 3GPP RAN1 Meeting #90, CATT;

  - R1-1704688, “Considerations on Support of Short TTI for LTE V2V Sidelink Communication”, 3GPP RAN1 Meeting #88bis, Intel;