3GPP TSG RAN WG1 Meeting 91                                            R1-1720366

Reno, USA, November 27th – December 1st , 2017

Agenda Item:		    7.8

Source:			Samsung

Title:		   DFT-based IGMA scheme

Document for:		Discussion and Decision

# Introduction

In RAN #75 meeting, the study item on non-orthogonal multiple access for NR has been approved [1].

This study will further progress on the evaluation of non-orthogonal multiple access schemes focusing on uplink, and provide recommendation on the non-orthogonal multiple access scheme(s) to be specified later.

Agreements, observations and evaluation assumption in Rel-14 study shall be the starting point. The detailed objectives are to study the following:

1 non-orthogonal multiple transmission scheme

- Transmitter side signal processing schemes for non-orthogonal multiple access [RAN1]:

  - Modulation and symbol level processing, including spreading, repetition, interleaving, new constellation mapping, etc.

  - Coded bit level processing including interleaving and/or scrambling, etc.

  - Symbol to resource element mapping, sparse or not, etc.

  - Demodulation reference signal. Other signal is not excluded.

- Receivers for non-orthogonal multiple access: [RAN1, RAN4]

  - MMSE receiver, successive/parallel interference cancellation (SIC/PIC) receiver, joint detection (JD) type receiver, combination of SIC and JD receiver, or other receivers

  - The study should consider performance, receiver complexity, etc.

- Procedures related to the non-orthogonal multiple access  [RAN1]

  - UL transmission detection

  - HARQ, including transmission scheme, feedback scheme, and combining scheme

  - Link adaptation MA signature allocation/selection

  - Synchronous and asynchronous operation

  - Adaptation between orthogonal and non-orthogonal multiple access

- Link and system level performance evaluation or analysis for non-orthogonal multiple access continued from performance metrics identified from Rel-14. The benchmark for comparison is OFDM contention based multiple access. Realistic modelling of Tx/Rx impairment including potential PAPR issue, channel estimation error, power control accuracy, collision, etc. should be considered. [RAN1]

  - Traffic model and Deployment scenarios of eMBB (small packet), URLLC and mMTC

  - Device power consumption

  - Coverage (link budget)

  - Latency and signalling overhead

  - BLER reliability, capacity and system load

  - Physical abstraction (link-to-system mapping model)

Note: targeting common solution for mMTC, URLLC and eMBB small packet.

In this contribution, the PAPR analysis targeting on mMTC scenario is given and then the low PAPR IGMA is proposed.

# Discussion

The demand of supporting massive Machine-Type Communication (mMTC) is proposed for NR. To improve the capacity of the multiple access technology, some Non-orthogonal Multiple Access (NoMA) technologies have been proposed, such as Sparse Code Multiple Access (SCMA), Pattern Defined Multiple Access (PDMA) and Multi-user Shared Access (MUSA), and Interleave-Grid Multiple Access (IGMA).

Even though NoMA could increase the number of served UEs, the mMTC devices usually have special requirements on power consumption and coverage. Generally, the mMTC devices are low-cost and require long battery life [2]. The UE is required to last for 15 years on the premise of satisfying a certain amount of uplink and downlink data transmission, without charging. Meanwhile, coverage of mMTC scenario has also extended, i.e., MCL requirement is 164 dB.  Considering that the Peak-to-Average Power Ratio (PAPR) of the signals has great influence on the power consumption. Moreover, high PAPR requires large power back-off, which will lead to lower average power and result in bad coverage. The PAPR performance of proposed scheme is worth studying.

## PAPR analysis

Most of the existing non-orthogonal multiple access modes are combined directly with Orthogonal Frequency Division Multiplex (OFDM), like SCMA, PDMA, so they generally have the problem of high PAPR and high terminal power consumption. On the other hand, the single-carrier frequency division multiple access (SC-FDMA), i.e., DFT-spread-OFDM (DFT-s-OFDM), is used to reduce PAPR in LTE system.

Compared with OFDM, in SC-FDMA, DFT precoding and the subcarrier mapping, are performed before the IDFT. Generally, the points of DFT should not be greater than the points of IDFT. If the points of DFT are equal to the points of IDFT, the transmitted data is consistent with the original data stream. The PAPR of the original data stream can be maintained. If the points of DFT are less than the points of IDFT and the subcarrier mapping module selects localized mapping method, the output data still can be maintained in a certain single-carrier structure, and the PAPR thereof still can be reduced when compared with OFDM. Moreover, the interleaved FDMA (IFDMA) can be regard as a special form of SC-FDMA. The difference from localized mapping SC-FDMA lies in that the subcarrier mapping has an equal interval, e.g., the symbols are mapped to subcarriers with an interval m as shown in Fig. 1.

Fig. 1 The subcarrier mapping modes for SC-FDMA (localized) and IFDMA (distributed)

It can be proved that the signal passing through IFDMA is the periodic repetition of the input signal with scaling-down. Therefore, a same PAPR as the input signal can be maintained for the output signal. If the horizontal amplitude modulation is performed on the input signal, for example, QPSK or PSK, the output signal can obtain a PAPR of 0dB. The power consumption of the output signal is greatly reduced.

Observation 1: the PAPR value with IFDMA could reach as low as 0dB.

Thus, in next section, the combination of IGMA and low PAPR configuration is considered. It could show that the PARP has significantly been reduced without sacrificing the BLER performance.

## Low PAPR IGMA

As discussed before, applying DFT precoding (SC-FDMA-like manner) could help reducing the PAPR. Thus, it is natural to study the combination of SC-FDMA and IGMA to eventually reduce the PAPR of IGMA. As shown in Fig. 2, apart from normal IGMA operations [3], one module of DFT-precoding is added after the symbol-level interleaving. The resource mapping is functioned as the subcarrier mapping. The distributed mapping manner (like the IFDMA) could be applied to further reduce the PAPR of IGMA.

Fig. 2 One example of low PAPR IGMA (DFT-based IGMA)

As illustrated in Fig. 3, the data sequence after DFT precoding is mapping on the subcarriers with equal distance. The original symbol sequence passes through symbol level interleaving. In addition to randomizing the symbol orders, the sparsity could also be considered in this step. Then the interleaved sequence goes through DFT operation without changing its length, which is equal to the points of DFT. After mapping to the subcarriers with equal distance manner, the sequence will go to carrier modulation, e.g., IDFT.

Fig. 3 Example of grid mapping in low PAPR IGMA

Regarding the purpose of distinguishing different UEs, the credentials in normal IGMA such as bit level interleaver and/or grid mapping pattern could still be applied. Furthermore, Different distance on subcarrier mapping could also be used to help to differentiate UEs. Note that now the Cell-edge UEs and cell-center UEs can be multiplexed in FDM manner. Considering the impact to the receiver, in addition to the advanced receiver like chip-by-chip MAP receiver, the modified ESE detector could be used for complexity reduction.

# Conclusion

In this contribution, the PAPR analysis targeting on mMTC scenario is given and then the low PAPR IGMA is discussed. The following are proposed:

Observation 1: the PAPR value with IFDMA could reach as low as 0dB.

# References

RP-170829, “New Study Item proposal: Study on Non-orthogonal Multiple Access for NR”, ZTE, CATT, Intel, Samsung, March, 2017

TR38.913, “Study on scenarios and requirements for next generation access technologies”.

R1-163992, “Non-orthogonal Multiple access candidate for NR”, Samsung