3GPP TSG RAN WG1 Meeting #91	   R1-1720488

Reno, USA, November 27th – December 1st, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluations of transmit diversity schemes for V2X

Agenda item:	6.2.3.3.2

Document for:	Discussion and Decision

# 1	Introduction

One of the objectives of the “V2X phase 2 based on LTE” work item is to study the feasibility and gains of PC5 operation with transmit diversity‎ with details as follows [1].

2. Study the feasibility and gain of PC5 operation with Transmit Diversity, assuming this PC5 functionality would co-exist in the same resource pools as Rel-14 functionality and use the same scheduling assignment format (which can be decoded by Rel-14 UEs), without causing significant degradation to Rel-14 PC5 operation compared to that of Rel-14 UEs, and specify this PC5 functionality if justified. [RAN1, RAN2, RAN4].

In RAN1#90, the potential transmit diversity schemes and evaluation results were discussed and the following working assumption were reached [2].

Working Assumption (may be revisited based on RAN4 response):

- For designing PSSCH, RAN1 assumes the use of two-port non-transparent transmit diversity

  - The use of non-transparent transmit diversity is configured.

  - Details, including diversity scheme, are FFS

  - Support of transmission and/or reception up to UE capability

    - Note: It is RAN1 understanding that requirements on capabilities can be set at regional level and are outside 3GPP scope

  - Send LS to RAN4 to ask their opinion about when non-transparent scheme for transmit diversity is used by Rel-15 UEs:

    - Impact on Rel-14 UEs of PSSCH-RSRP measurement accuracy

    - MPR for Rel-15 UEs

- Non-transparent Transmit diversity is not used in the following cases:

  - When communicating with Rel-14 UEs

  - When there is a high probability of resource collision with Rel-14 UEs

- Note: Some companies observe that the performance of MMSE-IRC receiver degrades when a non-transparent Transmit diversity scheme is used in interference limited scenarios with a dominant interferer

In this paper, we discuss the evaluation results for some candidate transmit diversity schemes with non-transparent two antenna ports. These transmit diversity schemes are discussed in detail in a companion paper [3].

# 2	Transmit diversity schemes

For PSSCH, as shown above, it was agreed as working assumption that RAN1 assumes the use of two-port non-transparent transmit diversity for designing PSSCH and details of transmit diversity schemes are FFS. The companion paper [3] discusses the potential two-port non-transparent transmit diversity schemes, including STBC, SFBC, PVS-T and Comb-data diversity. These transmit diversity schemes are evaluated in this paper as shown in next section.

# 3	Link level evaluation results

As discussed above, four candidate transmit diversity schemes are evaluated for PSSCH, i.e. STBC, SFBC, PVS-T and Comb-data diversity. The link level evaluation results of these transmit diversity schemes are introduced in this section. Two parts of the link level evaluation results are provided: evaluation results in noise-limited scenarios and results in interference-limited scenario, which are discussed in the following subsections respectively.

## 3.1 Noise-limited evaluation results

The candidate transmit diversity schemes with different DMRS designs are evaluated in this part. In the simulations, the 2-port DMRS with CDM CS and DMRS with different roots are simulated. The evaluation conditions for PSSCH with transmit diversity in noise-limited scenarios are shown in Table 1 in appendix A. The evaluation results are shown in Figure 1 for 30kmph (DMRS with CDM CS and DMRS with different roots) and 280kmph (DMRS with CDM CS and DMRS with different roots) respectively.

From the PSSCH evaluation results in noise-limited scenarios, we can get the following observations

Observation 1: In low mobility, SFBC and STBC achieve close performance and both outperform other transmission schemes by at least 2dB (at BLER 0.01).

Observation 2: In high mobility, STBC performance degrades greatly and SFBC outperforms the other transmission schemes by at least 3dB (at BLER 0.01).

Observation 3: The 2-port DMRS with different roots brings performance loss about 1.0dB compared with orthogonal CDM CS DMRS. With 2-port DMRS with different roots, SFBC still outperforms other transmission schemes greatly.

- 30kmph, DMRS CDM-CS for 2 ports                      (b)30kmph, DMRS w. diff. roots for 2 ports

(c) 280kmph, DMRS CDM-CS for 2 ports                      (d) 280kmph, DMRS w. diff. roots for 2 ports

Figure 1: PSSCH evaluation results

## 3.2 Interference-limited evaluation results

The evaluations in interference-limited case introduced in this subsection aim to clarify the concerns of the potential impact of transmit diversity PSSCH transmissions on the receiving performance of the legacy Rel-14 UEs as the Rel-15 PSSCH transmissions may co-exist with Rel-14 PSSCH in the same resource pool. In the simulations, the transmit diversity schemes of SFBC and Comb-data diversity are evaluated and compared with the single-port PSSCH interference. In principle, STBC and PVS-T have similar impact as SFBC and Comb-data diversity, respectively to the legacy UEs. Thus they are not evaluated in the simulations.

As per discussions in previous meetings, in the simulations the useful signal transmitter and the receiver are assumed to be both Rel-14 UEs and there is one or more interfering transmissions of PSSCH with transmit diversity. The useful PSSCH signal power to noise power ratio is set to 25dB and the performance metric is BLER vs. SINR.

As agreed in RAN1#89, MMSE-MRC receiving algorithm is the baseline for Rel-14 UEs. Additionally, we also provided evaluation results for advanced MMSE-IRC receiving algorithm. In the applied MMSE-IRC, the data detection is performed per PRB based on the estimated covariance matrix for interference plus noise. The estimation of covariance matrix is based on received DMRS, channel estimation of useful channel and the original useful DMRS signal.

The detailed evaluation conditions are listed in Table 2 in appendix A. The evaluation results are shown in Figure 2 with figure (a), (b) and (c) corresponding to one, two and four interferers. In case of multiple interferers, it is assumed that different interfering signals have the same average received power.

From the evaluation results, the following observations can be obtained

Observation 4: In the interference-limited case, with the baseline MMSE-MRC receiver the PSSCH with transmit diversity of SFBC or Comb-date diversity has almost the same impact as Rel-14 interfering PSSCH transmissions on receiving performance of Rel-14 UEs independent of single interferer or multiple interferers.

Observation 5: With MMSE-IRC, the impact of the interference of SFBC, Comb-data diversity and Rel-14 PSSCH on the legacy UE performance becomes smaller in turn in case of single dominate interferer. With increase of interferer number, the performance gap between different interference types become much smaller.

- Single interferer                                         (b) Two interferers

(c) Four interferers

Figure 2: Interference-limited results (interference to legacy UE), QPSK, 30kmph

# 4	Evaluations of potential RSRP loss on legacy UEs

As discussed in the companion paper [3], there are two possible DMRS designs for non-transparent two antenna ports, i.e. CDM DMRS with cyclic shifting and FDM DMRS with comb multiplexing for two antennas. It is noted that for CDM DMRS design, there is at least 3dB PSSCH-RSRP measurement loss for the legacy UEs and the potential impact of the 3dB loss on the sensing and resource (re)selection procedures of legacy UE in mode 4 is a technical concern and need to be clarified. To this end, some system level simulations are made to evaluate the possible impact of the 3dB PSSCH-RSRP measurement loss on the legacy UE performance.

The system level simulations are made for scenarios of freeway and urban at speed of 70kmph and 15kmph respectively. In the simulations with mixed UEs, half of the vehicle UEs are legacy UEs and the other half are UEs that transmit PSSCH with diversity scheme of SFBC. The performance metric is the average PRR for the packets transmitted by the legacy UEs, as the purpose is to evaluate the impact of RSRP loss on legacy UEs. The simulation conditions are listed in table 3 in appendix and the simulation results are shown in Figure 3. Note that in the figure, the curve with legend “w/o RSRP loss” corresponds to the case that the full PSSCH-RSRP value is ideally measured by the legacy UE. This ideal case is only for purpose of comparison.

From the simulation results, the following observation can be obtained:

Observation 6: For non-transparent two-port with CDM DMRS, there is at least 3dB PSSCH-RSRP measurement loss for legacy UEs. The system evaluations showed that the 3dB PSSCH-RSRP measurement loss has little impact on the performance of legacy UEs that uses sensing based resource (re)selection.

The potential reasons may be that the PSSCH-RSRP only impacts the resource exclusion operations for the candidate resource set and the more significant resource ranking and reporting of the available resources is based on the S-RSSI measurements which have no potential problems with the CDM DMRS design.

Figure 3: evaluations of potential impact of RSRP loss on legacy UEs

# 5	Conclusion

In this paper, the evaluation results for PSSCH with some candidate non-transparent two-port transmit diversity schemes are presented. Based on the evaluation results, the following observations are obtained.

Observation 1: In low mobility, SFBC and STBC achieve close performance and both outperform other transmission schemes by at least 2dB (at BLER 0.01).

Observation 2: In high mobility, STBC performance degrades greatly and SFBC outperforms the other transmission schemes by at least 3dB (at BLER 0.01).

Observation 3: The 2-port DMRS with different roots brings performance loss about 1.0dB compared with orthogonal CDM CS DMRS. With 2-port DMRS with different roots, SFBC still outperforms other transmission schemes greatly.

Observation 4: In the interference-limited case, with the baseline MMSE-MRC receiver the PSSCH with transmit diversity of SFBC or Comb-date diversity has almost the same impact as Rel-14 interfering PSSCH transmissions on receiving performance of Rel-14 UEs independent of single interferer or multiple interferers.

Observation 5: With MMSE-IRC, the impact of the interference of SFBC, Comb-data diversity and Rel-14 PSSCH on the legacy UE performance becomes smaller in turn in case of single dominate interferer. With increase of interferer number, the performance gap between different interference types become much smaller.

Observation 6: For non-transparent two-port with CDM DMRS, there is at least 3dB PSSCH-RSRP measurement loss for legacy UEs. The system evaluations showed that the 3dB PSSCH-RSRP measurement loss has little impact on the performance of legacy UEs that uses sensing based resource (re)selection.

# References

- RP-171740, WID “V2X phase 2 based on LTE”, 3GPP RAN#77.

- RAN1#90 chairman notes.

- R1-1720487, “Discussions on Transmit diversity schemes and DMRS for PSSCH”, RAN1-91.

# Appendix A

In this section, we provide simulation parameter setting of the link level simulations in this paper.

Table 1: Evaluation conditions for PSSCH in noise-limited scenario

| Parameters | Values |
| --- | --- |
| Carrier frequency | 6.0 GHz |
| Bandwidth | 10MHz |
| V2V packet size | 300 bytes |
| Resource allocation | 20 PRBs for QPSK, 12 PRBs for 16QAM Single transmission |
| MCS | QPSK, 16QAM |
| Transmit diversity | STBC (with the first symbol as orphan) SFBC, PVS-T and Comb-data diversity Refer to [3] for details. |
| DMRS | Two 2-port DMRS designs are simulated, one is CDM-CS DMRS and the other is CDM with different roots. |
| Antenna configurations | 2 TX antenna and 2 RX antennas, 1 TX antenna for comparison purpose |
| Channel model | ITU UMi NLOS with dual-mobility |
| Velocity | Relative velocities of 30kmph, 280kmph |
| Frequency offsets | Fixed 1.8kHz |
| Initial timing offset | 1us |
| Channel estimation method | Practical, including Half-symbol based timing/frequency synchronization, LMMSE channel estimation in frequency and linear interpolation in time |
| Data detection | Alamouti decoding for SFBC and STBC, MMSE-MRC for PVS-T, Comb-data diversity and single-antenna transmission. |

Table 2: Evaluation conditions for PSSCH in interference-limited scenario

| Parameters | Values |
| --- | --- |
| Carrier frequency | 6.0 GHz |
| Bandwidth | 10MHz |
| V2V packet size | 300 bytes * |
| Resource allocation | 20 PRBs for QPSK, Single transmission * |
| MCS | QPSK * |
| Transmit diversity for interfering transmitter | Rel-14 single-antenna transmission SFBC and Comb-data diversity |
| DMRS for interfering transmitter | For SFBC, CDM DMRS used with the legacy DMRS sequence on the first port. For Comb-data diversity, Comb FDM DMRS Useful signal DMRS and interfering DMRS have different root indexes |
| Antenna configurations | 2 TX antenna and 2 RX antennas, 1 TX antenna for comparison purpose |
| Channel model | ITU UMi NLOS with dual-mobility |
| Velocity | Relative velocities of 30kmph |
| Timing and frequency offsets | 0 |
| Channel estimation method | Practical, including Half-symbol based timing/frequency synchronization, LMMSE channel estimation in frequency and linear interpolation in time |
| Data detection | MMSE-MRC and MMSE-IRC |

Note: * is for both useful signal and interfering signal.

Table 3: System Level Evaluation conditions

| Parameters | Values |
| --- | --- |
| Deployment scenario | R14 V2V methodology freeway and urban Vehicle speed = 70 km/h for freeway and 15kmph for urban MTAD = 2.5s |
| Traffic model | 4 x 190 byte + 1 x 300 byte; 100 ms period |
| Number of packet TTIs | 1 |
| TTI structure | LTE Rel-14 legacy TTI structure |
| adjacencyPSCCH-PSSCH | Non-adjacent |
| Frequency resource allocation for PSSCH | 20 PRBs |
| Resource pool for PSSCH | 40 PRBs |
| Resource (re)selection | R14 sensing-based resource (re)selection procedure with the following parameters: t1=4, t2=20, Cresel=1, probResourceKeep=0.8 |
| Detection method of legacy UEs | MMSE-MRC |