3GPP TSG RAN WG1 Meeting 91			R1-1720489

Reno, USA, November 27th – December 1st, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	On Resource pool sharing between mode-3 and mode-4

Agenda item:	6.2.3.4

Document for:	Discussion and Decision

# Introduction

The 3GPP RAN work item “V2X phase 2 based on LTE” contains the following objective (RP-171740):

1.      Specify solutions for the following PC5 functionalities, which can co-exist in the same resource pools as Rel-14 functionality and use the same scheduling assignment format (which can be decoded by Rel-14 UEs), without causing significant degradation to Rel-14 PC5 operation compared to that of Rel-14 UEs:

…

d) Radio resource pool sharing between UEs using mode 3 and UEs using mode 4

This topic has not been treated in previous meetings.

# Discussion

## Background and Motivation

Up to now sharing a V2X resource pool between mode 3 and mode 4 has not been explicitly supported; a resource pool was intended either for mode 3 only or for mode 4 only.

Let us first examine the motivation and relevant scenarios for this new objective:

A possible motivation for this new objective is that in a deployment it may be required to support both mode 3 and mode 4 transmissions, e.g. in the following scenarios:

- some UEs only support mode 3 (capability parameter v2x-eNB-Scheduled-r14 is set, while ue-AutonomousWithFullSensing-r14 is not set) and some UEs only support mode 4;

- at the boundary of network coverage, the UEs inside network coverage can use mode 3, while those UEs outside network coverage can only use mode 4;

- UEs with a subscription with a mobile network operator can use mode 3, while those UEs without such a subscription can only use mode 4.

One option to support both modes is to use separate and non-overlapping resource pools, that is one pool dedicated to mode 3 and another pool dedicated to mode 4; but this has some drawbacks:

- it may be difficult to dimension the separate pools correctly; this may lead to resource fragmentation, it may happen that one pool is overloaded while the other pool is underutilized.

- the resource pool configuration is semi-static; reconfiguring the pools to adjust their sizes to the current load situation is not possible in real time.

## Challenges

Sharing a resource pool between mode 3 and mode 4 transmissions poses some new challenges:

- Mode 3 transmissions, even if they are scheduled using semi-persistent scheduling (SPS), and are hence periodic in nature, do not indicate a resource reservation in the SCI. As a result, the sensing behaviour of mode 4 UEs does not work properly in identifying these mode 3 transmissions as semi-persistent and mode 4 UEs are more likely to select resources that collide with semi-persistent transmissions of a mode 3 UE than they would be with equivalent transmissions of a mode 4 UE.

- The eNodeB is not aware of the resources occupied by mode 4 transmissions, hence cannot avoid allocating the same resources, resulting in resource collisions between mode 3 and mode 4 UEs. This applies for both the case of semi-persistent scheduling and the case of dynamic scheduling by the eNodeB.

Observation 1: In R14, for mode 3 transmissions using SPS, the UE sets the “resource reservation” field in its SCI format 1 to zero, not to the SPS interval.

## Possible Solutions

### Mode 3 SPS

The first problem described above can be partially addressed by modifying the specified behaviour of mode 3 UEs; the specification can be changed such that mode 3 UEs scheduled using SPS will indicate a resource reservation in their SCI transmissions. This is an obvious “low-hanging fruit” without any downsides.

Proposal 1: For mode 3 transmissions using semi-persistent scheduling (SPS), the UE shall set the “resource reservation” field in its SCI format 1 to the SPS interval instead of setting it to zero.

This change can obviously be made for UEs from Rel-15 onwards; however, the question arises what to do about Rel-14 UEs in mode 3 with SPS. If they keep the legacy behaviour (setting the resource reservation field to zero), then the problem is not addressed for R14 UEs using mode 3 with SPS in the shared pool. It would hence be desirable to make the same modification to the behaviour of R14 UEs, and it can be argued that this would actually be a correction of a mistake; however, feedback from UE vendors is needed to see if such a change is possible at this stage.

Proposal 2: Discuss if the same modification is still acceptable for Rel-14: For mode 3 transmissions using semi-persistent scheduling (SPS), the UE shall set the “resource reservation” field in its SCI format 1 to the SPS interval instead of setting it to zero.

### Other

Even if Proposal 1 is agreed it does not solve the problems completely:

- Persistent collisions between an R15 UE using mode 3 SPS and a mode 4 UE will still take longer to resolve than equivalent collisions between two mode 4 UEs would take: in the case of two mode 4 UEs colliding, the persistent collision will be resolved when either of the mode 4 UEs performs resource reselection; while in the case of a mode 3 UE and a mode 4 UE the persistent collision will be resolved only when the mode 4 UE performs resource reselection.

- Collisions between mode 4 TX and mode 3 TX with dynamic scheduling are not addressed.

- Collisions between mode 4 TX and R14 mode 3 TX with SPS are not addressed (unless Proposal 2 is agreed).

# Conclusions

In this contribution we discussed the objective of Radio resource pool sharing between UEs using mode 3 and UEs using mode 4 and make the following observations and proposals:

Observation 1: In R14, for mode 3 transmissions using SPS, the UE sets the “resource reservation” field in its SCI format 1 to zero, not to the SPS interval.

Proposal 1: For mode 3 transmissions using semi-persistent scheduling (SPS), the UE shall set the “resource reservation” field in its SCI format 1 to the SPS interval instead of setting it to zero.

Proposal 2: Discuss if the same modification is still acceptable for Rel-14: For mode 3 transmissions using semi-persistent scheduling (SPS), the UE shall set the “resource reservation” field in its SCI format 1 to the SPS interval instead of setting it to zero.

# References

- RP-171740, WID “V2X phase 2 based on LTE”