3GPP TSG-RAN WG1 Meeting #91	R1-1720533

Reno, USA, 27th of Dec – 1st of Dec, 2017

Agenda Item:	6.2.8.1

Source:	Ericsson

Title:	Remaining details of evaluation scenarios for URLLC

Document for:	Discussion, Decision

# Introduction

At RAN#75 a new Work Item on Ultra Reliable and Low Latency communication was approved [1].

In the first phase of the Work Item, different requirements on reliability and latency are to be identified together with any potential evaluation scenarios.

Phase 1 (till RAN#78) - Identify improved communication reliability and different latency constraints combinations for both wide and local area deployments [RAN1] - Consider the ITU IMT-2020 and the 3GPP TR 38.913 requirements on URLLC and the ability to enable the network to operation with a range of reliability targets and latency constraints. - Identify any potential new evaluations scenarios [RAN1]

This contribution presents our views on the remaining details of evaluation scenarios of URLLC for LTE.

It should be noted that two email discussions have been taken place since RAN1#90b, one with approval rights (relating to system level simulation assumptions, see [7]) and one summarizing company views (relating to link level simulation assumptions, see [8]), and hence these agreements are used as baseline for the discussion.

This is a revision of R1-1717452.

# Discussion

## Use cases and target requirements

At RAN1#90b there were two target requirements agreed for URLLC in LTE, see Table 1. We believe those are sufficient for the RAN work on URLLC in LTE.

No new target requirements for URLLC in LTE are needed

Table 1: Target RAN latency and reliability requirements for URLLC for LTE

| Target requirement | Latency bound | Reliability | Payload size [bytes] |
| --- | --- | --- | --- |
| 1 | 1ms | 10-5 | 32 |
| 2 | 10ms | 10-4 | 32 |

## Evaluation scenarios

### General

At RAN#90b, it was agreed to use the ITU methodology as baseline

Agreement: - Evaluation method: - Use Link level simulation based on ITU methodology (i.e. a step-wise approach) - The fulfilment of the reliability target is verified in link level simulations at a reference SINR, i.e. Q, resulting from system level simulations. - FFS: - The simulation assumptions to derive the reference SINR, i.e. Q - The reference SINR is calibrated among companies - Other link level simulation methodologies not focussing on the ITU requirement are not precluded. - FFS details - The error probability should be provided for a range of SNR

In this step-wise approach, system level simulations are used to get a SINR distribution that can be used for further evaluations in link level simulations. It is believed that this is sufficient and that no additional system level simulation methodologies are required.

No additional system level simulation methodology, in addition to the one part of the step-wise ITU methodology, is required

For link level evaluations the system level simulations are used to get the SINR CDF to set in the link simulations. Link simulations are then performed with the set of parameter settings being discussed in [8]. We do not see a need for further link simulation methodologies to be used for the work.

No additional link level simulation methodology, in addition to the one part of the step-wise ITU methodology, is required

### System level evaluations

The following assumptions were agreed from the RAN1#90b email approval:

Agreement: The SINR from system level evaluations is based on long-term SINR at a given position in the network and excludes fast fading component. Agreement: The SINR collected in system level simulations is the one at the antenna connector reference point (no combination of antenna ports is considered). Agreements: - Since the SINR is derived at the antenna connector without combination between ports, it is assumed that the assumption on number of antenna elements for the UE does not have an impact to the derived SINR point. - The channel model used for system level evaluations is the one described in 3GPP TR 38.901 (also called UMa B in ITU Eval document). - The following simulation settings for Urban Macro-URLLC are used: Parameter; Value. Carrier frequency for evaluation; 700 MHz. Inter-site distance; 500 m. UE power class; 23 dBm. BS antenna height; 25 m. Inter-site interference modelling; Explicitly modelled. BS noise figure; 5 dB. BS antenna element gain; 8 dBi. UE antenna element gain; 0 dBi. Thermal noise level; -174 dBm/Hz. UE antenna height; 1.5 m. Traffic model; Full buffer (Note: it is for SINR CDF distribution derivation). Number of URLLC UEs/TRxP; 10 for SINR CDF distribution derivation. URLLC UE location; 80% outdoor, 20% indoor. Total transmit power per TRxP; 49 dBm for 20 MHz bandwidth 46 dBm for 10 MHz bandwidth. Simulation bandwidth; 20 MHz. UL PUSCH power control parameters; α=1.0, P0,PUSCH=-106dBm (suggested value for UL SINR CDF distribution derivation and calibration) Other values are not precluded. If other values are used, it shall be reported.. UL PUCCH power control parameters; P0, subframe-PUCCH = -116 P0, slot-SPUCCH = -113 P0, subslot-SPUCCH = -108 (suggested value for UL SINR CDF distribution derivation and calibration). Bandwidth allocation; PUSCH: FFS PUCCH: 1 RB (To get a full load SINR for PUCCH, the same mutual interferers as for PUSCH are assumed but on a bandwidth of 1 RB). Number of antenna elements per TRxP; 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ. Number of TXRU per TRxP; 2TXRU, =(Mp,Np,P,Mg,Ng) = (1,1,2,1,1). Handover margin (dB); 0 (i.e., the strongest cell is selected). UT attachment; Based on RSRP (formula (8.1-1) in TR36.873) from port 0. - In addition to the Urban Macro-URLLC scenario, an indoor scenario is also defined, following the same ITU methodology. Details are FFS but Indoor Hotspot-eMBB from ITU IMT2020 Eval document is used as a starting point for setting the details.

Remaining parameter settings for the Urban Macro scenario is:

- UE noise figure

- eNB antenna tilt

- UL bandwidth allocation

- Percentage of high/low loss building types

- Wrapping

On the UE noise figure, it is proposed to follow the ITU assumptions on 7 dB since these are the simulation parameters that URLLC for LTE will be evaluated against in the self-evaluation campaign against ITU. As expected, and as shown in Table 2, this assumption has no impact due to the lack of power control on the DL and the use of full-buffer simulations, i.e. the DL is strictly interference limited.

Table 2: DL SINR distribution over different UE noise figures for Urban Macro-URLLC config B

There is no impact on UE noise figure in the resulting SINR due to the severe interference limitation

Adopt a UE noise figure of 7 dB

The antenna tilt values were discussed in the email approval but were not concluded on. The vast majority of companies preferred 8 or 12 degrees. It is believed that these antenna tilt values were not considering the full spatial channel model that is described in the “ITU IMT2020 Eval” document or alternatively in 3GPP TR 38.901. It seems reasonable to adopt a tilt that maximizes the UL and DL SINR. Below figure shows the SINR resulting from the tilt values proposed in the email discussion as well as for the tilt providing optimum SINR using the correct channel model.

Table 3: SINR distribution over different electrical tilt values for Urban Macro-URLLC config B

As can be seen, a tilt value of 8 or 12 degrees results in an inferior SINR distribution in both DL and UL compared to using the optimum value of 4 degrees. Hence, it is proposed to adopt a 4 degree tilt.

Adopt a 4 degree electrical tilt for the Urban-Macro URLLC scenario

Considering that there might have been different assumptions on the channel model assumptions from earlier evaluations with regards to the long-term SINR distribution, it would be good to clarify the channel model description, that this includes all spatial channel effects with the addition that the SINR is collected representing long-term SINR.

The full channel model in “ITU IMT2020 Eval“/38.901 is adopted for SLS, where the magnitude squared of the channel coefficients over time and frequency are averaged (to reflect long-term SINR) to determine the average path gain for each link

On the UL bandwidth allocation per UE, it is agreed that 10 UEs are to be simulated and that the total bandwidth is 20 MHz. With an equal bandwidth allocation, this results in a 10 RB allocation per UE. An alternative method has been discussed on the RAN1 reflector where 10 RBs would be an upper limit, but where users with a high pathloss would only be allocated a bandwidth that does not result in a power limited UE (if possible). Although this represents a valid network behaviour it also introduces questions on how to model a fully loaded system (10 UEs and at most 10 RBs are used per UE, resulting in a partially loaded network), it also couples more strongly the bandwidth assumed in system simulations with the ones in the link simulations (since the link simulations evaluate users in the worst coverage, where typically a smaller bandwidth is used). Also, what would be the smallest bandwidth allocation used in this case? It has been agreed that at least 4 RBs are allocated in the UL. It is proposed to take the simple approach, and commonly used approach, of equal bandwidth allocation.

The system bandwidth on the UL is split equally between the UEs of the cell

One of the parameter settings that have not been agreed yet, since not included in the email approval on system level assumptions, is the split between low and high loss building types in the channel model. It is proposed to follow the ITU assumption of 100% low-loss building types.

Adopt the ITU assumption on 100% low-loss building types in the channel model

| Percentage of high loss and low loss building type (Note 1) | 100% low loss |
| --- | --- |

To avoid border effects in network simulations, a wrap-around method is typically used. Two different methods can be applied, either a gain based, or distance based wrapping. Gain based wrapping that takes into account effects that are not only dependent on distance is more accurate and hence preferred.

Adopt a gain based wrapping method for system level simulations

| Wrapping around method | Gain based wrapping |
| --- | --- |

On the additional indoor-scenario agreed, the intention is to reuse as much as possible from the ITU Eval document, see below:

Agreement: In addition to the Urban Macro-URLLC scenario, an indoor scenario is also defined, following the same ITU methodology. Details are FFS but Indoor Hotspot-eMBB from ITU IMT2020 Eval document is used as a starting point for setting the details.

It is however proposed also for the hotspot scenario to change the carrier frequency to better align with typical LTE deployments. Since the hotspot scenario is not part of the ITU requirements for URLLC, a more typical carrier frequency should be used for the evaluations, hence it is proposed to use a 2 GHz carrier frequency.

Use the Indoor Hotspot-eMBB, Configuration A, and changing the carrier frequency to 2 GHz, evaluation configuration from “Guidelines for evaluation of radio interface technologies for IMT-2020 [IMT-2020.EVAL]” for deriving minimum SINR for link level evaluations for LA deployments

As with the macro scenario, the antenna configuration at the eNB needs to be decided. It is proposed to apply a configuration with 16 antenna elements. It can be noted that the same 4x4 configuration is assumed in the 3GPP assumptions for the Indoor scenario currently in the ongoing discussion on the ITU Ad Hoc reflector as well.

The antenna configuration per TRxP for the eNB in the Hotspot scenario is (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ

The number of TXRUs per TRxP for eNB in the Hotspot scenario is 2, mapping as (Mp,Np,P,Mg,Ng) = (4,4,2,1,1)

For more details on the indoor evaluation scenario, see [9].

### Link level evaluations

For the link level evaluations, there has been an email discussion after RAN1#90b relating to the assumptions required. Below is the view from us.

First of all, the NR assumptions for URLLC should be followed where applicable and reasonable for LTE.

For the link level evaluations, it is beneficial to align and follow NR assumptions for URLLC where applicable and reasonable for LTE

It is also of interest to align the simulation assumptions between companies and hence limit the parameter permutations possible in [4]. Some parameter values are also not of relevance and should therefore be removed.

Changes proposed to Table A.1.4-1 in [4] (original table also shown in Annex) with related motivation are:

- Only consider 700 MHz carrier frequency for the Urban-Macro scenario. In general, the carrier frequency used should be aligned between link and system level assumptions.

- Modulation and coding scheme: This should be for companies to evaluate and report

- Remove row on latency bound of 1ms and instead use the target requirements and the associated latency bound that is agreed.

- Remove row on SINR range and instead use the minimum average SINR requirements that will be derived based on IMT-2020 methodology. A range could however be added to ensure that not only a single value is reported.

- Remove row on sub-carrier spacing. This is only relevant for NR

- On the assumptions on TTI, it is our view that subslot, slot and subframe transmission should be within the scope of the work.

- Remove row on OFDM symbols per TTI. This is fixed with the LTE design of subframe, slot or subslot transmission duration

- Channel model is limited to TDL. Also, the 3GPP assumptions on RMS delay spread aligned with IMT-2020 evaluations (see [5] and [6]), as in Table 4. Also, only 3 km/h evaluated in for LA, while both 3 km/h and 30 km/h can be evaluated for WA.

Table 4: Channel model

| Model | RMS delay spread () [ns] | UE speed | Scenario |
| --- | --- | --- | --- |
| TDL-C | 363 | 3 km/h and/or 30 km/h | WA |
| TDL-E | 93 |  |  |
| TDL-A | 39 | 3 km/h | LA |
| TDL-D | 20 |  |  |

- BS antenna configuration is coupled to the assumption on antenna configuration for the system. In case of the macro scenario the assumption should be 2 Tx/Rx (due to cross-polarized antenna elements).

- UE antenna elements are limited to only 4 Tx/Rx. It is reasonable to assume that a URLLC UE is more on the high-end, and considering the importance of diversity for URLLC having more antenna elements is typically the type of properties that are of interest.

- Remove row on packet arrival rate assumptions. This is more of interest for system level simulations, and link simulations shall be carried out with no dependencies between user packet arrival, or different packets associated with the same user.

- Channel estimation should be practical and not ideal. Considering the low SINR point that URLLC is expected to operate at, the impact from practical channel estimation cannot be ignored.

The resulting parameters with the above-mentioned changes are shown in Table 5.

Table 5: Proposed link level simulation assumptions

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 700MHz (FDD and TDD) |
| Modulation and coding rate | Companies report |
| User bandwidth | Companies report |
| TTI length | Companies report (subslot, slot or subframe) |
| Channel model | TDL-A; = 39 ns; LA; 3 km/h; TDL-C; = 363 ns; WA; 3 km/h and/or 15 km/h TDL-D; = 20 ns; LA; 3 km/h TDL-E; = 93 ns; WA; 3 km/h and/or 15 km/h for details, see 3GPP TR 38.901 |
| UE antenna elements | 4 Tx/Rx ports |
| PHY Packet size | 32 B |
| ACK Feedback assumption | Ideal as start point (Note 1) |
| Channel estimation | Realistic |
| CQI feedback assumption | Companies report the feedback scheme if any |
| NOTE: Control channels including DL assignment/UL grant/ACK/NACK are to be evaluated further. NOTE 1: It is also possible that no ACK feedback is needed. |  |

The above changes can be captured in the following proposal:

For link evaluations for URLLC in LTE, use:

- Carrier frequency: 700 MHz

- MCS: To be reported

- Channel models:

  - TDL-A: = 39 ns; 	LA; 3 km/h;

  - TDL-C: = 363 ns; 	WA; 3 km/h and/or 15 km/h

  - TDL-D: = 20 ns; 	LA; 3 km/h

  - TDL-E: = 93 ns; 	WA; 3 km/h and/or 15 km/h

- UE antenna elements: 4 Tx/Rx ports

- PHY packet size:

  - 32 bytes (for IMT-2020 requirements)

  - 200 bytes (for requirement on 10ms, 10-5 reliability)

- Channel estimation: Realistic

For link evaluations companies are to report assumption on:

- User bandwidth

- MCS used

- TTI length

- DCI / UCI payload

- ACK/NAK feedback

- CQI feedback

The reporting has already been agreed to include the minimum SINR 5th percentile point from system level simulations. It is however reasonable that also a range is shown. To limit the simulation effort, it is proposed to only mandate a range 5*10r-1 ≤  R ≤ 5*10r+1 where the target reliability is R=10r (target reliability).

Link simulations should at least be shown for the range 5*10r-1 ≤  R ≤ 5*10r+1 where the target reliability is R=10r (target reliability).

### Evaluation metric

It was agreed at RAN1#90b to reuse the reliability definition from 3GPP TR 38.802:

Agreement: For LTE URLLC evaluation, reliability is used as metric. The reliability definition from NR in 3GPP TR 38.802 is reused. - Definition: Reliability is defined as the success probability R of transmitting X bits within L seconds, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface, at a certain channel quality Q (e.g., coverage-edge). - Spectral efficiency should be considered. - The latency bound L includes transmission latency, processing latency, retransmission latency and queuing/scheduling latency (including scheduling request and grant reception if any).

The definition of spectral efficiency was shortly discussed and not reused, and instead only a statement that this should be considered was added. One reason not to reuse the spectral efficiency definition was that it did not take different type of transmissions into account (for example HARQ where there is a gap in time between first and second transmission). Considering that the latency bound is set for the respective requirement, and that the packet size is fixed, it should be enough to report the resources used for each configuration simulated. Although only one payload size is used in the simulations, it might be of interest to make the metric independent of payload size and report the bits per resource element (BPRE)

The bit per resource element (BPRE), including reference signal overhead, should be reported with the simulations

## Derivation of Q (minimum SINR target)

Following the above assumptions, the minimum SINR target is shown in Table 6 for different configurations (assuming equal UL bandwidth allocation for each UE). It can be noted that the results are valid for both a UE noise figure of 7 dB and 9 dB.

Table 6: Q (5th percentile SINR) for different assumptions for the Macro-URLLC scenario

| Physical channel | Electrical down-tilt [degrees*] | Q |
| --- | --- | --- |
| PDCCH/PDSCH | 4 | -2.3 |
|  | 8 | -3.8 |
|  | 12 | -3.3 |
| PUSCH | 4 | -1.4 |
|  | 8 | -2.4 |
|  | 12 | -2.1 |
| PUCCH | 4 | -1.6 |
| * From horizontal plane |  |  |

# Conclusion

Based on the discussion in section 2 we observe the following:

Observation 1	No new target requirements for URLLC in LTE are needed

Observation 2	There is no impact on UE noise figure in the resulting SINR due to the severe interference limitation

Observation 3	For the link level evaluations, it is beneficial to align and follow NR assumptions for URLLC where applicable and reasonable for LTE

Based on the discussion in section 2 we the following proposals:

Proposal 1	No additional system level simulation methodology, in addition to the one part of the step-wise ITU methodology, is required

Proposal 2	No additional link level simulation methodology, in addition to the one part of the step-wise ITU methodology, is required

Proposal 3	Adopt a UE noise figure of 7 dB

Proposal 4	Adopt a 4 degree electrical tilt for the Urban-Macro URLLC scenario

Proposal 5	The full channel model in “ITU IMT2020 Eval“/38.901 is adopted for SLS, where the magnitude squared of the channel coefficients over time and frequency are averaged (to reflect long-term SINR) to determine the average path gain for each link

Proposal 6	The system bandwidth on the UL is split equally between the UEs of the cell

Proposal 7	Adopt the ITU assumption on 100% low-loss building types in the channel model

Proposal 8	Adopt a gain based wrapping method for system level simulations

Proposal 9	Use the Indoor Hotspot-eMBB, Configuration A, and changing the carrier frequency to 2 GHz, evaluation configuration from “Guidelines for evaluation of radio interface technologies for IMT-2020 [IMT-2020.EVAL]” for deriving minimum SINR for link level evaluations for LA deployments

Proposal 10	The antenna configuration per TRxP for the eNB in the Hotspot scenario is (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ

Proposal 11	The number of TXRUs per TRxP for eNB in the Hotspot scenario is 2, mapping as (Mp,Np,P,Mg,Ng) = (4,4,2,1,1)

Proposal 12	For link evaluations for URLLC in LTE, use:

	Carrier frequency: 700 MHz

	MCS: To be reported

	Channel models:

o	TDL-A: = 39 ns;  LA; 3 km/h;

o	TDL-C: = 363 ns;  WA; 3 km/h and/or 15 km/h

o	TDL-D: = 20 ns;  LA; 3 km/h

o	TDL-E: = 93 ns;  WA; 3 km/h and/or 15 km/h

	UE antenna elements: 4 Tx/Rx ports

	PHY packet size:

o	32 bytes (for IMT-2020 requirements)

o	200 bytes (for requirement on 10ms, 10-5 reliability)

	Channel estimation: Realistic

Proposal 13	For link evaluations companies are to report assumption on:

	User bandwidth

	MCS used

	TTI length

	DCI / UCI payload

	ACK/NAK feedback

	CQI feedback

Proposal 14	Link simulations should at least be shown for the range 5*10r-1 ≤  R ≤ 5*10r+1 where the target reliability is R=10r (target reliability).

Proposal 15	The bit per resource element (BPRE), including reference signal overhead, should be reported with the simulations

# References

RP-170796, New Work Item on Ultra Reliable Low Latency Communication for LTE, source Ericsson, Nokia, Alcatel-Lucent Shanghai Bell. RAN#76

“Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, ITU-R

R1-1717451, “Use cases and target requirements for URLLC”, source Ericsson. RAN1#90bis

3GPP TR 38.802 V14.2.0

“Guidelines for evaluation of radio interface technologies for IMT-2020”, ITU-R

3GPP TR 38.901 V14.2.0

R1-1720537, Summary of email discussion [90b-LTE-24] on system level evaluation assumption and methodology for URLLC for LTE, source Ericsson. RAN1#91

R1-1729501, Summary of email discussion [90b-LTE-25] on link level assumptions for URLLC, source Huawei, HiSilicon. RAN1#91

R1-1720536, Indoor evaluation scenario for URLLC, source Ericsson. RAN1#91

# Annex

## Snippet from IMT-2020 Eval [5]

### 7.1.5 Reliability The evaluator shall perform the following steps in order to evaluate the reliability requirement using system-level simulation followed by link-level simulations. Step 1: Run downlink or uplink full buffer system-level simulations of candidate RITs/SRITs using the evaluation parameters of Urban Macro-URLLC test environment see § 8.4.1, and collect overall statistics for downlink or uplink SINR values, and construct CDF over these values. Step 2: Use the CDF for the Urban Macro-URLLC test environment to save the respective 5th percentile downlink or uplink SINR value. Step 3: Run corresponding link-level simulations for either NLOS or LOS channel conditions using the associated parameters in the Table 8-X2 of this report, § 8.4, to obtain success probability, which equals to (1-Pe), where Pe is the residual packet error ratio within maximum delay time as a function of SINR taking into account retransmission. Step 4: The proposal fulfils the reliability requirement if at the 5th percentile downlink or uplink SINR value of Step 2 and within the required delay, the success probability derived in Step 3 is larger than or equal to the required success probability. It is sufficient to fulfil the requirement in either downlink or uplink in either NLOS or LOS channel conditions

## Link simulation assumptions from [4]

Table A.1.4-1 from [4] is shown below.

Table 7: Table A.1.4-1 reproduced from [4]

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 700MHz and 4 GHz (FDD and TDD) |
| Modulation and coding rate | QPSK, 16QAM, 64QAM 1/12, 1/6, 1/3 Other MCS not precluded Comparison should be made for the same spectrum efficiency |
| User bandwidth | Companies report |
| Latency bound | 1ms Other values are not precluded Companies report delay assumptions according to Table 1 in R1-166485 |
| SINR range | -5dB to 20dB Larger range is not precluded |
| Sub-carrier spacing | Companies report |
| TTI length | Companies report |
| OFDM symbols per TTI | Companies report |
| Channel model | TDL/CDL in TR 38.901 [15]; user speed = 3km/h, 15km/h (other user speed is not precluded) |
| BS antenna configuration | 2/4/8 Tx/Rx ports as start point Other values (i.e., up to 256) are not precluded |
| UE antenna elements | 2/4 Tx/Rx ports as start point Other values (i.e., up to 8) are not precluded |
| Packet arrive rate | Option 1: periodically Option 2: Poisson arrival with arrival rate  |
| PHY Packet size | 32 byte, 50 byte, 200 byte Other values are not precluded. |
| ACK Feedback assumption | Ideal as start point (Note 1) |
| Channel estimation | Ideal as start point; Realistic is not precluded when RS design is ready |
| CQI feedback assumption | Companies report the feedback scheme if any |
| NOTE: control channels including DL assignment/UL grant/ACK/NACK are to be evaluated further. NOTE 1: It is also possible that no ACK feedback is needed. |  |

## System simulation assumptions from [[5]]

Table 8: System level simulations for LA evaluations from ITU

| Parameters | Indoor Hotspot-eMBB |
| --- | --- |
|  | Configuration A |
| Baseline evaluation configuration parameters |  |
| Carrier frequency for evaluation | 4 GHz |
| BS antenna height | 3 m |
| Total transmit power per TRxP | 24 dBm for 20 MHz; 21 dBm for 10 MHz |
| UE power class | 23 dBm |
| Additional parameters for system-level simulation |  |
| Inter-site distance | 20 m |
| Number of antenna elements per TRxP | Up to 256 Tx/Rx |
| Number of UE antenna elements | Up to 8 Tx/Rx |
| Device deployment | Randomly and uniformly distributed over area 100% indoor |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |
| UE speeds of interest | 100% indoor, 3 km/h |
| Inter-site interference modeling | Explicitly modelled |
| BS noise figure | 5 dB |
| UE noise figure | 7 dB |
| BS antenna element gain | 8 dBi |
| UE antenna element gain | 0 dBi |
| Thermal noise level | -174 dBm/Hz |
| Traffic model | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD |
| UE density | 10 UEs per TRxP randomly and uniformly dropped throughout the geographical area |
| UE antenna height | 1.5 m |

Table 9: System level simulations for WA evaluations

| Parameters | Urban Macro–URLLC |
| --- | --- |
|  | Reliability Evaluation |
|  | Configuration B |
| Baseline evaluation configuration parameters |  |
| Carrier frequency for evaluation | 700 MHz |
| BS antenna height | 25 m |
| Total transmit power per TRxP | 49 dBm for 20 MHz bandwidth 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm |
| Percentage of high loss and low loss building type (Note 1) | 100% low loss |
| Additional parameters for system-level simulation |  |
| Inter-site distance | 500 m |
| Number of antenna elements per TRxP1 | Up to 64 Tx/Rx |
| Number of UE antenna elements | Up to 4Tx/Rx |
| Device deployment | 80% outdoor, 20% indoor |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |
| UE speeds of interest | 3 km/h for indoor and 30km/h for outdoor |
| Inter-site interference modelling | Explicitly modelled |
| BS noise figure | 5 dB |
| UE noise figure | 7 dB |
| BS antenna element gain | 8 dBi |
| UE antenna element gain | 0 dBi |
| Thermal noise level | -174 dBm/Hz |
| Traffic model | Full buffer (Note: it is for SINR CDF distribution derivation) |
| Simulation bandwidth | Up to 40 MHz Note: This value is used for SINR CDF distribution derivation |
| UE density | 10 UEs per TRxP for SINR CDF distribution derivation |
| UE antenna height | 1.5 m |