3GPP TSG-RAN WG1 Meeting #91	R1-1721063

Reno, USA, 27th of Dec – 1st of Dec, 2017

Agenda Item:	6.2.8.3

Source:	Ericsson

Title:	Indoor evaluation scenarios for URLLC

Document for:	Discussion, Decision

# Introduction

At RAN#75 a new Work Item on Ultra Reliable and Low Latency communication was approved [1].

In the first phase of the Work Item, different requirements on reliability and latency are to be identified together with any potential evaluation scenarios.

Phase 1 (till RAN#78) - Identify improved communication reliability and different latency constraints combinations for both wide and local area deployments [RAN1] - Consider the ITU IMT-2020 and the 3GPP TR 38.913 requirements on URLLC and the ability to enable the network to operation with a range of reliability targets and latency constraints. - Identify any potential new evaluations scenarios [RAN1]

It should be noted that two email discussions have been taken place since RAN1#90b, one with approval rights (relating to system level simulation assumptions, see [2]) and one summarizing company views (relating to link level simulation assumptions, see [3]), and hence these agreements are used as baseline for the discussion.

In the document, we present our views on the indoor scenario agreed to be added as a system level simulation scenario, in addition to the already agreed macro scenario. The resulting SINR distribution from the system level simulations are also shown.

This document is a revision of R1-1720536.

# Discussion

## Details of the indoor evaluation scenario

During the system level email discussion, it was agreed to, in addition to an urban macro scenario, also define an indoor scenario, see below.

Agreement: In addition to the Urban Macro-URLLC scenario, an indoor scenario is also defined, following the same ITU methodology. Details are FFS but Indoor Hotspot-eMBB from ITU IMT2020 Eval document is used as a starting point for setting the details.

The assumptions from ITU for the Hotspot-eMBB scenario is given in the Annex.

Out of the three scenarios defined for different carrier frequencies (4 GHz - A, 30 GHz - B, 70 GHz - C). Two of the scenarios (30 GHz and 70 GHz) are outside of the frequency domain defined in LTE and hence configuration A should be used as baseline.

Use the Indoor Hotspot-eMBB, Configuration A, and changing the carrier frequency to 2 GHz, evaluation configuration from “Guidelines for evaluation of radio interface technologies for IMT-2020 [IMT-2020.EVAL]” for deriving minimum SINR for link level evaluations for LA deployments

As with the macro scenario, the antenna configuration at the eNB needs to be decided. It is proposed to apply a configuration with 16 antenna elements. It can be noted that the same 4x4 configuration is assumed in the 3GPP assumptions for the Indoor scenario currently in the ongoing discussion on the ITU Ad Hoc reflector as well.

The antenna configuration per TRxP for the eNB in the Hotspot scenario is (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ

The number of TXRUs per TRxP for eNB in the Hotspot scenario is 2, mapping as (Mp,Np,P,Mg,Ng) = (4,4,2,1,1)

In addition, there are a number of assumptions that are missing from the ITU assumptions, of which some have already been agreed for the Urban Macro and can be reused in the indoor scenario. The parameters are listed below.

Adopt the Indoor Hotspot-eMBB, Configuration A, with the following additions

| UL PUSCH power control parameters | α=1.0, P0,PUSCH=-106dBm (suggested value for UL SINR CDF distribution derivation and calibration) Other values are not precluded. If other values are used, it shall be reported. |
| --- | --- |
| UL PUCCH power control parameters | P0, subframe-PUCCH = -116 P0, slot-SPUCCH = -113 P0, subslot-SPUCCH = -108 (suggested value for UL SINR CDF distribution derivation and calibration) |
| Bandwidth allocation | PUSCH: Equal bandwidth PUCCH: 1 RB (To get a full load SINR for PUCCH, the same mutual interferers as for PUSCH are assumed but on a bandwidth of 1 RB) |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) |
| Simulation bandwidth | 20 MHz |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Beam forming | Ideal |
| Wrapping around method | No wrapping around |
| TRxP number per site | 3 |
| Mechanical tilt | 110° in GCS |
| Electrical tilt | 90° in LCS |

## Derivation of Q (minimum SINR target)

Following the above assumptions, the minimum SINR target is shown in Table 1. The SINR distribution is also shown in Figure 1.

Table 1: Q (5th percentile SINR) for different assumptions for the Macro-URLLC scenario

| Physical channel | Q [dB] |
| --- | --- |
| PDCCH/PDSCH | -3.2 |
| PUSCH | -1.0 |
| PUCCH sub-slot | -2.1 |
| PUCCH slot | -4.9 |
| PUCCH subframe | -7.0 |

Figure 1: SINR distribution for the eMBB indoor scenario assuming the proposed simulation assumptions

# Conclusion

Based on the discussion in section 2 we the following proposals:

Proposal 1	Use the Indoor Hotspot-eMBB, Configuration A, and changing the carrier frequency to 2 GHz, evaluation configuration from “Guidelines for evaluation of radio interface technologies for IMT-2020 [IMT-2020.EVAL]” for deriving minimum SINR for link level evaluations for LA deployments

Proposal 2	The antenna configuration per TRxP for the eNB in the Hotspot scenario is (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ

Proposal 3	The number of TXRUs per TRxP for eNB in the Hotspot scenario is 2, mapping as (Mp,Np,P,Mg,Ng) = (4,4,2,1,1)

Proposal 4	Adopt the Indoor Hotspot-eMBB, Configuration A, with the following additions

| UL PUSCH power control parameters | α=1.0, P0,PUSCH=-106dBm (suggested value for UL SINR CDF distribution derivation and calibration) Other values are not precluded. If other values are used, it shall be reported. |
| --- | --- |
| UL PUCCH power control parameters | P0, subframe-PUCCH = -116 P0, slot-SPUCCH = -113 P0, subslot-SPUCCH = -108 (suggested value for UL SINR CDF distribution derivation and calibration) |
| Bandwidth allocation | PUSCH: Equal bandwidth PUCCH: 1 RB (To get a full load SINR for PUCCH, the same mutual interferers as for PUSCH are assumed but on a bandwidth of 1 RB) |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) |
| Simulation bandwidth | 20 MHz |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Beam forming | Ideal |
| Wrapping around method | No wrapping around |
| TRxP number per site | 3 |
| Mechanical tilt | 110° in GCS |
| Electrical tilt | 90° in LCS |

# References

RP-170796, New Work Item on Ultra Reliable Low Latency Communication for LTE, source Ericsson, Nokia, Alcatel-Lucent Shanghai Bell. RAN#76

R1-1720537, Summary of email discussion [90b-LTE-24] on system level evaluation assumption and methodology for URLLC for LTE, source Ericsson. RAN1#91

R1-1719501, Summary of email discussion [90b-LTE-25] on link level assumptions for URLLC, source Huawei, HiSilicon. RAN1#91

“Guidelines for evaluation of radio interface technologies for IMT-2020”, ITU-R

# Annex

## System simulation assumptions from [4]

Table 2: System level simulations for indoor hot spot from ITU

| Parameters | Indoor Hotspot-eMBB |  |  |
| --- | --- | --- | --- |
|  | Spectral Efficiency, Mobility, and Area Traffic Capacity Evaluations |  |  |
|  | Configuration A | Configuration B | Configuration C |
| Baseline evaluation configuration parameters |  |  |  |
| Carrier frequency for evaluation | 4 GHz | 30 GHz | 70 GHz |
| BS antenna height | 3 m | 3 m | 3 m |
| Total transmit power per TRxP | 24 dBm for 20 MHz; 21 dBm for 10 MHz | 23 dBm for 80 MHz bandwidth; 20 dBm for 40 MHz bandwidth EIRP should not exceed 58 dBm | 21 dBm for 80 MHz bandwidth 18 dBm for 40 MHz bandwidth EIRP should not exceed 58 dBm |
| UE power class | 23 dBm | 23 dBm EIRP should not exceed 43 dBm | 21 dBm EIRP should not exceed 43 dBm |
| Additional parameters for system-level simulation |  |  |  |
| Inter-site distance | 20 m | 20 m | 20 m |
| Number of antenna elements per TRxP | Up to 256 Tx/Rx | Up to 256 Tx/Rx | Up to 1024 Tx/Rx |
| Number of UE antenna elements | Up to 8 Tx/Rx | Up to 32 Tx/Rx | Up to 64 Tx/Rx |
| Device deployment | Randomly and uniformly distributed over area 100% indoor | Randomly and uniformly distributed over area 100% indoor | Randomly and uniformly distributed over area 100% indoor |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |
| UE speeds of interest | 100% indoor, 3 km/h | 100% indoor, 3 km/h | 100% indoor, 3 km/h |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 7 dB | 7 dB |
| UE noise figure | 7 dB | 10 dB | 10 dB3 |
| BS antenna element gain | 8 dBi | 5 dBi | 5 dBi |
| UE antenna element gain | 0 dBi | 5 dBi | 5 dBi |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 80 MHz for TDD, 40 MHz+40 MHz for FDD | 80 MHz for TDD, 40 MHz+40 MHz for FDD |
| UE density | 10 UEs per TRxP randomly and uniformly dropped throughout the geographical area | 10 UEs per TRxP randomly and uniformly dropped throughout the geographical area | 10 UEs per TRxP randomly and uniformly dropped throughout the geographical area |
| UE antenna height | 1.5 m | 1.5 m | 1.5 m |

10 dB for 30 GHz / 70 GHz is assumed for high performance UE. Higher UE noise figure values can be considered by the proponent, e.g. 13 dB for 30 GHz / 70 GHz.