3GPP TSG-RAN WG1 Meeting #91	R1-1721146

Reno Nevada, USA 27th Nov – 1th Dec 2017

Agenda Item:	6.2.6.3.3

Source: 	Feature lead Ericsson

Title:  	Summary of NB-IoT TDD Common aspects

Document for:	Discussion and Decision

# Summary of open issues and WFs

##### Common aspects

In RAN1 #90bis, the discussions focused on DL and UL aspects and no agreements were captured under this agenda item. A WF on Common Aspects was drafted by LG in R1-1719101 but it was not agreed.

The open issues listed below were built from pending WAs and the WFs submitted so far to RAN1 #91.

- WA on the support of the TDD configuration #6

Open Issue: Confirm or not the working assumption stating that TDD configuration#6 is not supported

- Support of the Special Subframe Configurations

Open Issue: Decide whether or not to support all the TDD special subframe configurations Reply LS: Reply LS to R1-1715304 LS on minimum time for DL-to-UL and UL-to-DL switching on one NB-IoT Carrier for TDD NB-IoT UEs. “Considering NB-IoT UEs should be low complexity, RAN4 further discussed the minimum time for DL-to-UL and UL-to-DL switching on one NB-IoT carrier for TDD NB-IoT UEs and evaluated it to be equal to 40ms for both direction.”

- MCL/Latency relaxation for TDD NB-IoT

Open Issue: Discuss the relaxation of MCL or Latency for TDD NB-IoT WF: R1-1721176 WF on MCL or Latency relaxation of NB-IoT TDD - Ericsson

- Cross-carrier scheduling for TDD NB-IoT

Open Issue: Support of the Cross-carrier scheduling in TDD NB-IoT WF: R1-1721221 WF on Cross-carrier scheduling in TDD – LG Electronics, Samsung

- HARQ in TDD NB-IoT

Open Issue: R1-17xxxxx HARQ in TDD in TDD NB-IoT (e.g., the support of two HARQ processes, scheduling delays) WF: R1-1721214 WF on HARQ in TDD NB-IoT – Huawei, HiSilicon

- Bitmaps

Open Issue: Discuss the support of bitmaps for TDD NB-IoT WF: R1-1721222 WF on subframe configurations in TDD – LG Electronics