3GPP TSG RAN WG1 Meeting #91	R1-1721168

Reno, USA, November 27 - December 1, 2017

Agenda Item:	6.2.5.2

Source:	Huawei, HiSilicon

Title:	Summary of Early Data Transmission for eMTC

Document for:	Discussion and decision

# Previous agreements

- From RAN1 point of view, it is feasible to support early UL data transmission in Msg3 from a BL/CE UE using some TBS value(s) from the TBS range specified for BL/CE UEs in Rel-13 with a maximum total TBS of 1000 bits.

  - Note: For Msg3 for Rel-13 BL/CE UEs, the maximum total TBS is 712 bits in CE mode A and 328 bits in CE mode B.

  - FFS if and how there will also be a larger supported maximum total TBS (than 1000 bits)

  - The detailed value(s) should consider the payload size of early data packets from RAN2.

- From RAN1 perspective, the physical layer design will assume eNB is not required to always provide a grant of a larger TBS for Msg3 and can decide to just provide a grant corresponding to Rel-13 Msg3 TBS instead.

# Incoming requests from other WGs

Incoming RAN2 LS (R1-1719305). We sent a partial reply last meeting.

RAN2 would like to kindly ask the following questions to RAN1:

- To support UL early data transmission in Msg3 during a RACH procedure initiated by a UE in RRC_IDLE, RAN2 assumes that Rel-13 PUSCH TB sizes can be used. Is such assumption viable? If not, what are the possible TB sizes for PUSCH transmission for EDT for eMTC and NB-IoT respectively?

- To support above TB sizes for Msg3, would there be need for new UL grant format(s) in RAR?  If yes, what changes are foreseen?

# Open issues

- Maximum supported TBS 	→ RAN1 could agree this week.

- Precise TBS values supported 	→ Wait for more progress in RAN2.

- UL grant in RAR format(s) 	→ RAN1 could investigate the options this week, but designs depend on open issue #2.

- PRACH resource partitioning 	→ Wait for more progress in RAN2.

- Reply LS to RAN2 on 1,2,3	 → We replied partially already.

# Summary of WFs

R1-17xxxxx WF on early data transmission	Huawei, HiSilicon, …

  - Depending on eNB’s implementation, PRACH partitioning can be based on preamble/time/frequency or its combination

  - Maximum TBS for early data transmission in Msg3 is 1000 bits

  - One reserved bit in MAC RAR is used to differentiate the UL grant in RAR